|  | 1. Introduction | 12 |  |
| --- | --- | --- | --- |
|  | 2. Acknowledgements | 12 |  |
|  | 3. Revision List | 13 |  |
|  | 4. Data Communications | 15 |  |
|  | 5. Why Telecommunications? | 15 |  |

|  | a. Voice Channels | 15 |  |
| --- | --- | --- | --- |
|  | b. Data Channels | 16 |  |

|  | 6. Introduction to Networking | 17 |  |
| --- | --- | --- | --- |

|  | a. The Big Picture | 17 |  |
| --- | --- | --- | --- |
|  | b. Telecommunications Components of The Big Picture | 20 |  |
|  | c. ISO OSI | 20 |  |

|  | 7. Breaking The Big Picture up! | 22 |  |
| --- | --- | --- | --- |

|  | a. The Local Loop | 22 |  |
| --- | --- | --- | --- |
|  | b. LANs | 23 |  |
|  | c. MANs | 24 |  |
|  | d. WANs. | 26 |  |

|  | 8. Trade Magazines | 27 |  |
| --- | --- | --- | --- |

|  | 9. The Role of Telecommunications in Networking | 29 |  |
| --- | --- | --- | --- |

|  | a. LANs | 29 |  |
| --- | --- | --- | --- |
|  | b. MANs | 29 |  |
|  | c. WANs | 30 |  |

|  | 10. Brief History of Networking | 31 |  |
| --- | --- | --- | --- |

|  | 11. Data Communication Network | 34 |  |
| --- | --- | --- | --- |

|  | a. Performance | 34 |  |
| --- | --- | --- | --- |
|  | b. Consistency | 34 |  |
|  | c. Reliability, | 35 |  |
|  | d. Recovery | 36 |  |
|  | e. Security | 36 |  |
|  | f. Applications | 36 |  |

|  | g. Basic Components | 38 |  |
| --- | --- | --- | --- |

|  | 12. Data Flow | 40 |  |
| --- | --- | --- | --- |
|  | 13. Modems | 43 |  |

|  | a. Basic Definition | 43 |  |
| --- | --- | --- | --- |
|  | b. Digital Connection | 43 |  |
|  | c. Analog Connection | 45 |  |
|  | d. External/Internal Modems | 45 |  |
|  | e. Modem Types | 47 |  |
|  | f. Features of Modems | 49 |  |
|  | g. Modem Speeds / Standards | 50 |  |
|  | h. Transfer Rate versus PC Bus Speed | 51 |  |
|  | h. V.90 56 kbps Modems | 51 |  |

|  | 14. Physical Connection | 52 |  |
| --- | --- | --- | --- |
|  | 15. Transmission Media - Guided | 53 |  |

|  | a. Open Wire | 53 |  |
| --- | --- | --- | --- |
|  | b. Twisted Pair | 55 |  |
|  | c. Coaxial Cable | 57 |  |
|  | d. Optical Fibre | 57 |  |

|  | i. Optical Transmission Modes | 59 |  |
| --- | --- | --- | --- |
|  | ii. Step Index Mode | 61 |  |
|  | iii. Grade Index Mode | 61 |  |
|  | iv. Single Mode | 61 |  |
|  | v. Comparison of Optical Fibres | 63 |  |
|  | vi. Advantages of Optical Fibre | 64 |  |
|  | vii. Disadvantages of Optical Fibre | 65 |  |

|  | e. Media versus Bandwidth | 65 |  |
| --- | --- | --- | --- |

|  | 16. Transmission Media - Unguided | 65 |  |
| --- | --- | --- | --- |

|  | a. RF Propagation | 66 |  |
| --- | --- | --- | --- |

|  | i. Ground Wave Propagation | 66 |  |
| --- | --- | --- | --- |
|  | ii. Ionospheric Propagation | 67 |  |
|  | iii. Line of Sight Propagation | 67 |  |

|  | b. Radio Frequencies | 68 |  |
| --- | --- | --- | --- |
|  | c. Microwave | 69 |  |
|  | d. Satellite | 70 |  |
|  | e. Iridium Telecom System | 72 |  |

|  | 17. RS-232D Serial Interface Standard | 74 |  |
| --- | --- | --- | --- |

|  | a. Mechanical Characteristics of the RS-232D | 74 |  |
| --- | --- | --- | --- |
|  | b. Electrical Characteristics of the RS-232D | 74 |  |
|  | c. Function of Each Signal | 76 |  |
|  | d. Subsets of Signals for Certain Applications | 78 |  |

|  | 18. RS-232D Flow Control | 80 |  |
| --- | --- | --- | --- |

|  | a. Hardware Handshaking | 81 |  |
| --- | --- | --- | --- |
|  | b. Hardware Null Modems | 88 |  |
|  | c. Software Handshaking (Xon/Xoff) | 89 |  |
|  | d. Software Null Modem | 89 |  |
|  | e. Terminals & PCs | 91 |  |

|  | 19. Timing | 92 |  |
| --- | --- | --- | --- |

|  | a. Asynchronous vs. Synchronous Transmission | 93 |  |
| --- | --- | --- | --- |

|  | 20. Asynchronous Communications | 95 |  |
| --- | --- | --- | --- |

|  | a. Start/Stop bits | 95 |  |
| --- | --- | --- | --- |
|  | b. 7/8 Bit Codes | 99 |  |
|  | c. Parity Bits | 101 |  |

|  | 21. Line Encoding | 104 |  |
| --- | --- | --- | --- |

|  | a. Unipolar Encoding | 104 |  |
| --- | --- | --- | --- |
|  | b. Polar Encoding | 106 |  |
|  | c. Bipolar Line Encoding | 108 |  |
|  | d. Manchester Line Encoding | 108 |  |

|  | 22. Standard Digital Codes | 110 |  |
| --- | --- | --- | --- |

|  | a. EBCDIC - Extended Binary Coded Decimal Interchange Code | 110 |  |
| --- | --- | --- | --- |
|  | b. ASCII - American Standard Code for Information Interchange | 116 |  |

|  | 23. Voice Channel Communications | 121 |  |
| --- | --- | --- | --- |

|  | a. Voice Channel Specification | 121 |  |
| --- | --- | --- | --- |
|  | b. Voice Channel Constraints | 122 |  |
|  | c. Nyquist Theorem | 123 |  |

|  | 24. Telephone Networks | 125 |  |
| --- | --- | --- | --- |

|  | a. POTS - Plain Old Telephone Set | 125 |  |
| --- | --- | --- | --- |
|  | b. Local Loops | 129 |  |
|  | c. Central Office | 131 |  |
|  | d. Hierarchical Phone Networks | 131 |  |

|  | 25. Telephone Line Characteristics | 135 |  |
| --- | --- | --- | --- |

|  | a. Attenuation Distortion | 135 |  |
| --- | --- | --- | --- |
|  | b. Propagation Delay | 137 |  |
|  | c. Envelope Delay Distortion | 139 |  |

|  | 26. Line Impairments | 140 |  |
| --- | --- | --- | --- |

|  | a. Crosstalk | 140 |  |
| --- | --- | --- | --- |
|  | b. Echo or Signal Return | 140 |  |
|  | c. Frequency Shift | 142 |  |
|  | d. Non-Linear Distortion | 142 |  |
|  | e. Jitter: Amplitude and Phase | 143 |  |
|  | f. Transients: Impulse Noise, Gain Hits, Dropouts & Phase Hits | 144 |  |

|  | 27. Modulation Techniques | 147 |  |
| --- | --- | --- | --- |

|  | a. AM - Amplitude Modulation | 147 |  |
| --- | --- | --- | --- |
|  | b. FM - Frequency Modulation | 149 |  |
|  | c. PM - Phase Modulation | 149 |  |

|  | 28. Modem Modulation | 151 |  |
| --- | --- | --- | --- |

|  | a. FSK - Frequency Shift Keying | 151 |  |
| --- | --- | --- | --- |
|  | b. QPSK - Quadrature Phase Shift Keying | 155 |  |
|  | c. QAM - Quadrature Amplitude Modulation | 157 |  |

|  | 29. AT Command Set | 159 |  |
| --- | --- | --- | --- |

|  | a. Basic AT commands | 160 |  |
| --- | --- | --- | --- |

|  | 30. Multiplexing | 161 |  |
| --- | --- | --- | --- |

|  | a. FDM - Frequency Division Multiplexing | 164 |  |
| --- | --- | --- | --- |
|  | b. TDM - Time Division Multiplexing | 166 |  |
|  | c. STDM - Statistical Time Division Multiplexing | 168 |  |

|  | 31. Telecommunication Multiplexing | 168 |  |
| --- | --- | --- | --- |

|  | a. FDM - Channel Groups | 169 |  |
| --- | --- | --- | --- |
|  | b. TDM - T1 Carrier System | 169 |  |

|  | 32. Introduction to the ISO - OSI Model | 172 |  |
| --- | --- | --- | --- |

|  | a. OSI Model Explained | 172 |  |
| --- | --- | --- | --- |
|  | b. Layer 7 - Application Layer | 172 |  |
|  | c. Layer 6 - Presentation Layer | 176 |  |
|  | d. Layer 5 - Session Layer | 177 |  |
|  | e. Layer 4 - Transport Layer | 177 |  |
|  | f. Layer 3 - Network Layer | 179 |  |
|  | g. Layer 2 - Data Link Layer | 179 |  |
|  | h. Layer 1 - Physical Layer | 180 |  |
|  | i. Layer Specific Communication | 181 |  |
|  | j. OSI Model Functional Drawing | 183 |  |

|  | 33. Synchronous Transmission | 185 |  |
| --- | --- | --- | --- |

|  | a. Clocking: Self & Manchester Encoding | 186 |  |
| --- | --- | --- | --- |

|  | 34. Basic Frame Structure | 188 |  |
| --- | --- | --- | --- |

|  | a. Preamble: Starting Delimiter/Alert Burst/Start of Header | 188 |  |
| --- | --- | --- | --- |
|  | b. Address Field(s): Source and/or Destination | 188 |  |
|  | c. Control Field | 190 |  |
|  | d. Data/Message and optional Pad | 190 |  |
|  | e. CRC/ Frame Check Sequence | 190 |  |
|  | f. End Frame Delimiter | 190 |  |

|  | 35. Physical Layer | 192 |  |
| --- | --- | --- | --- |

|  | a. Asynchronous & Synchronous Communication | 192 |  |
| --- | --- | --- | --- |

|  | 36. IEEE-802.3 Protocol | 194 |  |
| --- | --- | --- | --- |

|  | a. CSMA/CD (Carrier Sense Multiple Access/ Collision Detect) | 194 |  |
| --- | --- | --- | --- |
|  | b. IEEE 802.3 Ethernet Media Types | 195 |  |
|  | c. IEEE 802.3 10Base5 | 196 |  |
|  | d. IEEE 802.3a 10Base2 | 200 |  |
|  | e. IEEE 802.3i 10BaseT | 203 |  |
|  | f. MAC - Medium Access Control | 206 |  |
|  | g. Total Length of a MAC Frame | 209 |  |
|  | h. MAC Frame | 211 |  |
|  | i. Packet Sniffing | 212 |  |
|  | j. Packet Sniffing Block Diagram | 216 |  |

|  | 37. IEEE 802.2 LLC - Logical Link Control Layer | 217 |  |
| --- | --- | --- | --- |

|  | a. Service Access Ports (SAPs) | 219 |  |
| --- | --- | --- | --- |
|  | b. Types of LLC Operation | 220 |  |
|  | c. Classes of LLC | 224 |  |
|  | d. LLC PDU Control Field Formats | 224 |  |

|  | 38. Network Interface Cards | 229 |  |
| --- | --- | --- | --- |

|  | a. IRQs, DMAs and Base Addresses | 230 |  |
| --- | --- | --- | --- |
|  | b. Legacy | 234 |  |
|  | c. NIC Diagnostic Tools | 236 |  |
|  | d. Network Interface Card Drivers | 238 |  |

|  | i. NDIS Drivers | 241 |  |
| --- | --- | --- | --- |
|  | ii. ODI Drivers | 243 |  |
|  | iii. Packet Drivers | 245 |  |
|  | iv. Software Interrupts | 245 |  |

|  | 39. Repeaters | 247 |  |
| --- | --- | --- | --- |

|  | a. Purpose of a Repeater | 247 |  |
| --- | --- | --- | --- |
|  | b. Repeater's OSI Operating Layer | 249 |  |
|  | c. Repeater's Segment to Segment Characteristics | 249 |  |
|  | d. Repeater Addressing: MAC Layer and Network Segment | 251 |  |

|  | 40. Hubs | 253 |  |
| --- | --- | --- | --- |

|  | a. Purpose of Hubs | 253 |  |
| --- | --- | --- | --- |
|  | b. Hub's OSI Operating Layer | 255 |  |
|  | c. Hub's Segment to Segment Characteristics | 255 |  |
|  | d. Hub's Addressing | 257 |  |
|  | e. Half-Duplex & Full-Duplex Ethernet Hubs | 257 |  |
|  | f. Switching Hubs | 258 |  |

|  | 41. Bridges | 260 |  |
| --- | --- | --- | --- |

|  | a. Bridge OSI Operating Layer | 260 |  |
| --- | --- | --- | --- |
|  | b. Purpose of a Bridge | 260 |  |
|  | c. Bridge Segment to Segment Characteristics | 263 |  |
|  | d. Bridge Methodologies | 265 |  |
|  | e. Reasons to use a Bridge | 270 |  |
|  | f. Bridge Addressing | 270 |  |
|  | g. Collapsed Backbones | 270 |  |

|  | 42. Routers | 272 |  |
| --- | --- | --- | --- |

|  | a. Purpose of Routers | 272 |  |
| --- | --- | --- | --- |
|  | b. Router OSI Operating Layer | 272 |  |
|  | c. Router Segment to Segment Characteristics | 274 |  |
|  | d. Router Addressing | 276 |  |
|  | e. Routing Protocols | 276 |  |
|  | f. RIP - Routing Information Protocol | 276 |  |
|  | g. EGRP - Exterior Gateway Routing Protocol | 279 |  |
|  | h. OSPF - Open Shortest Path First | 279 |  |

|  | 43. Brouters (Bridge/Routers) | 281 |  |
| --- | --- | --- | --- |

|  | 44. Gateway | 282 |  |
| --- | --- | --- | --- |

|  | a. Gateway's OSI Operating Layer | 282 |  |
| --- | --- | --- | --- |
|  | b. Gateway Segment to Segment Characteristics | 283 |  |
|  | c. Gateway Addressing | 283 |  |

|  | 45. Token Ring | 284 |  |
| --- | --- | --- | --- |

|  | a. IBM Token Ring | 285 |  |
| --- | --- | --- | --- |
|  | b. IEEE 802.4 Token Bus | 286 |  |
|  | c. IEEE 802.5 Token Ring | 286 |  |
|  | d. IEEE 802.5 Bus Arbitration | 286 |  |
|  | e. 4 / 16 Mbps Transfer Rate | 292 |  |
|  | f. IEEE 802.5 Topology | 292 |  |
|  | g. MSAUs | 292 |  |

|  | i. Token Ring connectors | 294 |  |
| --- | --- | --- | --- |
|  | ii. MSAU Relay | 296 |  |
|  | iii. Ring In/ Ring Out | 296 |  |
|  | iv. Wrapping | 298 |  |
|  | v. Physical Star/ Logical Ring | 299 |  |

|  | h. IEEE 802.5 and the OSI Model | 299 |  |
| --- | --- | --- | --- |
|  | i. Token Ring Cabling | 302 |  |

|  | i. Shielded Twisted Pair | 302 |  |
| --- | --- | --- | --- |
|  | ii. Unshielded Twisted Pair - Type 3 | 302 |  |
|  | iii. IBM Cabling System | 303 |  |

|  | j. Ring Insertion | 304 |  |
| --- | --- | --- | --- |
|  | k. CAUs & LAMs | 305 |  |
|  | l. Ring Calculations | 306 |  |

|  | i. Maximum Ring Length | 306 |  |
| --- | --- | --- | --- |
|  | ii. Ring Length Calculations | 306 |  |
|  | iii. Mixing Cables and Ring Length | 307 |  |
|  | iii. Active Concentrators and Ring Length | 309 |  |

|  | m. Token Ring Monitors and Servers | 311 |  |
| --- | --- | --- | --- |

|  | i. Active Monitor (AM) | 311 |  |
| --- | --- | --- | --- |
|  | ii. Standby Monitor (SM) | 316 |  |
|  | iii. Ring Parameter Server (RPS) | 318 |  |
|  | iv. Configuration Report Server (CRS) | 318 |  |
|  | v. Ring Error Monitor (REM) | 320 |  |
|  | vi. Where are these Monitors? | 324 |  |

|  | n. Token Ring Hierarchy | 324 |  |
| --- | --- | --- | --- |
|  | o. IEEE 802.5 Frames | 326 |  |

|  | 46. Linux and Token Ring | 336 |  |
| --- | --- | --- | --- |

|  | 47. Source Routing | 342 |  |
| --- | --- | --- | --- |

|  | 48. ISDN - Integrated Services Digital Network | 344 |  |
| --- | --- | --- | --- |

|  | 49. ADSL - Asymmetrical Digital Subscriber Line | 347 |  |
| --- | --- | --- | --- |

|  | 50. Cable Modems | 350 |  |
| --- | --- | --- | --- |

|  | 51. Quick Introduction to Unix | 352 |  |
| --- | --- | --- | --- |

|  | a. Basic Unix Commands | 359 |  |
| --- | --- | --- | --- |
|  | b. Access and Permissions | 362 |  |
|  | c. Links, Instances & Processes | 365 |  |
|  | d. Background Processing | 369 |  |
|  | e. Shell Programs | 371 |  |
|  | f. Communicating with Other Users | 373 |  |
|  | g. Creating Users and Groups | 375 |  |

|  | 52. SAMBA, Win95, NT and HP Jetdirect | 377 |  |
| --- | --- | --- | --- |

|  | 53. The Suite of TCP/IP Protocols | 387 |  |
| --- | --- | --- | --- |

|  | 54. Internet Protocol | 389 |  |
| --- | --- | --- | --- |

|  | a. IP Addresses | 389 |  |
| --- | --- | --- | --- |
|  | b. IP Address Classifications | 390 |  |

|  | i. Class A addresses | 390 |  |
| --- | --- | --- | --- |
|  | ii. Class B addresses | 390 |  |
|  | iii. Class C addresses | 391 |  |
|  | iv. Class D addresses | 391 |  |
|  | v. Class E addresses | 391 |  |

|  | c. Reserved IP Addresses | 392 |  |
| --- | --- | --- | --- |
|  | d. Network Masking | 393 |  |
|  | e. Domain Names | 398 |  |
|  | f. IP Header | 401 |  |

|  | 55. Address Resolution Protocol (ARP) | 404 |  |
| --- | --- | --- | --- |

|  | 56. Reverse Address Resolution Protocol (RARP) | 406 |  |
| --- | --- | --- | --- |

|  | 57. Internet Control Messaging Protocol (ICMP) | 407 |  |
| --- | --- | --- | --- |

|  | 58. Transmission Control Protocol (TCP) | 416 |  |
| --- | --- | --- | --- |

|  | 59. User Datagram Protocol (UDP) | 420 |  |
| --- | --- | --- | --- |

|  | 60. Simple Network Management Protocol | 422 |  |
| --- | --- | --- | --- |

|  | a. SNMPv2 to the Rescue | 423 |  |
| --- | --- | --- | --- |
|  | b. MIB - Management Information Base | 423 |  |
|  | c. RMON - Remote Network Monitoring | 423 |  |

|  | 61. Handy Unix Network Troubleshooting Commands | 425 |  |
| --- | --- | --- | --- |

|  | 62. X.25 | 429 |  |
| --- | --- | --- | --- |

|  | a. X.25 OSI Layers | 431 |  |
| --- | --- | --- | --- |
|  | b. X.25 High overhead | 433 |  |
|  | c. X.25 Packet Formats | 435 |  |

|  | 63. Frame Relay | 439 |  |
| --- | --- | --- | --- |

|  | a. Decreased Protocol Overhead | 439 |  |
| --- | --- | --- | --- |
|  | b. LAPD - Link Access Protocol D channel | 441 |  |
|  | c. LAN to Frame Relay Connection | 441 |  |

- Appendix

|  | a. PC Block Diagram | 442 |  |
| --- | --- | --- | --- |
|  | b. PC Quick ID Guide | 445 |  |
|  | c. Ethernet Type Field | 463 |  |
|  | d. Ethernet Address Assignments | 466 |  |
|  | e. IP Protocol Address Space | 470 |  |
|  | f. IP Multicast Addresses | 472 |  |
|  | g. IP Header Protocols | 476 |  |
|  | h. IP Hardware Types | 478 |  |
|  | i. TCP/IP Well Known Ports | 479 |  |
|  | j. AT Command Set (Partial listing) | 493 |  |
|  | k. ISO 3166 Country Codes | 497 |  |
|  | l. Token Ring - Major Vector IDs | 499 |  |
|  | m. The GNU General Public License | 502 |  |
|  | n. Copyleft Rules & Regulations | 508 |  |

- pagonline.kiev.ua/docs/www.techbooksforfree.com/intro

- 1. Introduction

This book was written over a period of five years in my spare time while consulting at the Southern Alberta Institute of Technology (SAIT) for various academic departments. Some of the material is reprints of articles that I have written for the Linux Gazette.

When I started consulting in 1994, there were very few books that explored data communications for network computing. The books that I read on data communications only gave a partial view of the "big picture" and tended to assume that the reader had previous knowledge of networking and data communications.

I've tried to sort out the confusing issues in this book and to focus on only the topics of the "moment". I've been successful in the classroom with this approach and hope that you find it meaningful too. I find that most books on the Linux operating system do not cover the data communications aspects of networking. The purpose of this book is to fill this void and introduce the concepts of data communication with a slight leaning towards the Linux operating system.

- 2. Acknowledgements

I would like to thank my wife, Susan, for putting up with my obsessive behaviour while I was writing the original course material. Without her in my life, this book would never had been started. I would especially like to thank Harold Sylven for the support and faith that he has had in me.

I would also like to thank Michael Wilson for his hard work and dedication to the first Area Network Technical Analyst program and who never received the credit that he deserved. I would like to thank Doug Spurgeon who has been my "partner in crime" at SAIT and who I have relied on extensively for his support in Windows NT and Novell Netware.

Lastly but not least, I would like to thank my parents for supporting and guiding me throughout my life.

# 3. Revision List

#### Thursday, May 18, 2000

- Made Rev 1.2 pdf file. Web book and pdf file now match.

#### Sunday, April 23, 2000

- Realized that telnet, http, ftp, tftp and nntp have much better resources available than what I could write. So I nixed them from the book. Now I can work on revising the existing chapters.

#### Saturday, April 22, 2000

- Added a short chapter on UDP.

#### Friday, April 21, 2000

- Added chapter on TCP. ICMP is more interesting...

#### Monday, April 17, 2000

- Added a chapter on ICMP. Quite an interesting protocol. You sure learn alot by writing about stuff...

#### Sunday, April 16, 2000

- Special thanks to Ahmet from Swinburne University for providing an anonymous ftp site :-)

#### Mar 16, 2000

- Received a rude awakening from my ISP. Downloads from my website were costing me an arm and a leg! Took down the website until I could think of some other method of transfer (nonpaying). The big culprit is the pdf file. So for now if anyone wants it, they can email me.

#### Tuesday, Feb 8, 2000

- Fixed page238.html's wrong gif for Class D, added comments courtesy of Jacques Sincennes to Page 58.html, added more info on stop bits page60.html, changed ASDL to ASDL on page 211.html (I'm sure I've done this one before?), corrected ADSL does NOT require modifications to C.O.,

#### Saturday, Feb 5, 2000

- Fixed TOC with tables and page numbers - aaaaaagh! Viewed pdf file there are numerous changes to make to get rid of the wasted pages. At least the pdf is usable now.

#### Saturday, Jan 29, 2000

- Have updated all the webpages with section numbers. Now have to add page numbers to TOC.

#### Thursday, Jan 27, 2000

- Caught a major faux pas in the pdf file. Forgot to add the section numbers to the webpages and page numbers in TOC. Removed pdf file until its corrected. Sigh...

#### Friday, Jan 21, 1980

- Decided to remove the compressed html and pdf files, just left the pdf file. I have limited webspace and my 10 MB quota was being used up real quick on the different compression types (tgz and zip).

#### Thursday, 01/20/100

- Finally generated some tarballed pdf and html files for downloading.

#### Wednesday, January 19, 19100

- Checking all webpages for unique internal links in preparation for generating the pdf file. What a pain in the butt......

#### Tuesday, January 11, 1900

- Changed directory structure of website. Still working on finishing the final stuff and a pdf file.

#### Saturday, December 32, 1999

- No Y2K bugs to report.

#### Wednesday, December 22, 1999

- Added lilo boot info and minor corrections to Linux Token Ring section

#### Saturday, December 04, 1999

- added Frame Relay and SNMP section

#### Monday, November 29, 1999

- - Added X.25 section

- 4. Data Communications

Data Communications is the transfer of data or information between a source and a receiver. The source transmits the data and the receiver receives it. The actual generation of the information is not part of Data Communications nor is the resulting action of the information at the receiver. Data Communication is interested in the transfer of data, the method of transfer and the preservation of the data during the transfer process.

In Local Area Networks, we are interested in "connectivity", connecting computers together to share resources. Even though the computers can have different disk operating systems, languages, cabling and locations, they still can communicate to one another and share resources.

The purpose of Data Communications is to provide the rules and regulations that allow computers with different disk operating systems, languages, cabling and locations to share resources. The rules and regulations are called protocols and standards in Data Communications.

- 5. Why Telecommunications?

What does networking have to do with telephones?

Telephones and networking work hand in hand. The telecommunications industry has been gradually integrating with the computer industry and the computer industry has been gradually integrating with the telecommunications industry. The common goal is to join distantly located Local Area Networks into Metropolitan and Wide Area Networks (MANs and WANs).

- 5a. Voice Channels

First thing that comes to mind is telephone systems and the phone at home. Talking to someone on the phone uses Voice Channels. This doesn't seem to have much to do with Networks!

We do use voice channels for modem communications to connect to BBSs (Bulletin Board Services) or to connect to the Internet. We also use voice channels to connect LANs using remote access. Due to the bandwidth limits on the Voice Channel, the data transfer rate is relatively slow.

Voice Channel: Dial-up connection through a modem using standard telephone lines. Typical Voice Channel communication rates are: 300, 1200, 2400, 9600, 14.4k, 19.2k, 28.8k, 33.6k and 56 kbps (bits per second).

- 5b. Data Channels

- Data channels are dedicated lines for communicating digitized voice and data. At the end of 1996, there was a major milestone where more data was communicated in North America's telecommunications system than voice.

- 5b. Data Channels (cont'd)

Data Channels are special communications channels provided by the "common carriers" such as Telus, Sprint, Bell Canada, AT&T, etc.. for transferring digital data. Data Channels are also called "Leased Lines". They are "directly" connected and you don't have to dial a connection number. The connections are up and running 24 hours per day. They appear as if there were a wire running directly between the source and destination. Typical transfer rates for data communication are: 56 k, 128k, 1.544 M, 2.08 M, 45M and 155 Mbps.

Common carriers charge for data connections by

- the amount of data transferred (megabytes per month)

- the transfer rate (bits per second)

- the amount of use (time per month)

- 6. Introduction to Networking

What is a Network? This is a difficult question to answer. A network can consist of two computers connected together on a desk or it can consist of many Local Area Networks (LANs) connected together to form a Wide Area Network (WAN) across a continent.

The key is that 2 or more computers are connected together by a medium and they are sharing resources. The resources can be files, printers, harddrives or cpu number crunching power.

- 6a. The Big Picture

Many individuals have asked to see The Big Picture of networking: "where does everything fit in?". Where does Microsoft NT fit in with routers and the OSI layers? What about UNIX, Linux and Novell? The following page has a graphic showing The Big Picture. It attempts to show all areas of networking and how they tie into each other. The following key describes the graphical symbols used:

- Circles  Network Operating Systems

- Squares  Communication & cabling protocols (OSI Transport to Physical Layer)

- Storm Clouds  Telecommunications media or Information providers that connect to the Internet

- Machine symbol  Network "linker" can be a Bridge, Router, Brouter or Gateway

- The Internet  jagged haphazard dotted line

| Introduction to Networking (cont'd) | Next |
| --- | --- |

- 6b. Telecommunications Components of The Big Picture

- ISDN  Integrated Services Digital Network

- Private Branch Exchanges  PBXs, Key Systems

- Telcos  AT&T, Bell Telephone, Sprint, Telus

- DataPac & DataRoute  packet switching and analog switching WAN protocols

- Cell Relay  Digital packet switching WAN protocol

- Frame Relay  Digital packet switching WAN protocol

- X.25  Analog packet switching WAN protocol

- ATM  Asynchronous Transfer Mode WAN protocol

- World Wide Web  Hypertext based multimedia system

- ADSL  Asymmetrical digital subscriber line

- 6c. ISO OSI

The International Standards Organization (ISO) Open Systems Interconnect (OSI) is a standard set of rules describing the transfer of data between each layer. Each layer has a specific function. For example the Physical layer deals with the electrical and cable specifications.

The OSI Model clearly defines the interfaces between each layer. This allows different network operating systems and protocols to work together by having each manufacturere adhere to the standard interfaces. The application of the ISO OSI model has allowed the modern multiprotocol networks that exist today. There are 7 Layers of the OSI model:

- 7. Application Layer (Top Layer)

- 6. Presentation Layer

- 5. Session Layer

- 4. Transport Layer

- 3. Network Layer

- 2. Data Link Layer

- 1. Physical Layer (Bottom Layer)

The OSI model provides the basic rules that allow multiprotocol networks to operate. Understanding the OSI model is instrument in understanding how the many different protocols fit into the networking jigsaw puzzle. The OSI model is discussed in detail in Introduction to the ISO - OSI Model.

- 7. Breaking The Big Picture up!

The Big Picture still doesn't give us a good idea of the placement of the many protocols involved in networking and telecommunications. The Big Picture can be broken up according to their protocols into the following 4 areas:

- 7a. Local Loop ,

- 7b. LANs ,

- 7c. MANs and

- 7d. WANs.

- 7a. The Local Loop

The Local Loop is often called "the last mile" and it refers to the last mile of analog phone line that goes from the central office (CO) to your house. Typical local loop protocols are:

- Voice lines

- Modem connections  56 kbps

- ISDN (Integrated Services Digital Network)  2 x 64 kbps digital lines

- ADSL (Asymmetrical Digital Subscriber Line)  up to 8 Mbps

- Cable Modems  up to 30 Mbps

Note: Cable modems are not part of the Local Loop but do fall in the category of "the last mile" or how to get high speed digital communication to the premise (home). It would incredibly expensive to replace the existing cabling structure. All of these protocols are used to overcome the existing cabling limitations in the local loop and provide high speed digital data tranmission. The existing cabling was designed for voice communications and not digital.

- 7b. LANs

LANs (local area networks) are networks that connect computers and resources together in a building or buildings close together.

The components used by LANs can be divided into cabling standards, hardware and protocols. Examples of cabling standards used on LANs are:

- Cat 3, 4 and 5 cables

- IBM Type 19 cabling standards

- EIA568A and 568B

- Ethernet cabling standards: IEEE 802.3 (10Base5), IEEE 802.3a (10Base2), IEEE 802.3i (10BaseT)

- Unshielded Twisted Pair (UTP)

- Shielded Twisted Pair (STP)

- Connectors: RJ45, RJ11, Hermaphroditic connectors, RS232, DB25, BNC, TEE

- 7b. LANs (cont'd)

Examples of hardware devices are:

- Network Interface Cards NICs

- Repeaters

- Ethernet Hubs or multiport repeaters

- Token Ring MultiStation Access Units (MSAUs), Control Access Units (CAUs) and Lobe Access Modules (LAMs)

- Bridges

- Brouters

- Routers

- Gateways

- Print servers

- File servers

- Switches

Examples of LAN protocols are:

- Ethernet frame types: Ethernet_II, Ethernet_SNAP, Ethernet_802.2, Ethernet_802.3

- Media Access Control layer (MAC layer)

- Token Ring: IBM and IEEE 802.5

- Logical Link Control Layer (LLC) IEEE 802.2

- TCP/IP

- SMB, NetBIOS and NetBeui

- IPX/SPX

- Fiber Distributed Data Interchange (FDDI)

- Asynchronous Transfer Mode (ATM)

- 7c. MANs

Metropolitan Area Networks (MANs) are networks that connect LANs together within a city.

The main criteria for a MAN is that the connection between LANs is through a local exchange carrier (the local phone company). The protocols that are used for MANs are quite different from LANs except for ATM which can be used for both under certain conditions.

Examples of MAN protocols are:

- RS232, V35

- X.25 (56kbps), PADs

- Frame Relay (up to 45 Mbps), FRADs

- Asynchronous Transfer Mode (ATM)

- ISDN (Integrated Services Digital Network) PRI and BRI

- Dedicated T1 lines (1.544 Mbps) and Fractional T1

- T3 (45 Mbps) and OC3 lines (155 Mbps)

- ADSL (Asymmetrical Digital Subscriber Line)  up to 8 Mbps

- xDSL (many different types of Digital Subscriber Lines)

- 7d. WAN

Wide Area Networks (WANs) connect LANs together between cities.

The main difference between a MAN and a WAN is that the WAN uses Long Distance Carriers. Otherwise the same protocols and equipment are used as a MAN.

- 8. Trade Magazines

In 1994, TCP/IP was considered dead by many  Unix was considered obsolete. World Wide Web didn't exist as we know it today! Today TCP/IP is the king of network transport protocols! In a matter of months, the computing world completed reversed its direction. The only way to keep current in the computing industry is to read trade publications.

Educational institutes are not able to keep up with the pace of the computing industry. The fast track education cycle takes 6 months to a year to propose, develope and finally run a new course! In that time, there could be major changes or revisions of the product. An excellent example of change is the Linux kernel revisions over the past year.

Anything you read that is over 2 years old is pretty much obsolete! For example: anything you read about fibreoptics that is 3 months old is obsolete. To succeed you must read regularly every trade and computer magazine possible. You just have to skim the magazines and read only the articles that are of interest.

There are many free trade publications available to the computing industry if you qualify. Some examples are:

Free Publications:

- Internetwork

- Computing Canada

- Comnputer Service News

- Communication News

- LAN Computing

- The Computer Paper

Other publications that are worthwhile reading are:

- Byte Magazine

- MacWorld

- PC Computing

- Linux Journal

- LAN magazine

Most trade magazines now offer webpage versions of their magazines on the Internet. In addition, they provide a searchable database of previous articles and programs. Access to the Internet is a necessity if you are going to succeed in the field of network computing. Examples of online resources are:

- Linux Gazette

- Slashdot (news for nerds)

- ZDnet

- Linux Documentation Project

- Linux.org

- 9. The Role of Telecommunications in Networking

From The Big Picture, we see that telecommunications provides a connection service (storm clouds) between networks (circles). Telecommunications provides the external connection service for joining networks across cities, provinces and countries.

- 9a. LANs

Local Area Networks - a system of computers that share resources such as hard-drives, printers, data, CPU power, fax/modem, applications, etc... They usually have distributed processing - means that there is many desktop computers distributed around the network and that there is no central processor machine (mainframe). Can be campus wide like a college or university.

Location: In a building or individual rooms or floors of buildings or nearby buildings.

- 9b. MANs

Metropolitan Area Networks: a system of LANs connected through out a city or metropolitan. MANs are used to connect to other LANs. A MAN has to have the requirement of using a telecommunication media such as Voice Channels or Data Channels. Branch offices are connected to head offices through MANs. Examples of companies that use MANs are universities and colleges, grocery chains and banks.

Location: Separate buildings distributed throughout a city.

- 9c. WANs

Wide Area Networks: a network system connecting cities, countries, continents together. TransCanada Pipeline has a WAN that stretches from Alberta to Boston. It goes from Alberta to Ontario then through the States and ends up in Boston. The maintenance and control of the network resides in Calgary. WANs are connected together using one of the telecommunications media.

Location: City to city, across a country or across a continent.

- 10. Brief History of Networking

The following is a brief history of computers, networking and telecommunication milestones:

1. CRT (Cathode Ray Tube) credited to Braun in 1897

2. Teletype (telegraph 5 bit) during WW1

3. ARQ (Automatic Repeat reQuest) credited to Van Duuren during WWII

- error checking and auto request for retransmission

4. ENIAC credited to DOD / MIT during WWII

- Electronic Numerical Integrator And Calculator

- Used for decoding enemy messages

- 1st generation computer: used vacuum tubes

- Programmed with jumpers and switches

- MTBF (Mean Time Between Failure): 7 minutes

- 337 multiplications per second

5. SAGE (Semi-Automatic Ground Environment) MIT 1950s

- 23 centres for ground/air enemy detection systems

- error checking, keyboard & CRT terminals

- duplexed computers, voice grade (300-4KHz)

- 300 baud, light pens, multiuser system

- magnetic core memory

- Ground to air data Tx

- 1st commercial use was Sabre Reservation System

6. Jacquard's Loom

- First programmable machine

7. Transistorized Computers - 2nd Generation 1960s

- One of the 1st inventors: Cray

- Batch programming: 1 pgm @ a time

- Punch cards

- Stored programs: held in memory

- 50K instructions/second

- ex. IBM 7905

8. CTSS (Compatible Time Sharing System) credited to Cobato/MIT in 1961

- time slices multiusers

9. Synchronous Orbit Communication Satellites. Idea by Arthur C. Clarke in 1945

- Geostationary orbit around equator by Rose/Hughes Aerospace in1963

- 36,000 miles altitude

10. LASER credited to Maiman in 1960

- A narrow band source of optical radiation suitable for use as a carrier of info.

- Light Amplification by Stimulated Emission of Radiation

11. T-1 Carrier System credited to Bell Labs in 1961

- TDM (Time Domain Multiplexing)

- 24 channels = 64 Kbps ea.

- 1.544 Mbps (mega bits per sec)

12. RS232 developed in 1960 and revised since.

- Standard plug and "protocol" convention between modems and machines: 25 pin

- Europe uses V.24 compatible standard

13. Auto Equalization Techniques of Phone lines credited to Lucky et al. in 1965

- adapt to characteristics of telephone line to increase speed

14. Fibre Glass credited to Kao & Hockman in 1966

- proposed "fibre glass " optics developed at Standard Telecom Labs

15. Integrated Circuits Computers - 3rd Generation - 1967

- SSI/MSI (Small Scale Integration/Medium Scale Integration)

- 10 transistors/chip and 100 transistors/chip

- Multi-user systems

- Multitasking

16. Carterfone - FCC Decision in 1968 -

- FCC decision allows other manufacturer's to use phone lines

- opens up competition among phone systems

17. Low-loss Fibre credited to Kapron in 1970

- speeds: 45-90 Mbps developed at Corning Glass Works

- 1984: attained 405-565 Mbps in single mode

- Early 1990s: attained 1.7 Gbps

18. ARPA Network (ARPANET) developed by the DOD in the 1970s

- Advanced Research Projects Agency of the Department of Defence - US

- 1st use of Packet Switching, layered protocols

- Beginning of the Internet

19. VLSI Integration - 4th Generation Computers developed by Intel in 1971

- Very large scale integration: 20,000+ transistors/chip

- Intel 4004 microprocessor - 4 bit

- Grandparent of processors today

20. Layered Network Architecture

- SNA: System Network Architecture IBM Mainframe

- DNA: Digital Network Architecture DEC for DECNET

21. Ethernet developed by Xerox in 1974 -

- Ether is the mysterious invisible fluid that transfers heat

- Originally based on the ALOHA radio protocol

22. Videotex developed by Teletel (France) in the 1980s

- Interactive video Minitel

23. Reference Model for Open Systems Interconnect developed by the ISO in 1983

- Continuously evolving model for layering network protocols

24. AT&T Divestiture in 1984 -

- Break-up of AT&T monopoly into Baby Bells

25. ISDN developed in 1984 -

- Integrated Services Digital Network

- Strong in Europe

- A network evolving from a telephony integrated digital network supporting: voice, teletex, videotex, fax, slowscan video, etc..

- 26. Linux Version 0.01 released Sept 17, 1991

- 11. Data Communication Network

The major criteria that a Data Communication Network must meet are:

- 11a. Performance

- 11b. Consistency

- 11c. Reliability,

- 11d. Recovery and

- 11e. Security

- 11a. Performance

Performance is the defined as the rate of transferring error free data. It is measured by the Response Time. Response Time is the elasped time between the end of an inquiry and the beginning of a response. Request a file transfer and start the file transfer. Factors that affect Response Time are:

- Number of Users: More users on a network - slower the network will run

- Transmission Speed: speed that data will be transmitted measured in bits per second (bps)

- Media Type: Type of physical connection used to connect nodes together

- Hardware Type: Slow computers such as XT or fast such as Pentiums

- Software Program: How well is the network operating system (NOS) written

- 11b. Consistency

Consistency is the predictability of response time and accuracy of data.

- Users prefer to have consistent response times, they develop a feel for normal operating conditions. For example: if the "normal" response time is 3 sec. for printing to a Network Printer and a response time of over 30 sec happens, we know that there is a problem in the system!

- Accuracy of Data determines if the network is reliable! If a system loses data, then the users will not have confidence in the information and will often not use the system.

- 11c. Reliability

- Reliability is the measure of how often a network is useable. MTBF (Mean Time Between Failures) is a measure of the average time a component is expected to operate between failures. Normally provided by the manufacturer. A network failure can be: hardware, data carrying medium and Network Operating System.

- 11d. Recovery

Recovery is the Network's ability to return to a prescribed level of operation after a network failure. This level is where the amount of lost data is nonexistent or at a minimum. Recovery is based on having Back-up Files.

- 11e. Security

Security is the protection of Hardware, Software and Data from unauthorized access. Restricted physical access to computers, password protection, limiting user privileges and data encryption are common security methods. Anti-Virus monitoring programs to defend against computer viruses are a security measure.

- 11f. Applications

The following lists general applications of a data communication network:

- Electronic Mail (e-mail or Email) replaces snail mail. E-mail is the forwarding of electronic files to an electronic post office for the recipient to pick up.

- Scheduling Programs allow people across the network to schedule appointments directly by calling up their fellow worker's schedule and selecting a time!

- Videotext is the capability of having a 2 way transmission of picture and sound. Games like Doom, Hearts, distance education lectures, etc..

- Groupware is the latest network application, it allows user groups to share documents, schedules databases, etc.. ex. Lotus Notes.

- Teleconferencing allows people in different regions to "attend" meetings using telephone lines.

- Telecommuting allows employees to perform office work at home by "Remote Access" to the network.

- Automated Banking Machines allow banking transactions to be performed everywhere: at grocery stores, Drive-in machines etc..

- Information Service Providers: provide connections to the Internet and other information services. Examples are Compuserve, Genie, Prodigy, America On-Line (AOL), etc...

- Electronic Bulletin Boards (BBS - Bulletin Board Services) are dialup connections (use a modem and phone lines) that offer a range of services for a fee.

- Value Added Networks are common carriers such as AGT, Bell Canada, etc.. (can be private or public companies) who provide additional leased line connections to their customers. These can be Frame Relay, ATM (Asynchronous Transfer Mode), X.25, etc.. The leased line is the Value Added Network.

- 11g. Basic Components

Source: It is the transmitter of data. Examples are:

- Terminal,

- Computer,

- Mainframe

Medium: The communications stream through which the data is being transmitted. Examples are:

- Cabling,

- Microwave,

- Fibre optics,

- Radio Frequencies (RF),

- Infrared Wireless

Receiver: The receiver of the data transmitted. Examples are:

- Printer,

- Terminal,

- Mainframe,

- Computer,

DCE: The interface between the Source & the Medium, and the Medium & the Receiver is called the DCE (Data Communication Equipment) and is a physical piece of equipment.

DTE: Data Terminal Equipment is the Telecommunication name given to the Source and Receiver's equipment.

An example of this would be your PC dialing into a BBS (Bulletin Board System):

- 12. Data Flow

Data flow is the flow of data between 2 points. The direction of the data flow can be described as:

Simplex: data flows in only one direction on the data communication line (medium). Examples are Radio and Television broadcasts. They go from the TV station to your home television.

Half-Duplex: data flows in both directions but only one direction at a time on the data communication line. Ex. Conversation on walkie-talkies is a half-duplex data flow. Each person takes turns talking. If both talk at once - nothing occurs!

Bi-directional but only 1 direction @ a time!

HALF-DUPLEX

Full-Duplex: data flows in both directions simultaneously. Modems are configured to flow data in both directions.

Bi-directional both directions simultaneously!

FULL-DUPLEX

- 13. Modems

A modem is a Modulator/Demodulator, it connects a terminal/computer (DTE) to the Voice Channel (dial-up line).

- 13a. Basic Definition

The modem (DCE - Data Communication Equipment) is connected between the terminal/computer (DTE - Data Terminal Equipment) and the phone line (Voice Channel). A modem converts the DTE (Data Terminal Equipment) digital signal to an analog signal that the Voice Channel can use.

A modem is connected to the terminal/computer's RS232 serial port (25 pin male D connector) and the outgoing phone line with an RJ11 cable connector (same as on a phone extension cord). Male connectors have pins, female connectors have sockets.

- 13b. Digital Connection

The connection between the modem and terminal/computer is a digital connection. A basic connection consists of a Transmit Data (TXD) line, a Receive Data (RXD) line and many hardware hand-shaking control lines.

The control lines determine: whose turn it is to talk (modem or terminal), if the terminal/computer is turned on, if the modem is turned on, if there is a connection to another modem, etc..

- 13c. Analog Connection

The connection between the modem and outside world (phone line) is an analog connection. The Voice Channel has a bandwidth of 0-4 kHz but only 300 - 3400 Hz is usable for data communications.

The modem converts the digital information into tones (frequencies) for transmitting through the phone lines. The tones are in the 300-3400 Hz Voice Band.

- 13d. External/Internal Modems

There are 2 basic physical types of modems: Internal & External modems. External modems sit next to the computer and connect to the serial port using a straight through serial cable.

Internal modems are a plug-in circuit board that sits inside the computer. It incorporates the serial port on-board. They are less expensive than external modems because they do not require a case, power supply and serial cable. They appear to the communication programs as if they were an external modem for all intensive purposes.

- 13e. Modem Types

There are many types of modems, the most common are:

- Optical Modems
Uses optical fibre cable instead of wire. The modem converts the digital signal to pulses of light to be transmitted over optical lines. (more commonly called a media adapter or transceiver)

- Short Haul Modems
Modems used to transmit over 20 miles or less. Modems we use at home or to connect computers together between different offices in the same building.

- Acoustic Modem
A modem that coupled to the telephone handset with what looked like suction cups that contained a speaker and microphone. Used for connecting to hotel phones for travelling salespeople.

- Smart Modem
Modem with a CPU (microprocessor) on board that uses the Hayes AT command set. This allows auto-answer & dial capability rather than manually dialing & answering.

- Digital Modems
Converts the RS-232 digital signals to digital signals more suitable for transmission. (also called a media adapter or transceiver)

- V.32 Modem
Milestone modem that used a 2400 Baud modem with 4 bit encoding. This results in a 9600 bps (bits per second) transfer rate. It brought the price of high speed modems below $5,000.

Baud is the speed at which the Analog data is changing on the Voice Channel and bps is the speed that the decoded digital data is being transferred.

- 13f. Features of Modems

- Speed
The speed at which the modem can send data in bps (bits per second). Typically modem speeds are: 300, 600, 1200, 2400, 4800, 9600, 14.4K, 19.2K, 28.8K bps

- Auto Dial /Redial
Smart Modems can dial the phone number and & auto redial if a busy signal is received.

- Auto Answer
Most modems can automatically answer the phone when an incoming call comes in. They have Ring Detect capability.

- Self-Testing
New modems have self-testing features. They can test the digital connection to the terminal /computer and the analog connection to a remote modem. They can also check the modem's internal electronics.

- Voice over Data
Voice over Data modems allow a voice conversation to take place while data is being transmitted. This requires both the source and destination modems to have this feature.

- Synchronous or Asynchronous Transmission
Newer modems allow a choice of synchronous or asynchronous transmission of data. Normally, modem transmission is asynchronous. We send individual characters with just start and stop bits. Synchronous transmission or packet transmission is used in specific applications.

- 13g. Modem Speeds / Standards

| Bell 103 | 300 bps FSK -Half duplex |
| --- | --- |
| Bell 113 | 300 bps FSK - Full duplex |
| Bell 202 | 1200 baud half duplex |
| Bell 212A | 1200 bps DPSK (Dibit Phase Shift Keying) - V.22 compatible 300 bps FSK (Frequency Shift Keying) - NOT V.22 compatible |
| MNP1-3 | Microcon Networking Protocol - Basic error detection and control of errors. |
| MNP4 | Error correction + adapts to line conditions. |
| MNP5 | Error correction + adapts to line conditions and adds Compression technique used to double the data transfer rate. |
| RS-232D | Cable and connector standard |
| V.22 | 1200 bps DPSK (Dibit Phase Shift Keying) - Bell 212A compatible 600 bps PSK (Phase Shift Keying) - NOT Bell 212A compatible |
| V.22bis | 2400 bps - International Standard Fallback in Europe to V.22 Fallback in America to Bell 212A |
| V.24 | European Mechanical specifications for RS-232D |
| V.26 . | Synchronous 2400 bps modem 1200 bps DPSK full duplex |
| V.27 | Synchronous 4800 bps DPSK modem |
| V.28 | European Electrical specifications for RS-232D |
| V.29 | Synchronous 9600 bps QAM |
| V.32 | 9600 bps QAM |
| V.32bis | 14.4 Kbs QAM1 |
| V.33 | 14.4 Kbps Trellis Coded Modulation for noise immunity. |
| V.34 | 28.8 Kbps modem standard |
| V.34bis | 33.6 Kbps modem standard |
| V.42bis | Compression technique to roughly double the data transfer rate. Uses Automatic Repeat Request ARQ and CRC (Cyclic Redundancy Checking) |
| WE201 | Synchronous Western Electric 2400 bps DPSK |
| WE208 | Synchronous 4800 bps DPSK |
| WE209 | Synchronous 9600 bps |

- 13h. Transfer Rate versus PC Bus Speed

The lowliest XT PC can out-perform the fastest modem transfer rate. For example: an XT has an 8 bit parallel expansion bus operating at 4.77 MHz. This equates to a data transfer rate of:

8 bits x 4.77 MHz = 38.16 Mbps

Compare this to the fastest modem transfer rates of 57.6 kbps!

- 14. Physical Connection

The physical connection determines how many bits (1's or 0's) can be transmitted at a single instance of time. If only 1 bit of information can be transmitted over the data transmission medium at a time then it is considered a Serial Communication.

If more than 1 bit of information is transmitted over the data transmission medium at a time then it is considered a Parallel Communication.

| Communications | Advantages | Disadvantages |
| --- | --- | --- |
| Parallel | Fast Transfer Rates | Short distances only |
| Serial | Long Distances | Slow transfer rates |

- 15. Transmission Media - Guided

There are 2 basic categories of Transmission Media:

- Guided and

- Unguided.

Guided Transmission Media uses a "cabling" system that guides the data signals along a specific path. The data signals are bound by the "cabling" system. Guided Media is also known as Bound Media. Cabling is meant in a generic sense in the previous sentences and is not meant to be interpreted as copper wire cabling only.

Unguided Transmission Media consists of a means for the data signals to travel but nothing to guide them along a specific path. The data signals are not bound to a cabling media and as such are often called Unbound Media.

There 4 basic types of Guided Media:

- Open Wire

- Twisted Pair

- Coaxial Cable

- Optical Fibre

- 15a. Open Wire

Open Wire is traditionally used to describe the electrical wire strung along power poles. There is a single wire strung between poles. No shielding or protection from noise interference is used. We are going to extend the traditional definition of Open Wire to include any data signal path without shielding or protection from noise interference. This can include multiconductor cables or single wires. This media is susceptible to a large degree of noise and interference and consequently not acceptable for data transmission except for short distances under 20 ft.

- 15b. Twisted Pair

The wires in Twisted Pair cabling are twisted together in pairs. Each pair would consist of a wire used for the +ve data signal and a wire used for the -ve data signal. Any noise that appears on 1 wire of the pair would occur on the other wire. Because the wires are opposite polarities, they are 180 degrees out of phase (180 degrees - phasor definition of opposite polarity). When the noise appears on both wires, it cancels or nulls itself out at the receiving end. Twisted Pair cables are most effectively used in systems that use a balanced line method of transmission: polar line coding (Manchester Encoding) as opposed to unipolar line coding (TTL logic).

Unshielded Twisted Pair

The degree of reduction in noise interference is determined specifically by the number of turns per foot. Increasing the number of turns per foot reduces the noise interference. To further improve noise rejection, a foil or wire braid shield is woven around the twisted pairs. This "shield" can be woven around individual pairs or around a multi-pair conductor (several pairs).

Shielded Twisted Pair

Cables with a shield are called Shielded Twisted Pair and commonly abbreviated STP. Cables without a shield are called Unshielded Twisted Pair or UTP. Twisting the wires together results in a characteristic impedance for the cable. A typical impedance for UTP is 100 ohm for Ethernet 10BaseT cable.

UTP or Unshielded Twisted Pair cable is used on Ethernet 10BaseT and can also be used with Token Ring. It uses the RJ line of connectors (RJ45, RJ11, etc..)

STP or Shielded Twisted Pair is used with the traditional Token Ring cabling or ICS - IBM Cabling System. It requires a custom connector. IBM STP (Shielded Twisted Pair) has a characteristic impedance of 150 ohms.

- 15c. Coaxial Cable

Coaxial Cable consists of 2 conductors. The inner conductor is held inside an insulator with the other conductor woven around it providing a shield. An insulating protective coating called a jacket covers the outer conductor.

Coax Cable

The outer shield protects the inner conductor from outside electrical signals. The distance between the outer conductor (shield) and inner conductor plus the type of material used for insulating the inner conductor determine the cable properties or impedance. Typical impedances for coaxial cables are 75 ohms for Cable TV, 50 ohms for Ethernet Thinnet and Thicknet. The excellent control of the impedance characteristics of the cable allow higher data rates to be transferred than Twisted Pair cable.

- 15d. Optical Fibre

Optical Fibre consists of thin glass fibres that can carry information at frequencies in the visible light spectrum and beyond. The typical optical fibre consists of a very narrow strand of glass called the Core. Around the Core is a concentric layer of glass called the Cladding. A typical Core diameter is 62.5 microns (1 micron = 10-6 meters). Typically Cladding has a diameter of 125 microns. Coating the cladding is a protective coating consisting of plastic, it is called the Jacket.

An important characteristic of Fibre Optics is Refraction. Refraction is the characteristic of a material to either pass or reflect light. When light passes through a medium, it "bends" as it passes from one medium to the other. An example of this is when we look into a pond of water.

If the angle of incidence is small, the light rays are reflected and do not pass into the water. If the angle of incident is great, light passes through the media but is bent or refracted.

Optical Fibres work on the principle that the core refracts the light and the cladding reflects the light. The core refracts the light and guides the light along its path. The cladding reflects any light back into the core and stops light from escaping through it - it bounds the media!

Optical Transmission Modes

There are 3 primary types of transmission modes using optical fibre. They are

- Step Index

- Grade Index

- Single Mode

Step Index has a large core the light rays tend to bounce around, reflecting off the cladding, inside the core. This causes some rays to take a longer or shorted path through the core. Some take the direct path with hardly any reflections while others bounce back and forth taking a longer path. The result is that the light rays arrive at the receiver at different times. The signal becomes longer than the original signal. LED light sources are used. Typical Core: 62.5 microns.

Step Index Mode

Grade Index has a gradual change in the Core's Refractive Index. This causes the light rays to be gradually bent back into the core path. This is represented by a curved reflective path in the attached drawing. The result is a better receive signal than Step Index. LED light sources are used. Typical Core: 62.5 microns.

Grade Index Mode

Note: Both Step Index and Graded Index allow more than one light source to be used (different colours simultaneously!). Multiple channels of data can be run simultaneously!

Single Mode has separate distinct Refractive Indexes for the cladding and core. The light ray passes through the core with relatively few reflections off the cladding. Single Mode is used for a single source of light (one colour) operation. It requires a laser and the core is very small: 9 microns.

Single Mode

### Comparison of Optical Fibres

The Wavelength of the light sources is measured in nanometers or 1 billionth of a meter. We don't use frequency to talk about speed any more, we use wavelengths instead.

Indoor cable specifications:

- LED (Light Emitting Diode) Light Source

- 3.5 dB/Km Attenuation (loses 3.5 dB of signal per kilometre)

- 850 nM - wavelength of light source

- Typically 62.5/125 (core dia/cladding dia)

- Multimode - can run many light sources.

Outdoor Cable specifications:

- Laser Light Source

- 1 dB/Km Attenuation (loses 1 dB of signal per kilometre)

- 1170 nM - wavelength of light source

- Monomode (Single Mode)

Advantages of Optical Fibre:

- Noise immunity: RFI and EMI immune (RFI - Radio Frequency Interference, EMI -ElectroMagnetic Interference)

- Security: cannot tap into cable.

- Large Capacity due to BW (bandwidth)

- No corrosion

- Longer distances than copper wire

- Smaller and lighter than copper wire

- Faster transmission rate

Disadvantages of Optical Fibre:

- Physical vibration will show up as signal noise!

- Limited physical arc of cable. Bend it too much & it will break!

- Difficult to splice

The cost of optical fibre is a trade-off between capacity and cost. At higher transmission capacity, it is cheaper than copper. At lower transmission capacity, it is more expensive.

- 15e. Media versus Bandwidth

The following table compares the usable bandwidth between the different Guided Transmission Media

| Cable Type | Bandwidth |
| --- | --- |
| Open Cable | 0 - 5 MHz |
| Twisted Pair | 0 - 100 MHz |
| Coaxial Cable | 0 - 600 MHz |
| Optical Fibre | 0 - 1 GHz |

- 16. Transmission Media - Unguided

Unguided Transmission Media is data signals that flow through the air. They are not guided or bound to a channel to follow. They are classified by the type of wave propagation.

- 16a. RF Propagation

There are 3 types of RF (Radio Frequency) Propagation:

- Ground Wave,

- Ionospheric and

- Line of Sight (LOS) Propagation.

Ground Wave Propagation follows the curvature of the Earth. Ground Waves have carrier frequencies up to 2 MHz. AM radio is an example of Ground Wave Propagation.

Ionospheric Propagation bounces off of the Earths Ionospheric Layer in the upper atmosphere. It is sometimes called Double Hop Propagation. It operates in the frequency range of 30 - 85 MHz. Because it depends on the Earth's ionosphere, it changes with weather and time of day. The signal bounces off of the ionosphere and back to earth. Ham radios operate in this range.

Line of Sight Propagation transmits exactly in the line of sight. The receive station must be in the view of the transmit station. It is sometimes called Space Waves or Tropospheric Propagation. It is limited by the curvature of the Earth for ground based stations (100 km: horizon to horizon). Reflected waves can cause problems. Examples of Line of Sight Propagation are: FM Radio, Microwave and Satellite.

- 16b. Radio Frequencies

The frequency spectrum operates from 0 Hz (DC) to Gamma Rays (1019 Hz).

| Name | Frequency (Hertz) | Examples |
| --- | --- | --- |
| Gamma Rays | 10^19 + |  |
| X-Rays | 10^17 |  |
| Ultra-Violet Light | 7.5 x 10^15 |  |
| Visible Light | 4.3 x 10^14 |  |
| Infrared Light | 3 x 10^11 |  |
| EHF - Extremely High Frequencies | 30 GHz (Giga = 10^9) | Radar |
| SHF - Super High Frequencies | 3 GHz | Satellite & Microwaves |
| UHF - Ultra High Frequencies | 300 MHz (Mega = 10^6) | UHF TV (Ch. 14-83) |
| VHF - Very High Frequencies | 30 MHz | FM & TV (Ch2 - 13) |
| HF - High Frequencies | 3 MHz2 | Short Wave Radio |
| MF - Medium Frequencies | 300 kHz (kilo = 10^3) | AM Radio |
| LF - Low Frequencies | 30 kHz | Navigation |
| VLF - Very Low Frequencies | 3 kHz | Submarine Communications |
| VF - Voice Frequencies | 300 Hz | Audio |
| ELF - Extremely Low Frequencies | 30 Hz | Power Transmission |

Radio Frequencies are in the range of 300 kHz to 10 GHz. We are seeing an emerging technology called wireless LANs. Some use radio frequencies to connect the workstations together, some use infrared technology.

- 16c. Microwave

Microwave transmission is line of sight transmission. The Transmit station must be in visible contact with the receive station. This sets a limit on the distance between stations depending on the local geography. Typically the line of sight due to the Earth's curvature is only 50 km to the horizon! Repeater stations must be placed so the data signal can hop, skip and jump across the country.

Microwaves operate at high operating frequencies of 3 to 10 GHz. This allows them to carry large quantities of data due to the large bandwidth.

Advantages:

- They require no right of way acquisition between towers.

- They can carry high quantities of information due to their high operating frequencies.

- Low cost land purchase: each tower occupies small area.

- High frequency/short wavelength signals require small antenna.

Disadvantages:

- Attenuation by solid objects: birds, rain, snow and fog.

- Reflected from flat surfaces like water and metal.

- Diffracted (split) around solid objects

- Refracted by atmosphere, thus causing beam to be projected away from receiver.

- 16d. Satellite

Satellites are transponders that are set in a geostationary orbit directly over the equator. A transponder is a unit that receives on one frequency and retransmits on another. The geostationary orbit is 36,000 km from the Earth's surface. At this point, the gravitational pull of the Earth and the centrifugal force of Earths rotation are balanced and cancel each other out. Centrifugal force is the rotational force placed on the satellite that wants to fling it out to space.

| 16. Transmission Media - Unguided (cont'd) | Next |
| --- | --- |

The uplink is the transmitter of data to the satellite. The downlink is the receiver of data. Uplinks and downlinks are also called Earth stations due to be located on the Earth. The footprint is the "shadow" that the satellite can transmit to. The shadow being the area that can receive the satellite's transmitted signal.

- 16e. Iridium Telecom System

The Iridium telecom system is a new satellite sytem that will be the largest private aerospace project. It is a mobile telecom system to compete with cellular phones. It relies on satellites in Lower Earth Orbit (LEO). The satellites will orbit at an altitude of 900 - 10,000 km and are a polar non-stationary orbit. They are planning on using 66 satellites. The user's handset will require less power and will be cheaper than cellular phones. There will be 100% coverage of the Earth.

They were planning to launch starting 1996-1998 and having 1.5 million subscribers by end of the decade. Unfortunately at the time of this writing, the Iridium project looked very financially unstable.

- 17. RS-232D Serial Interface Standard

The RS-232D Serial Interface Standard added the mechanical characteristics to the RS-232C Standard. The RS-232D standard defines:

- The Mechanical Characteristics of the Interface

- The Electrical Characteristics of the Interface

- The Function of Each Signal

- Subsets of the Signals for Certain Applications

The European version of RS-232D is defined in:

- V.24 - Mechanical Standard

- V.28 - Electrical Standard

- 17a. Mechanical Characteristics of the RS-232D

Mechanical Characteristics of the RS-232D Interface defines:

- The connector is a DB25 connector. DB9 is not universally accepted.

- The connector gender is Male at the DTE and Female at the DCE.

- The assignments of signals to pins

- The maximum cable length is 50 ft.

- The maximum cable capacitance = 2500 pF. Typical cable has 50 pF/foot capacitance.

- 17b. Electrical Characteristics of the RS-232D

Electrical Characteristics of the RS-232D Interface defines:

The transmitter side generates a voltage between +5 and +25 Volts for a Space (digital 0 or Low) and generates a voltage between -5 and -25 Volts for a Mark (digital 1 or High).

The receiving side recognizes a Space (digital 0 or Low) as any voltage between +3 and +25V and a Mark (digital 1 or High) as any voltage between -3 and -25V. The standard allows for a voltage loss through the cable and noise immunity by reducing the receive requirements to +/-3 Volts!

- 17c. Function of Each Signal

| Pin | Name | Description | EIA Circuit |
| --- | --- | --- | --- |
| 1 | GND | Chassis ground | AA |
| 2 | TXD | Transmit Data (TXD) | BA |
| 3 | RXD | Receive Data (RXD) | BB |
| 4 | RTS | Ready to Send | CA |
| 5 | CTS | Clear to Send | CB |
| 6 | DSR | Data Set Ready (DCE Ready) | CC |
| 7 | SGND | Signal ground | AB |
| 8 | DCD | Carrier Detect (CD or RLSD) (RLSD - Received Line Signal Detector) | CF |
| 9 | n/u |  |  |
| 10 | n/u |  |  |
| 11 | n/u |  |  |
| 12 | DCD2 | Secondary Carrier Detect (SRLSD) | SCF |
| 13 | CTS2 | Secondary Clear to Send | SCB |
| 14 | TXD2 | Secondary Transmit Data | SBA |
| 15 | TxSigC | Transmitter Signal Element Timing - DCE | DB |
| 16 | RXD2 | Secondary Receive Data | SBB |
| 17 | RxSig | Receive Signal Element Timing - DCE | DD |
| 18 | LL | Local Loopback |  |
| 19 | RTS2 | Secondary Ready to Send | SCA |
| 20 | DTR | Data Terminal Ready (DTE Ready) | CD |
| 21 | SQ/RL | Signal Quality/Remote Loopback | CG |
| 22 | RI | Ring Indicator | CE |
| 23 | DSRS | Data Signal Rate Selector | CH/CI |
| 24 | TxSigT | Transmitter Signal Element Timing - DTE | DA |
| 25 | TM | Test Mode |  |

The signals in Bold/Italic are required for a basic asynchronous modem connection.

- 17d. Subsets of Signals for Certain Applications

Data Signals

| 2 | TXD | Transmit Data Data generated by DTE | BA |
| --- | --- | --- | --- |
| 3 | RXD | Receive Data Data generated by DCE | BB |

Control Signals

| 4 | RTS | Ready to Send | DTE wishes to transmit |
| --- | --- | --- | --- |
| 5 | CTS | Clear to Send | DCE ready to receive |
| 6 | DSR | Data Set Ready (DCE Ready) | DCE powered on & ready to go |
| 20 | DTR | Data Terminal Ready (DTE Ready) | DTE powereded on & ready to go |
| 22 | RI | Ring Indicator | Phones ringing |

Test Modes

| 18 | LL | Local Loopback | Initiate Local Loopback Self-Test |
| --- | --- | --- | --- |
| 21 | SQ/RL | Signal Quality/Remote Loopback | Initiate Remote Loopback Self-Test |
| 25 | TM | Test Mode | Initiate Test Mode |

Synchronous Control Signals

| 21 | SQ/RL | Signal Quality/Remote Loopback | Error in received data! |
| --- | --- | --- | --- |
| 23 | DSRS | Data Signal Rate Selector | DTE can dynamically select 1 of 2 data rates |

Synchronous Timing Signals

| 15 | TxSigC | Transmitter Signal Element Timing - DCE | DCE generated |
| --- | --- | --- | --- |
| 17 | RxSig | Receive Signal Element Timing - DCE | DCE generated |
| 24 | TxSigT | Transmitter Signal Element Timing - DTE | DTE generated |

Ground/Shield

| 1 | GND | Chassis ground | Shield DTE side only for noise protection. Do NOT connect to signal ground! |
| --- | --- | --- | --- |
| 7 | SGND | Signal ground | Signal return path |

- 18. RS-232D Flow Control

Flow control is the communication between the data transmitter and data receiver to determine whose turn it is to talk. Another name for flow control is handshaking. Flow control is the exchange of predetermined codes and signals between two devices to establish and maintain a connection.

Modem flow control is used between the PC and modem to determine if the modem is ready to receive data from the terminal, if carrier is present, if the line is ringing, etc....

Source to Destination (End to End) flow control

Source to destination flow control is used to control the data communication from the sendor to the receiver. It may or may not have a modem involved. Source to destination may involve direct PC to PC communication or PC to Serial Printer communication. It is also called end to end flow control.

DTE-DCE Flow Control

There are 2 basic types of DTE-DCE Flow Control used with RS-232D connections:

- Hardware handshaking

- Software handsh

- 18a. Hardware Handshaking

- Hardware Handshaking uses the physical signals in the RS-232D cable such as RTS, CTS, DSR and TSR to control the flow of data. Hardware Handshaking is used primarily with modems: PC to modem connection or terminal to modem connection.

PC to Modem Handshaking (DTE-DCE)

The basic signals required for DTE-DCE Hardware Handshaking are:

The following diagram indicates the signals used when two PCs are communicating using hardware handshaking.

The procedure for connecting between 2 PCs using modems and the telephone line is as follows:

DTE(Tx) is originating the call and DTE(Rx) is answering the call. DTE(Rx) is in the auto-answer mode with DTR(Rx) and DSR(Rx) High is ready to answer a call.

1. The communication program controls the handshaking. The DTE (Tx) dials the phone number:

| a. | PC sends DTR(Tx) | - PC is awake! |
| --- | --- | --- |
| b. | Modem replies with DSR(Tx) | - Modem is awake, too! |
| c. | PC sends RTS(Tx) | - Ready for some data? |
| d. | Modem replies with CTS(Tx) | - Okay send away! |
| e. | PC transmits data on TXD | - Initialize dial this telephone number. |

2. DTE(Rx) is in the auto-answer mode with DTR(Rx) and DSR(Rx) High, indicating the receive end is ready to answer a call. This has been setup by the communication program similar to dialing the number in the previous step except the modem is told to go to auto-answer mode. The phone rings:

| a. | Modem sends RI(Rx) | - Hey the phone's ringing! |
| --- | --- | --- |
| b. | Modem picks up phone line |  |
| c. | Modem sends answer carrier |  |

The modem since it was initialized in the auto-answer mode, picks up the phone line and sends Answer Carrier (2125 Hz). Everytime the phone rings, the RI line goes high. The communication program will usually display the word "ring" on the screen.

3. Back at the Transmit End:

| a. | Modem sends CD(Tx) | - We're connected and they are sending us good carrier! |
| --- | --- | --- |
| b. | PC sends RTS(Tx) | - Okay send them our carrier (1170 Hz). |
| c. | Modem waits | - Delay so that Rx modem can lock to the carrier |
| d. | Modem sets CTS(Tx) | - Okay now we should be ready to send data |
| e. | PC sends TxD(Tx) | - Here's some data to send over. |

4. At the Receive End:

| a. | Modem sends CD(Rx) | - We're connected & they're sending good carrier (1170 Hz |
| --- | --- | --- |
| b. | Modem sends Rxd(Rx) | - Here's some data for you. |

The communication program then interprets the data and decides if a reply is required or if more data is coming. The communication programs handle the transfer of the data and the direction.

5. Both Originate or Answer can end the communication:

| a. | DTE(Tx) drops RTS(Tx) or DTR(Tx) | - I'm done, hang-up the phone. |
| --- | --- | --- |
| b. | DCE(Tx) modem drops DSR(Tx) and the Carrier (1170 Hz) | - I've disconnected. |
| c. | DCE(Rx) modem drops CD(Rx) | - No carrier, they're hanging up |
| d. | DTE(Rx) drops RTS(Rx) | - Hang up on them |
| e. | DCE(Rx) modem drops DSR(Rx) and the Carrier (2125 Hz) | - I've disconnected. |

- 18b. Hardware Null Modems

Null modems are a way of connecting 2 DTEs together without using a modem - we are nulling out the modems. This gives way to the term Null Modem. When 2 DTEs are connected together, the TXD Pin2 of one DTE is crossed to Pin 3 RXD of the other DTE. We also have to fool the DTEs into believing that they are connected to DCE devices. This is done by crossing the control lines as follows:

Notice that RI (Ring Indicator) and CD (Carrier Detect) are not used when connecting directly from DTE to DTE. They are a function of a telephone system and by nulling out the modems, we've eliminated the telephone system. This can cause problems when transferring files directly because most communication programs detect loss of carrier (CD) as a disconnect command. The communication program will abort the data transfer if CD is not present. This can usually be over-ridden by de-selecting "Transfer Aborted if CD Lost" (or something similar) in one of the communication software configuration menus.

- 18c. Software Handshaking (Xon/Xoff)

Software Handshaking does not use the RS-232D control signals, it uses the software commands Xon/Xoff to control the data flow. Do not use software handshaking with a modem, because you will lose several important function of the modem such as: RI, and CD.

| Xon | Transmit On | - ASCII Character DC1 |
| --- | --- | --- |
| Xoff | Transmit Off | - ASCII Character DC2 |

Software handshaking is a simple flow control method that is used mainly with DTE to DTE and DTE to Serial Printer connections. The receiving device controls the flow of data by issuing Xon (okay to transmit data) commands and Xoff (stop - let me catch up) commands. A good example is the DTE to Serial Printer connection.

For example, a dot-matrix printer cannot physically print faster than a transfer rate of 300 bps. Printers are usually equipped with a memory buffer to store the data before it is printed. The printer buffer allows large chunks of data to be downloaded to the printer from the DTE, thereby freeing up the DTE to do other tasks rather than wait for a page to be printed.

When the data is first being downloaded to the printer, the printer issues a Xon command to the DTE. As the print buffer becomes full (90%), the printer issues an Xoff command to stop transmitting data until the printer catches up. When the print buffer becomes almost empty (20%) than the printer issues a Xon command. This goes on until the complete document is printed.

- 18d. Software Null Modem

Since we are using software to control the data flow, we can eliminate a few of the control lines used in the Hardware Null Modem cable. In its simplest form, the Null Modem cable consists of SGND, and the TXDs & RXDs crossed.

Usually we find that we have to add a few control lines to fool the DTE's hardware. There is no standard Software Null Modem configuration for Xon/Xoff. The exact connection will vary from device manufacturer to device manufacturer.

- 18e. Terminals & PCs

Terminals are considered dumb devices. They can only display data on the screen and input data from a keyboard. They communicate with a mainframe or minicomputer which does the number crunching and work. Terminals do not have hard-drives for storing files or RAM for running programs. Terminals cannot work by themselves, they are an extension of the mainframe or minicomputer's display and keyboard.

PCs have microprocessors which are the smarts or brains that can do number crunching and work. They have hard-drives for storage and RAM for running programs. They are stand-alone devices.

The purpose of communication programs like Procomm Plus, Kermit, PCLink or Quicklink II is to turn your PC into a terminal. It is the computer world's equivalent of a lobotomy.

- 19. Timing

Timing refers to how the receiving system knows that it received the start of a group of bits and the end of a group of bits. Two major timing schemes are used: Asynchronous and Synchronous Transmission.

- Asynchronous Transmission sends only 1 character at a time. A character being a letter of the alphabet or number or control character. Preceding each character is a Start bit and ending each character is 1 or more Stop bits.

- Synchronous Transmission sends packets of characters at a time. Each packet is preceded by a Start Frame which is used to tell the receiving station that a new packet of characters is arriving and to synchronize the receiving station's internal clock. The packets also have End Frames to indicate the end of the packet. The packet can contain up to 64,000 bits. Both Start and End Frames have a special bit sequence that the receiving station recognizes to indicate the start and end of a packet. The Start and End frames may be only 2 bytes each.

Packet

Conventional representation has asynchronous data flowing left to right and synchronous data flowing right to left.

- 19a. Asynchronous vs. Synchronous Transmission

Asynchronous transmission is simple and inexpensive to implement. It is used mainly with Serial Ports and dialup connections. Requires start and stop bits for each character - this adds a high overhead to transmission. For example: for every byte of data, add 1 Start Bit and 2 Stop Bits. 11 bits are required to send 8 bits! Asynchronous is used in slow transfer rates typically up to 56 kbps.

Synchronous transmission is more efficient as little as only 4 bytes (3 Start Framing bytes and 1 Stop Framing byte) are required to transmit up to 64 kbits. Synchronous transmission is more difficult and expensive to implement. It is used with all higher comunication transfer rates: Ethernet, Token Ring etc... Synchronous is used in fast transfer rates typically 56 kbps to 100 Mbps.

Historically, synchronous communications were operating over 2400/4800 baud modems on point-to-point communications, for example: IBM2770/IBM2780/IBM3780 (historical information courtesy of Jacques Sincennes, University of Ottawa)

Example: Compare a 10K Byte data transmission using Asynchronous transmission & Synchronous Transmission. Determine the efficiency (10 kBytes = 80 kbits).

Asynchronous: Add 3 bits (1 Start and 2 Stop bits) for every byte transmitted.

80 kbits + 30 kbits = total of 110 kbits transmitted

Synchronous: Add 4 bytes (32 bits) for the complete 10K byte data packet.

80 kbits + 32 bits = total of 80.032 kbits transmitted

efficiency = data transmitted x 100 = 80 kbits x 100 = 99.9%

| Transmission | Advantages | Disadvantages |
| --- | --- | --- |
| Asynchronous | Simple & Inexpensive | High Overhead |
| Synchronous | Efficient | Complex and Expensive |

- 20. Asynchronous Communications

Asynchronous communications or transmission sends individual characters one at a time framed by a start bit and 1 or 2 stop bits.

- 20a. Start/Stop bits

The purpose of the Start bit is to notify the receiving station of a new character arriving. Typically data is shown moving left to right. This is how it would appear on a Storage Oscilloscope or Network Analyser. The MSB ( Most Significant Bit) is sent first and the LSB (Least Significant Bit) is sent last.

The purpose of the Stop bits is to indicate the end of data. There could be 1 or 2 stop bits with 1 being the typical number of stop bits used today. In Asynchronous transmission, the characters are sent individually with a quiet period in between (quiet meaning 0 bit level). Asynchronous communications requires the transmitting station and the receiving station to have individual internal free-running clocks operating at the same frequency. Free-running means that the clocks are not locked together.

Both clocks operating at same frequency:

The receive station starts checking for data after the Start bit is received (Start bit is a wake up call!).

The receive station samples the transmitted data in the middle of each data bit. The samples are evenly spaced and match the transmitted data because both transmit and receive clocks are operating at the same frequency.

Receive clock frequency higher than transmitted frequency:

If the receive station's clock is higher in frequency, the samples will be spaced closer together (higher frequency - shorter period). In the above example, we transmitted the following data: 0100 1010 but we received the data: 0100 0101. The samples are out of synchronization with the transmitting data. We would have an error in receiving data.

Clocks are controlled by crystals (abbreviated: Xtal). Crystals are metal cans that hold a piezo-electric element that resonates at a certain frequency when a voltage is applied to it. If you drop a crystal or a printed circuit board (PCB) that has a crystal on it, the crystal can fracture inside the metal can. Either it will stop working or change its frequency, both result in a malfunctioning circuit! Crystals are also temperature sensitive and change frequency with temperature!

Receive clock frequency lower than transmitted frequency:

If the receiving station's clock is lower in frequency than the transmitted frequency, then the samples become farther apart (lower frequency - wider period). Again the samples become out of sync with the transmitted data!

The transmitted data is 0100 1010 but the receive data is 0101 0101! Again we would have receive data errors.

This is a basic problem with asynchronous communications, both transmitter and receiver require a very stable clock to work properly. At high frequencies (which result in high transfer rates), clock stability is critical and asynchronous transmission is very difficult to accomplish. Because of this inherent problem with asynchronous transmission, it is used at low frequency/slow transfer rates.

- 20b. 7/8 Bit Codes

There are 2 common data transfer codes in data communication:

- 7 bit code (Text)

- 8 bit (Binary)

7 Bit Code or Text:

7 bit data code transfer is used to transfer text files. These are files consisting of ASCII text characters only. There are only 27 or 128 different characters in the ASCII text transfer type.

Usually, files that are meant to be read by the human eye used 7 bit code! Text editors like DOS's EDLIN & EDITOR or Unix's pico or vi are used to change or modify the files. Examples of text files: autoexec.bat, config.sys, .signature, E-mail, stories, information.

8 Bit Code or Binary:

8 bit code is used to transfer binary files that contain information that is to be "read" specifically by an application or microprocessor. They contain 8 bit (1 byte) control codes and have 28 or 256 different characters. Examples of binary files are: drawings.bmp (bit mapped graphics), win.com (application), newtext.zip (compressed files).

Common Problems:

If you download a binary (8 bit) file, using text (7 bit) mode, you lose 1 bit from each character. In a binary file this is disastrous! The text transfer mode ignores the 8th bit and discards it into the bit bucket. In the following example the number 202 is transmitted but the number 74 is received. You end up with a corrupted file!

|  | Decimal | Binary |  |
| --- | --- | --- | --- |
| Transmitted | 202 | 1100 1010 | - 8 bit data |
| Received | 74 | 100 1010 | - 7 bit data (MSB is ignored) |

If you download a text file (7 bit) using binary (8 bit) mode, an extra bit is inserted into the data. The bit is set to 0 and placed as the MSB or 8th bit.

|  | Decimal | Binary |  |
| --- | --- | --- | --- |
| Transmitted | 74 | 100 1010 | - 7 bit data |
| Received | 74 | 0100 1010 | - 8 bit data |

The received file works beautifully! If there is a choice or you are not sure what the number of data bits are, always pick Binary or 8 bit transfer mode! Originally, when transfer rates were very slow (300 to 1200 bps), sending 7 or 8 bits would make a big difference in transfer times.

- 20c. Parity Bits

In asynchronous communications, a simple error checking method is used: Parity Checking. There are 3 types of Parity Bits: Even, Odd and None. None means that there is no Parity Checking and the Parity Checking is disabled!

Even Parity Generation

Even Parity counts the number of 1s in the data to see if the total is an even number. If the number of 1s is an even number then the Parity bit is set to 0. If the number of 1s is an odd number, then the Parity bit is set to 1 to make the total number of 1s an even number. The Even Parity Bit is used to make the total number of 1s equal to an even number.

| Data | Even Parity Bit |  |
| --- | --- | --- |
| 0100 1010 | 1 | 3 x 1s in Data: 3 is an odd number, Parity Bit = 1 |
| 0111 1110 | 0 | 6x 1s in Data: 6 is an even number, Parity Bit = 0 |
| 1010 1010 | ? | What should the parity bit be? |

Even Parity Checking

When a data with even parity is received. The number of 1s in both the data and the parity bit are counted. If the number of 1s is an even number than the data is good data, if it is an odd number than the data is corrupted.

| Data | Even Parity Bit |  |
| --- | --- | --- |
| 0100 1010 | 1 | 4 x 1s in data and parity bit = Good data |
| 0111 1110 | 1 | 7 x 1s in data and parity bit = Bad data |
| 1010 1010 | 0 | Is this good or bad data? |

Odd Parity Generation

Odd Parity is the opposite of Even Parity. Odd Parity counts the number of 1s in the data to see if the total is an odd number. If the number of 1s is an odd number then the Parity bit is set to 0. If the number of 1s is an even number, then the Parity bit is set to 1 to make the total number of 1s an odd number. The Odd Parity Bit is used to make the total number of 1s equal to an odd number.

| Data | Odd Parity Bit |  |
| --- | --- | --- |
| 0100 1010 | 1 | 3 x 1s in Data: 3 is an odd number, Parity Bit = 0 |
| 0111 1110 | 0 | 6x 1s in Data: 6 is an even number, Parity Bit = 1 |
| 1010 1011 | ? | What should the parity bit be? |

Odd Parity Checking

When a data with odd parity is received. The number of 1s in both the data and the parity bit are counted. If the number of 1s is an odd number than the data is good data, if it is an even number than the data is corrupted.

| Data | Odd Parity Bit |  |
| --- | --- | --- |
| 0100 1010 | 0 | 3 x 1s in data and parity bit = Good data |
| 0111 1110 | 0 | 6 x 1s in data and parity bit = Bad data |
| 1010 1010 | 0 | Is this good or bad data? |

Parity Agreement

Both receive and transmit stations must agree on the type of parity checking used before transmitting. Usually it is setup in the communications parameters setup. Most common transfer are: 8n1 (8 data bits, no parity, 1 stop bit) or 7e2 (7 data bits, even parity, 2 stop bits).

The parity bit is added in the asynchronous bit stream just before the stop bits and adds to the overhead for asynchronous transmission. A total of 12 bits must be transmitted in order to send 8 bits of data.

Problems with Parity Checking

There is a problem with parity checking. It only works reliably if there is only 1 bit error in the transmitted character stream. If there are 2 bit errors, the parity checking may not detect that there is an error. For example:

|  | Data | Odd Parity Bit |  |
| --- | --- | --- | --- |
| Transmitted | 0100 1010 | 0 | 3 x 1s in data and parity bit = Good data |
| Received | 0110 1110 | 0 | 5 x 1s in data and parity bit = Good data? |

Parity checking would pass the received data as good data even though 2 bits are corrupted!

- 21. Line Encoding

The waveform pattern of voltage or current used to represent the 1s and 0s of a digital signal on a transmission link is called line encoding. The common types of line encoding are Polar, Unipolar, Bipolar and Manchester encoding.

- 21a. Unipolar Encoding

Unipolar encoding has 2 voltage states with one of the states being 0 volts. Since Unipolar line encoding has one of its states being 0 Volts, it is also called Return to Zero (RTZ). A common example of Unipolar line encoding is the TTL logic levels used in computers and digital logic.

Unipolar line encoding works well for inside machines where the signal path is short but is unsuitable for long distances due to the presence of stray capacitance in the transmission medium. On long transmission paths, the constant level shift from 0 volts to 5 volts causes the stray capacitance to charge up (remember the capacitor charging formula 1-e-t/RC !). There will be a "stray" capacitor effect between any two conductors that are in close proximity to each other. Parallel running cables or wires are very suspectible to stray capacitance.

If there is sufficient capacitance on the line and a sufficient stream of 1s, a DC voltage component will be added to the data stream. Instead of returning to 0 volts, it would only return to 2 or 3 volts! The receiving station may not recognize a digital low at voltage of 2 volts!

Unipolar line encoding can have synchronization problems between the transmitter and receiver's clock oscillator. The receiver's clock oscillator locks on to the transmitted signal's level shifts (logic changes from 0 to 1). If there is a long series of logical 1s or 0s in a row. There is no level shift for the receive oscillator to lock to. The receive oscillator's frequency may drift and become unsynchronized. It could lose track of where the receiver is supposed to sample the transmitted data!

Receive oscillator may drift during the period of all 1s

- 21b. Polar Encoding

When the digital encoding is symmetrical around 0 Volts, it is called a Polar Code. The RS-232D interface uses Polar line encoding. The signal does not return to zero, it is either a +ve voltage or a -ve voltage. Polar line encoding is also called None Return To Zero (NRZ). Polar line encoding is the simplest pattern that eliminates most of the residual DC problem.

There is still a small residual DC problem but Polar line encoding is a great improvement over Unipolar line encoding. Polar encoding has an added benefit in that it reduces the power required to transmit the signal by one-half compared with unipolar.

RS-232D TXD

Polar line encoding has the same synchronization problem as Unipolar line encoding. If there is a long string of logical 1s or 0s, the receive oscillator may drift and become unsynchronized.

- 21c. Bipolar Line Encoding

Bipolar line encoding has 3 voltage levels, a low or 0 is represented by a 0 Volt level and a 1 is represented by alternating polarity pulses. By alternating the polarity of the pulses for 1s, the residual DC component cancels.

Bipolar Line Encoding

Synchronization of receive and transmit clocks is greatly improved except if there is a long string of 0s transmitted. Bipolar line encoding is also called Alternate Mark Inversion (AMI).

- 21d. Manchester Line Encoding

In the Manchester Line Encoding, there is a transition at the middle of each bit period. The mid-bit transition serves as a clocking mechanism and also as data: a low to high transition represents a 1 and a high to low transition represents a 0.

Manchester line encoding has no DC component and there is always a transition available for synchronizing receive and transmit clocks. Manchester line encoding is also called a self clocking line encoding. It has the added benefit of requiring the least amount of bandwidth compared to the other line encoding. Manchester line encoding requires 2 frequencies: the base carrier and 2 x the carrier frequency. All others require a range from 0 hertz to the maximum transfer rate frequency.

Manchester line encoding can detect errors during transmission. a transition is expected for during every bit period. The absence of a transition would indicate an error condition.

- 22. Standard Digital Codes

Computers process information in digital form. Characters are assigned a 7 or 8 bit code to indicate which character it is. This 7 or 8 bit code becomes a number (usually hexadecimal) that the computer can work with. The characters stored in a computer include:

| Lower case letters: |  | a - z |
| --- | --- | --- |
| Upper case letters: |  | A - Z |
| Digits: |  | 0 - 9 |
| Punctuation Marks: |  | . , ; : ! ? etc... |
| Unit Symbols: |  | # $ % & * etc... |
| Control Codes: |  | EOF, etc.. |

There are 2 major codes existing today: ASCII (pronounced ah-skee) and EBCDIC (pronounced eb-ce-dic).

- 22a. EBCDIC - Extended Binary Coded Decimal Interchange Code

EBCDIC is used mainly by IBM mainframes and compatibles. It is not common in the PC LAN world unless you are connecting to the IBM mainframe world. In order to connect, you would require either an IBM 3270 terminal emulation program or a device called a gateway.

Table 18-1 shows the EBCDIC translation table. Computers speak in binary code which is 1s and 0s. The computers do not know what the letter "A" is. Instead they speak of the letter "A" as the binary number 1100 0001. It is not easy for humans to remember binary numbers such as 1100 0001 but it is easier to remember the hexadecimal number C1. The hexadecimal number C1 is equal to the binary number 1100 0001.

The hexadecimal number C1 is equal to the decimal number 193. The table 18-1 shows both the decimal (dec) number and the hexadecimal (hex) number for the capital letter "A". Lower case "a" is represented by the EBCDIC decimal code 129 or hexadecimal code 81.

Besides character codes such as the previous letter "A", the EBCDIC code also defines control characters. These are characters that have special meaning. For example, the control character FF stands for Form Feed and is used by printers to advance one page or to eject a page. The decimal code for FF is 12 and the hexadecimal code is C.

Both hexadecimal and decimal codes are indicated because many times, a program or interface will report the EBCDIC code in one or the other formats. You may have to use Table 18-1 to translate from the numerical code to the actual character.

Note: Some EBCDIC codes are not defined and have no name.

| 22a. EBCDIC - Extended Binary Coded Decimal Interchange Code (cont'd) | Next |
| --- | --- |

| Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | NUL |  | 32 | 20 | DS |  | 64 | 40 | RSP |  | 96 | 60 | - |  |
| 1 | 1 | SOH |  | 33 | 21 | SOS |  | 65 | 41 |  |  | 97 | 61 | / |  |
| 2 | 2 | STX |  | 34 | 22 | FS |  | 66 | 42 |  |  | 98 | 62 |  |  |
| 3 | 3 | ETX |  | 35 | 23 | WUS |  | 67 | 43 |  |  | 99 | 63 |  |  |
| 4 | 4 | SEL |  | 36 | 24 | BYP |  | 68 | 44 |  |  | 100 | 64 |  |  |
| 5 | 5 | HT |  | 37 | 25 | LF |  | 69 | 45 |  |  | 101 | 65 |  |  |
| 6 | 6 | RNL |  | 38 | 26 | ETB |  | 70 | 46 |  |  | 102 | 66 |  |  |
| 7 | 7 | DEL |  | 39 | 27 | ESC |  | 71 | 47 |  |  | 103 | 67 |  |  |
| 8 | 8 | GE |  | 40 | 28 | SA |  | 72 | 48 |  |  | 104 | 68 |  |  |
| 9 | 9 | SPS |  | 41 | 29 | SFE |  | 73 | 49 |  |  | 105 | 69 |  |  |
| 10 | A | RPT |  | 42 | 2A | SM |  | 74 | 4A | ў |  | 106 | 6A | ¦ |  |
| 11 | B | VT |  | 43 | 2B | CSP |  | 75 | 4B | . |  | 107 | 6B | , |  |
| 12 | C | FF |  | 44 | 2C | MFA |  | 76 | 4C | < |  | 108 | 6C | % |  |
| 13 | D | CR |  | 45 | 2D | ENQ |  | 77 | 4D | ( |  | 109 | 6D | - |  |
| 14 | E | SO |  | 46 | 2E | ACK |  | 78 | 4E | + |  | 110 | 6E | > |  |
| 15 | F | SI |  | 47 | 2F | BEL |  | 79 | 4F | к |  | 111 | 6F | ? |  |
| 16 | 10 | DLE |  | 48 | 30 |  |  | 80 | 50 | & |  | 112 | 70 |  |  |
| 17 | 11 | DC1 |  | 49 | 31 |  |  | 81 | 51 |  |  | 113 | 71 |  |  |
| 18 | 12 | DC2 |  | 50 | 32 | SYN |  | 82 | 52 |  |  | 114 | 72 |  |  |
| 19 | 13 | DC3 |  | 51 | 33 | IR |  | 83 | 53 |  |  | 115 | 73 |  |  |
| 20 | 14 | RES |  | 52 | 34 | PP |  | 84 | 54 |  |  | 116 | 74 |  |  |
| 21 | 15 | NL |  | 53 | 35 | TRN |  | 85 | 55 |  |  | 117 | 75 |  |  |
| 22 | 16 | BS |  | 54 | 36 | NBS |  | 86 | 56 |  |  | 118 | 76 |  |  |
| 23 | 17 | POC |  | 55 | 37 | EOT |  | 87 | 57 |  |  | 119 | 77 |  |  |
| 24 | 18 | CAN |  | 56 | 38 | SBS |  | 88 | 58 |  |  | 120 | 78 |  |  |
| 25 | 19 | EM |  | 57 | 39 | IT |  | 89 | 59 |  |  | 121 | 79 | ` |  |
| 26 | 1A | UBS |  | 58 | 3A | RFF |  | 90 | 5A | ! |  | 122 | 7A | : |  |
| 27 | 1B | CU1 |  | 59 | 3B | CU3 |  | 91 | 5B | $ |  | 123 | 7B | # |  |
| 28 | 1C | IFS |  | 60 | 3C | NAK |  | 92 | 5C | * |  | 124 | 7C | @ |  |
| 29 | 1D | IGS |  | 61 | 3D |  |  | 93 | 5D | ) |  | 125 | 7D | ' |  |
| 30 | 1E | IRS |  | 62 | 3E | SUB |  | 94 | 5E | ; |  | 126 | 7E | = |  |
| 31 | 1F | IUS |  | 63 | 3F | SP |  | 95 | 5F | щ |  | 127 | 7F | " |  |

Table 18-1 EBCDIC code

| 22a. EBCDIC - Extended Binary Coded Decimal Interchange Code (cont'd) | Next |
| --- | --- |

| Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 80 |  |  | 160 | A0 |  |  | 192 | C0 | { |  | 224 | E0 | \ |
| 129 | 81 | a |  | 161 | A1 | ~ |  | 193 | C1 | A |  | 225 | E1 | NSP |
| 130 | 82 | b |  | 162 | A2 | s |  | 194 | C2 | B |  | 226 | E2 | S |
| 131 | 83 | c |  | 163 | A3 | t |  | 195 | C3 | C |  | 227 | E3 | T |
| 132 | 84 | d |  | 164 | A4 | u |  | 196 | C4 | D |  | 228 | E4 | U |
| 133 | 85 | e |  | 165 | A5 | v |  | 197 | C5 | E |  | 229 | E5 | V |
| 134 | 86 | f |  | 166 | A6 | w |  | 198 | C6 | F |  | 230 | E6 | W |
| 135 | 87 | g |  | 167 | A7 | x |  | 199 | C7 | G |  | 231 | E7 | X |
| 136 | 88 | h |  | 168 | A8 | y |  | 200 | C8 | H |  | 232 | E8 | Y |
| 137 | 89 | i |  | 169 | A9 | z |  | 201 | C9 | I |  | 233 | E9 | Z |
| 138 | 8A |  |  | 170 | AA |  |  | 202 | CA | SHY |  | 234 | EA |  |
| 139 | 8B |  |  | 171 | AB |  |  | 203 | CB |  |  | 235 | EB |  |
| 140 | 8C |  |  | 172 | AC |  |  | 204 | CC |  |  | 236 | EC |  |
| 141 | 8D |  |  | 173 | AD |  |  | 205 | CD |  |  | 237 | ED |  |
| 142 | 8E |  |  | 174 | AE |  |  | 206 | CE |  |  | 238 | EE |  |
| 143 | 8F |  |  | 175 | AF |  |  | 207 | CF |  |  | 239 | EF |  |
| 144 | 90 |  |  | 176 | B0 |  |  | 208 | D0 | } |  | 240 | F0 | 0 |
| 145 | 91 | j |  | 177 | B1 |  |  | 209 | D1 | J |  | 241 | F1 | 1 |
| 146 | 92 | k |  | 178 | B2 |  |  | 210 | D2 | K |  | 242 | F2 | 2 |
| 147 | 93 | l |  | 179 | B3 |  |  | 211 | D3 | L |  | 243 | F3 | 3 |
| 148 | 94 | m |  | 180 | B4 |  |  | 212 | D4 | M |  | 244 | F4 | 4 |
| 149 | 95 | n |  | 181 | B5 |  |  | 213 | D5 | N |  | 245 | F5 | 5 |
| 150 | 96 | o |  | 182 | B6 |  |  | 214 | D6 | O |  | 246 | F6 | 6 |
| 151 | 97 | p |  | 183 | B7 |  |  | 215 | D7 | P |  | 247 | F7 | 7 |
| 152 | 98 | q |  | 184 | B8 |  |  | 216 | D8 | Q |  | 248 | F8 | 8 |
| 153 | 99 | r |  | 185 | B9 |  |  | 217 | D9 | R |  | 249 | F9 | 9 |
| 154 | 9A |  |  | 186 | BA |  |  | 218 | DA |  |  | 250 | FA |  |
| 155 | 9B |  |  | 187 | BB |  |  | 219 | DB |  |  | 251 | FB |  |
| 156 | 9C |  |  | 188 | BC |  |  | 220 | DC |  |  | 252 | FC |  |
| 157 | 9D |  |  | 189 | BD |  |  | 221 | DD |  |  | 253 | FD |  |
| 158 | 9E |  |  | 190 | BE |  |  | 222 | DE |  |  | 254 | FE |  |
| 159 | 9F |  |  | 191 | BF |  |  | 223 | DF |  |  | 255 | FF | EO |

Table 18-1 EBCDIC code (cont'd)

- 22b. ASCII - American Standard Code for Information Interchange

ASCII is the most popular code and is used by the majority of the computing world. ASCII itself is a 7 bit code which allows only 128 characters (27). Most applications follow IBM's Extended ASCII code which uses 8 bits and allows an addition 128 graphic characters for a total of 256 characters (28). We will be concentrating on 7 bit ASCII codes.

Format effectors

Format effectors control the movement of the cursor on the screen and the print head in a printer. The format effectors are:

| BS | Backspace |
| --- | --- |
| HT | Horizontal Tab |
| LF | Line Feed |
| CR | Carriage Return |
| FF | Form Feed |
| VT | Vertical Tab |

Communication Controls

Communication Controls are used in controlling data transmission over a communication network. They are used in both Asynchronous and Synchronous Transmissions. They are used in "handshaking".

| STX | Start of Text |
| --- | --- |
| ETX | End of Text |
| EOT | End of Transmission |
| ENQ | End of Inquiry |
| ACK | Acknowledge |
| NAK | Negative Acknowledge |
| EXT | Interrupt |
| SYN | Synchronous idle |
| ETB | End of Block |
| EOF | End of File |

Information Separators

Information separators are used to separate database enquiries and files:

| FS | File Separator (in a PC - used as cursor R, L, U, D) |
| --- | --- |
| GS | Group Separator |
| RS | Record Separator |
| US | Unit Separator |

Additional Control Codes

Of the remaining codes used by the computer, the most important ones are:

| NUL | Nothing character |
| --- | --- |
| BEL | Rings the bell! |
| DC1 - 4 | Device Control 1 - 4 |
| ESC | Escape - used for formatting printers & terminals |
| DEL | Delete - deletes characters under cursor |

DC1 & DC2 are used in the Xon/Xoff software handshaking to control data transfer.

Displaying ASCII codes directly to the screen

You can type in the ASCII codes directly to the screen on IBM capatible computers. You press the "ALT" key and a 3 digit number on the numeric keypad. The 3 digit number is the ASCII decimal code for the character. You must use the numeric keypad, the QWERTY numbers will NOT work.

For example, the character "A" corresponds to the ASCII decimal code 65. To access the ASCII code directly, hold down the ALT key and type in 065 on the numeric keypad. On releasing the ALT key, the letter A will appear on the screen.

Table 18-2 shows the ASCII codes according to decimal numbers and hexadecimal numbers. If a network sniffer or analyzer is used, it will show raw data in decimal or hexadecimal formats. You may have to perform a manual translation using Table 18-2.

| 22b. ASCII - American Standard Code for Information Interchange (cont'd) | Next |
| --- | --- |

| Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |  | Dec | Hex | Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | NUL |  | 32 | 20 | Space |  | 64 | 40 | @ |  | 96 | 60 | ` |
| 1 | 1 | SOH |  | 33 | 21 | ! |  | 65 | 41 | A |  | 97 | 61 | a |
| 2 | 2 | STX |  | 34 | 22 | " |  | 66 | 42 | B |  | 98 | 62 | b |
| 3 | 3 | ETX |  | 35 | 23 | # |  | 67 | 43 | C |  | 99 | 63 | c |
| 4 | 4 | EOT |  | 36 | 24 | $ |  | 68 | 44 | D |  | 100 | 64 | d |
| 5 | 5 | ENQ |  | 37 | 25 | % |  | 69 | 45 | E |  | 101 | 65 | e |
| 6 | 6 | ACK |  | 38 | 26 | & |  | 70 | 46 | F |  | 102 | 66 | f |
| 7 | 7 | BEL |  | 39 | 27 | ў |  | 71 | 47 | G |  | 103 | 67 | g |
| 8 | 8 | BS |  | 40 | 28 | ( |  | 72 | 48 | H |  | 104 | 68 | h |
| 9 | 9 | HT |  | 41 | 29 | ) |  | 73 | 49 | I |  | 105 | 69 | i |
| 10 | A | LF |  | 42 | 2A | * |  | 74 | 4A | J |  | 106 | 6A | j |
| 11 | B | VT |  | 43 | 2B | + |  | 75 | 4B | K |  | 107 | 6B | k |
| 12 | C | FF |  | 44 | 2C | , |  | 76 | 4C | L |  | 108 | 6C | l |
| 13 | D | CR |  | 45 | 2D | - |  | 77 | 4D | M |  | 109 | 6D | m |
| 14 | E | S0 |  | 46 | 2E | . |  | 78 | 4E | N |  | 110 | 6E | n |
| 15 | F | S1 |  | 47 | 2F | / |  | 79 | 4F | O |  | 111 | 6F | o |
| 16 | 10 | DLE |  | 48 | 30 | 0 |  | 80 | 50 | P |  | 112 | 70 | p |
| 1 | 11 | DC1 |  | 49 | 31 | 1 |  | 81 | 51 | Q |  | 113 | 71 | q |
| 18 | 12 | DC2 |  | 50 | 32 | 2 |  | 82 | 52 | R |  | 114 | 72 | r |
| 19 | 13 | DC3 |  | 51 | 33 | 3 |  | 83 | 53 | S |  | 115 | 73 | s |
| 20 | 14 | DC4 |  | 52 | 34 | 4 |  | 84 | 54 | T |  | 116 | 74 | t |
| 21 | 15 | NAK |  | 53 | 35 | 5 |  | 85 | 55 | U |  | 117 | 75 | u |
| 22 | 16 | SYN |  | 54 | 36 | 6 |  | 86 | 56 | V |  | 118 | 76 | v |
| 23 | 17 | ETB |  | 55 | 37 | 7 |  | 87 | 57 | W |  | 119 | 77 | w |
| 24 | 18 | CAN |  | 56 | 38 | 8 |  | 88 | 58 | X |  | 120 | 78 | x |
| 25 | 19 | EM |  | 57 | 39 | 9 |  | 89 | 59 | Y |  | 121 | 79 | y |
| 26 | 1A | SUB |  | 58 | 3A | : |  | 90 | 5A | Z |  | 122 | 7A | z |
| 27 | 1B | ESC |  | 59 | 3B | ; |  | 91 | 5B | [ |  | 123 | 7B | { |
| 28 | 1C | FS |  | 60 | 3C | < |  | 92 | 5C | \ |  | 124 | 7C | ¦ |
| 29 | 1D | GS |  | 61 | 3D | = |  | 93 | 5D | ] |  | 125 | 7D | } |
| 30 | 1E | RS |  | 62 | 3E | > |  | 94 | 5E | ^ |  | 126 | 7E | ~ |
| 31 | 1F | US |  | 63 | 3F | ? |  | 95 | 5F | _ |  | 127 | 7F | DEL |

Table 18-2 ASCII code

- 23. Voice Channel Communications

The voice channel or dial-up line is the line from our telephone/modem to the outside world.

As the name implies "voice" channel is designed to carry human speech over the telephone wires.

- 23a. Voice Channel Specification

Human speech covers the frequency range of 100 to 7000 Hz (hertz) but research has shown that the intelligence part of human speech is carried in the 300 - 3400 Hz range. This range is called the Voice Band.

The Voice Channel has a range of 0 to 4 kHz (4000 Hz). The area from 3400 to 4000 Hz is used for system control and is called Out of Band Signalling.

- 23b. Voice Channel Constraints

Due to the limited Bandwidth (BW) of the Voice Channel (0-4 kHz), we are limited to the amount of data that we can pass through the Voice Channel. The Nyquist Theorem addresses this limitation.

- 23c. Nyquist Theorem

In a digital Public phone system, the signal leaving our telephone at our house is an analog signal. It goes to the Central Office through the Local Loop. The Local Loop is the name for the wires that run from our house to the Central Office. The Central Office (also called a local exchange) is the building that all the neighbourhood phones with the same local connect. A local is the 1st 3 digits of your 7 digit phone number or LDN (Listed Directory Number).

At the Central Office, the analog signal is converted into a digital signal consisting of 1s and 0s.

The Nyquist Theorem states that to accurately reproduce an analog signal with a digital signal, the analog signal must be sampled a minimum of 2x the highest frequency of the analog signal.

This means that for the Voice Channel (0 to 4 kHz) to be digitized, we must sample the Voice Channel at 2x the highest frequency (4 kHz) which would be 8 kHz. This means that as soon as you digitize an analog signal, you must immediately double the bandwidth

- 24. Telephone Networks

The telephone network consists of your phone at home, that is connected by the Local Loop to the Central Office which is connected to a Hierarchical Phone Network. Worldwide there are over 300 million (300,000,000) telephones - 98% of them interconnected.

- 24a. POTS - Plain Old Telephone Set

The POTS or Plain Old Telephone Set consists of 5 sections:

- Ringer Unit

- Hook Switch

- Dialer Unit

- Hybrid/Speech Network

- Hand Set

The connection to the CO (Central Office) is with only 2 wires: Tip and Ring. This connection is called the Local Loop.

The Tip is +ve and coloured green.. The Ring is -ve and coloured Red. If you look at a phone jack in your house, you will see that it is wired for 4 wires: Red, Green, Black and Yellow. Black and Yellow are not normally used.

The black and yellow wires can be used for a second telephone line or they can be used for running a Network Physical layer protocol called Phonenet by Farralon. Phonenet uses the Black and Yellow for Network communications. It is for use with Appletalk and is a replacement for Localtalk. It runs at the Localtalk speed of 230 Kbps which is reasonable for small networks.

i. Ringer Unit

The ringer is a device to alert you to an incoming call. It interprets the ringing voltage from the Central Office. Originally, the ringer was a electromagnetic bell but today, most ringers are electronic devices.

The Central Office sends:

- a 90 to 120 VAC ringing voltage

- Frequency of 20 Hz

- Cadence for North America is 2 sec On/ 4 sec Off

ii. Hook Switch

The hook switch is a switch that is activated by lifting the handset off the cradle. The position of the hook switch determines whether the telephone is waiting for a call or actively using the line. The Off-hook position informs the network of a request for use. The On-hook position releases the use of the network.

iii. Dialer Unit

There are two types of Dialer Units: Rotary Dial and Touch Tone. Rotary Dial are the old put your finger in the hole and spin type. The rotary dial operates by toggling the Hook Switch on and off.

Touch Tone is the modern method where 2 frequencies per push button are sent. Touch Tone is a trade name, the correct name is DTMF (Dual Tone Multi Frequency).

iv. Hybrid/Speech Network

The Hybrid/Speech Network performs several functions:

- It converts the Tx/Rx 4 wires from the Handset to the 2 wires for the Local Loop.

- It interfaces the signals from the Dialer Unit to the telephone line.

- It provides auto line compensation for line length to keep the volume constant.

v. Handset

The Handset contains transducers for converting mechanical energy into electrical energy. The microphone converts speech into electrical energy. The diaphragm or speaker converts electrical signals into audible signals.

Functions of a Telephone Set:

- Request use of network from the CO (Central Office).

- Inform you of the network status: Dial-tone, Ringing, Busy, Fast Busy (Talk Mail)

- Informs CO of desired number.

- Informs you when a call is incoming (phone rings).

- Releases use of network when call is complete (hang-up)

- Transmit speech on network & receives speech from distant caller.

- Adjust power levels and compensates for line length.

- 24b. Local Loops

The Local Loop is the connection between the Central Office and the home or business. To every home is run 2 wires (1 pair). The pair does not go directly to the Central Office, instead it goes to those big green boxes called "Serving Area Interfaces" (SIA) that you see on the street corners. Then large multi-conductor bundles of wires go from there to the Central Office.

- 24c. Central Office

The Central Office provides the following functions:

- It supplies the battery voltage for the telephone system. The On-hook voltage is 48 Vdc +/- 2V. Off-hook voltage is -6.5 Vdc.

- It supplies the Ringing Generator - 90 to 120 VAC, 20 Hz, 2 sec on/ 4 sec off

- It supplies the Busy signal (480 + 620 Hz, 0.5 sec On/ 0.5 sec Off), Dial Tone (350 + 440 Hz) and Fast Busy (480 + 620 Hz, 0.2 sec On/ 0.3 sec Off).

- It has the digital switching gear that determines if the number is an Interoffice call (local) or an Intraoffice call (Toll - long distance).

- 24d. Hierarchical Phone Networks

The PSTN (Public Switch Telephone Network) is divided into a hierarchical network. There are 5 classes of switching centres in North America:

| Class | Centre | Abbreviation | Symbol | Examples |
| --- | --- | --- | --- | --- |
| 1 | Regional Center | RC |  | 2 in Canada: West - Regina East - Montreal |
| 2 | Sectional Center | SC |  | Calgary serves Alberta |
| 3 | Primary Center | PC |  | Edmonton |
| 4 | Toll Center | TC |  | Drumheller |
| 4b | Toll Point | TP |  | Rainbow Lake |
| 5 | Central Office (Local Loop) | CO |  | 284-xxxx |

In the following example:

The Hierarchical portion is seen as:

| Trunk |  | Long distance telephone cable |
| --- | --- | --- |
| Toll Trunk |  | Connects CO (Central Office) to TC (Toll Center) |
| Intertoll Trunk |  | Everything above TC (Toll Center) and TC to TC |
| Interoffice Trunk |  | Between CO (Central Office) |
| Intraoffice Trunk |  | Call between 2 subscribers within the same CO (284-7079 to 284-8181). |

Call routing:

- Preferred route

- Second choice

- Third Choice

Call routing is determined by network engineering and physical location. When all lines are idle, the call routing selects the preferred route. If the preferred route is busy, then the call is routed to the second choice. Because the second choice is routed through one toll center, the charge for the call is greater than the preferred route. The third choice is used when the second choice is busy. The third choice goes through 2 toll centers and is the most expensive route.

A Central Office can have up to 10,000 subscribers: for example 284-0000 to 284-9999. Most have 4,000 to 5,000 subscribers. The Central Office bases the loading requirements on roughly 10% of the phones will be in use at any one time. The use of Internet dialup access has drastically changed this!

- 25. Telephone Line Characteristics

Telephone lines are not perfect devices due to their analog nature. The quality of the telephone line determines the rate that modulated data can be transferred. Good noise free lines allow faster transfer rates such as 14.4 kbps, poor quality lines require the data transfer rate to be stepped down to 9600 bps or less. Phone lines have several measurable characteristics that determine the quality of the line:

- Attenuation Distortion

- Propagation Delay

- Envelope Delay Distortion

- 25a. Attenuation Distortion

Attenuation Distortion is the change in amplitude of the transmitted signal over the Voice Band. It is the frequency response curve of the Voice Band.

Attenuation versus Frequency

To measure Attenuation Distortion, the phone line has a test frequency transmitted from 0 - 4 kHz into the line at a standard amplitude of 0 db. The loss of signal or attenuation is measured at the receiving end and compared to a standard reference frequency: 1004 Hz.

db is short for decibel which is a relative unit of measure (similar to a unit like a dozen). It is a log unit and a +3 db gain will indicate an amplitude of 2x the reference. It is a logarithmic ratio between input voltage and output voltage. It is calculated by the following formula:

db =10 x log (Vout/Vin)

The resulting information is graphed on an Attenuation vs. Frequency chart. Attenuation is a loss of signal amplitude - the receive signal is a smaller amplitude than the transmitted signal. It is indicated by a positive db. It is also possible to have a signal appear at the receiving end with a larger amplitude than when it started - this is indicated by negative db.

| 25a. Attenuation Distortion (cont'd) | Next |
| --- | --- |

The attenuation is due to the many pieces of electronic equipment and transmission media that the signal has to pass through, some can amplify the signal (make it a larger amplitude) and some may attenuate the signal (make it smaller).

There are maximum and minimum acceptable limits for Attenuation Distortion for phone lines. The Basic channel conditioning is:

| Frequency Range |  | Loss (db) |
| --- | --- | --- |
| 500 - 2500 |  | -2 to +8 |
| 300 - 3000 |  | -3 to +12 |

The above Loss is a range of acceptable values for the frequency range. In the Basic Channelling Conditioning, it is acceptable to have a loss in signal in the frequency range of 500-2500 Hz of "8 db loss to -2 db loss" referenced to the amplitude at 1 kHz. Note that on the graph on the previous page that this is shown as -8db and +2 db.

+3 db attenuation is equal to -3 db in signal amplitude and +8 db attenuation equates to -8 db in signal amplitude.

- 25b. Propagation Delay

Signals transmitted down a phone line will take a finite time to reach the end of the line. The delay from the time the signal was transmitted to the time it was received is called Propagation Delay. If the propagation delay was the exact same across the frequency range, there would be no problem. This would imply that all frequencies from 300 to 3000 Hz have the same amount of delay in reaching their destination over the phone line. They would arrive at the destination at the same time but delayed by a small amount called the propagation delay.

This is heard as the delay when talking on long distance telephones. We have to wait a little longer before we speak to ensure that the other person hasn't already started to talk. All phone lines have propagation delay.

If the Propagation Delay is long enough, the modem or communications package may time-out and close the connection. It may think that the receive end has shut off!

- 25c. Envelope Delay Distortion

If the Propagation Delay changes with frequency than we would have the condition where the lower frequencies such as 300 Hz may arrive earlier or later than the higher frequencies such as 3000 Hz. For voice communication, this would probably not be noticable but for data communication using modems, this could affect the phase of the carrier or the modulation technique used to encode the data.

When the Propagation Delay varies across the frequency range, we call this Envelope Delay Distortion. We measure propagation delay in microseconds (us) and the reference is from the worst case to the best case.

- 26. Line Impairments

Line Impairments are faults in the line due to improper line terminations or equipment out of specifications. These cannot be conditioned out but can be measured to determine the amount of the impairment.

- 26a. Crosstalk

Crosstalk is when one line induces a signal into another line. In voice communications, we often hear this as another conversation going on in the background. In digital communication, this can cause severe disruption of the data transfer. Cross talk can be caused by overlapping of bands in a multiplexed system or by poor shielding of cables running close to one another. There are no specific communications standards applied to the measurement of crosstalk.

- 26b. Echo or Signal Return

All media have a preferred termination condition for perfect transfer of signal power. The signal arriving at the end of a transmission line should be fully absorbed otherwise it will be reflected back down the line to the sender and appear as an Echo. Echo Suppressors are often fitted to transmission lines to reduce this effect.

Normally during data transmission, these suppressors must be disabled or they will prevent return communication in full duplex mode. Echo suppressors are disabled on the phone line if they hear carrier for 400ms or more. If the carrier is absent for 100 mSec, the echo suppressor is re-enabled.

Echo Cancellers are currently used in Modems to replicate the echo path response and then combine the results to eliminate the echo. Thus no signal interruption is necessary.

- 26c. Frequency Shift

Frequency shift is the difference between the transmitted frequency and the received frequency. This is caused by the lack of synchronization of the carrier oscillators.

- 26d. Non-Linear Distortion

Non-linear distortion is distortion that changes the waveshape of the signal. If the signal was transmitted as a sinewave and arrived as a squarewave, this would be an example of severe non-linear distortion. Amplitude modulated carriers would suffer drastically if the original wave shape was distorted.

- 26e. Jitter: Amplitude and Phase

There are 2 types of Jitter:

- Amplitude Jitter

- Phase Jitter

Amplitude Jitter is the small constantly changing swings in the amplitude of a signal. It is principally caused by power supply noise (60 Hz) and ringing tone (20 Hz) on the signal.

Phase Jitter is the small constantly changing swings in the phase of a signal. It may result in the pulses moving into time slots allocated other data pulses when used with Time Domain Multiplexing.

Telephone company standards call for no more than 10 degrees between 20 and 300 Hz and no more than 15 degrees between 4 and 20 Hz.

- 26f. Transients: Impulse Noise, Gain Hits, Dropouts & Phase Hits

Transients are irregular timed impairments. They appear randomly and are very difficult to troubleshoot. There are 4 basic types of Transients:

- Impulse Noise

- Gain Hits

- Dropouts

- Phase Hits

i. Impulse Noise

Impulse noise is sharp quick spikes on the signal caused from electromagnetic interference, lightning, sudden power switching, electromechanical switching, etc.. These appear on the telephone line as clicks and pops which are not a problem for voice communication but can appear as a loss of data or even as wrong data bits during data transfers. Impulse noise has a duration of less than 1 mSec and their effect is dissipated within 4 mSec.

ii. Gain Hits

Gain Hits are sudden increase in amplitude that last more than 4 mSec. Telephone company standards allow for no more than 8 gain hits in any 15 minute interval. A gain hit would be heard on a voice conversation as if the volume were turned up for just an instance. Amplitude modulated carriers are particularly sensitive to Gain Hits.

| 26f. Transients: Impulse Noise, Gain Hits, Dropouts & Phase Hits | Next |
| --- | --- |

iii. Dropouts

Dropouts are sudden loss of signal amplitude greater than 12 db that last longer than 4 mSec. They cause more errors than any other type of transients. Telephone company standards allow no more than 1 dropout for every 30 minute interval. Dropouts would be heard on a voice conversation similar to call waiting, where the line goes dead for a 1/2 second. This is a sufficient loss of signal for some digital transfer protocols such as SLIP, that the connection is lost and would have to be re-established.

iv. Phase Hits

Phase Hits are sudden large changes in the received signal phase (20 degrees) or frequency lasting longer than 4 mSec. Phase Hits generally occur when switching between Telcos, common carriers or transmitters. FSK and PSK are particularly sensitive to Phase Hits. The data may be incorrect until the out of phase condition is rectified. The telephone company standard allows no more than 8 phase hits in any 15 minutes.

- 27. Modulation Techniques

Modulation techniques are methods used to encode digital information in an analog world. The 3 basic modulation techniques are:

- AM (amplitude modulation)

- FM (frequency modulation)

- PM (phase modulation)

All 3 modulation techniques employ a carrier signal. A carrier signal is a single frequency that is used to carry the intelligence (data). For digital, the intelligence is either a 1 or 0. When we modulate the carrier , we are changing its characteristics to correspond to either a 1 or 0.

- 27a. AM - Amplitude Modulation

Amplitude Modulation modifies the amplitude of the carrier to represent 1s or 0s. In the above example, a 1 is represented by the presence of the carrier for a predefined period of 3 cycles of carrier. Absence or no carrier indicates a 0.

Advantages:

- Simple to design.

Disadvantages:

- Noise spikes on transmission medium interfere with the carrier signal.

- Loss of connection is read as 0s.

- 27b. FM - Frequency Modulation

Frequency Modulation modifies the frequency of the carrier to represent the 1s or 0s. In the above example, a 0 is represented by the original carrier frequency and a 1 by a much higher frequency ( the cycles are spaced closer together).

Advantages:

- Immunity to noise on transmission medium.

- Always a signal present. Loss of signal easily detected

Disadvantages:

- Requires 2 frequencies

- Detection circuit needs to recognize both frequencies when signal is lost.

- 27c. PM - Phase Modulation

Phase Modulation modifies the phase of the carrier to represent a 1 or 0.

The carrier phase is switched at every occurrence of a 1 bit but remains unaffected for a 0 bit. The phase of the signal is measured relative to the phase of the preceding bit. The bits are timed to coincide with a specific number of carrier cycles (3 in this example = 1 bit).

Advantage:

- Only 1 frequency used

- Easy to detect loss of carrier

Disadvantages:

- Complex circuitry required to generate and detect phase changes.

- 28. Modem Modulation

There are 3 basic types of modulation used in modems:

- FSK - Frequency Shifted Keying

- QPSK - Quadrature Phase Shifted Keying

- QAM - Quadrature Amplitude Modulation

Modern modems use a combination of the above basic modulation techniques and compression to achieve the high data transfer rates (14.4 Kbps and up).

- 28a. FSK - Frequency Shift Keying

Frequency Shift Keying or FSK is the frequency modulation of a carrier to represent digital intelligence. For Simplex or Half Duplex operation, a single carrier (1170 Hz) is used - communication can only be transmitted in one direction at a time. A Mark or 1 is represented by 1270 Hz, and a Space or 0 is represented by 1070 Hz. The following diagram shows the Voice Channel with Simplex/Half Duplex FSK.

Simplex/Half Duplex FSK

Full Duplex FSK

For Full Duplex, (data communication in both directions simultaneously) the upper bandwidth of the Voice Channel is utilized. Another carrier is added at 2125 Hz. A Mark or 1 is represented by 2225 Hz, and a Space or 0 is represented by 2025 Hz. The originating modem (the one which dials the phone number and starts the connection) uses the lower carrier (1170 Hz) and the answer modem (the one which answers the ringing phone line) uses the upper carrier (2125 Hz). This allocation of carriers is done automatically by the modem's hardware. The following diagram shows the Voice Channel with Full Duplex FSK.

Example of Originate's Frequency Modulated Carrier:

The originate modem transmits on the 1170 Hz carrier and receives on the 2125 Hz carrier. The answer modem receives on the 1170 Hz carrier and transmits on the 2125 Hz carrier. This way both modems can be transmitting and receiving simultaneously!

The FSK modem described above is used for 300 baud modems only. The logical question is "Why not use it for higher modems?". Higher data rates require more bandwidth: this would require that the Mark and Space frequencies for each band be moved farther apart (the originate and answer bands become wider). The two carriers would have to move farther apart from each other to prevent crosstalk (interference with each other). The limit for present phone lines is 1200 Baud Half Duplex (one way) used by Bell 202 compatible modems.

- 28b. QPSK - Quadrature Phase Shift Keying

Quadrature Phase Shift Keying employs shifting the phase of the carrier at a 600 baud rate plus an encoding technique. QPSK is used in Bell 212A compatible modems and V.22 - both are 1200 bps Full Duplex standards. The originate modem transmits at 1200 Hz and receives on 2400 Hz. The answer modem receives on 1200 Hz and transmits on 2400 Hz.

The digital information is encoded using 4 (Quad) level differential PSK at 600 baud.

Remember that baud indicates how fast the analog signal is changing in the Voice Channel. The data is encoded as follows:

| DIBIT | Phase Shift |
| --- | --- |
| 00 | +90 |
| 01 | 0 |
| 10 | 180 |
| 11 | 270 |

For every change in the baud rate (phase shift), we can decode 2 bits! This leads to:

2 bits x 600 baud = 1200 bps

Example of Carrier Phase Modulation:

- 28c. QAM - Quadrature Amplitude Modulation

Quadrature Amplitude Modulation refers to QPSK with Amplitude Modulation. Basically, it is a mix of phase modulation and amplitude modulation. QAM phase modulates the carrier and also modulates the amplitude of the carrier.

Phase Modulated and Amplitude Modulated Carrier:

There are two types: 8-QAM and 16-QAM. 8-QAM encodes 3 bits of data (23=8) for every baud and 16-QAM encodes 4 bits of data (24=16) for every baud. Both are used in the V.32 standard for 9600 bps modem (milestone for communications!). 8-QAM transfers 4800 bps and 16-QAM transfers 9600 bps. The baud rate used with QAM is 2400 baud half-duplex.

16-QAM has 12 phase angles, 4 of which have 2 amplitude values! 16-QAM changes phase with every baud change.

16-QAM Phasor Diagram

Higher transfer rates use much more complex QAM methods. For example, V.32bis (14.4 kbps) uses a 64 point constellation to transfer 6 bits per baud. Compare that to the above 16 point constellation!

- AT Command Set

Hayes modems were the first smart modems. They had built-in CPUs that could interpret a special series of commands. These commands are called the AT command set. The basic command for getting a modem's attention was the characters "AT" (older modems may only recognize lower case "at"). Once the modem's attention was available, character's are added immediately after that specify instructions.

Smart modems operate in two modes: command and communication mode. In command mode, the modem is waiting for AT command instructions. In communication mode, the modem is transferring data from sender to receiver.

To talk to a modem, you must use either a terminal or a terminal emulation software on a PC such as Procomm or Hyperterminal. A basic test to see if the modem is communicating properly with the terminal, is to type "AT". If the modem responds with "OK", then the software's configuration matches the modems configuration.

The following configuration issues must match before proper modem to terminal communication will work:

| Configuration Point |  | Typical value |
| --- | --- | --- |
| Com port of modem: |  | Com2 for external, Com4 for internal |
| IRQ of modem: |  | IRQ3 |
| Number of data bits: |  | 8 |
| Type of parity: |  | n (none) |
| Number of stop bits: |  | 1 |
| Transfer speed: |  | 56 kbps (depends on modem) |
| Terminal emulation: |  | vt100 |

If the modem is on-line (communicaton mode), to enter command mode, type "+++" (3 pluses in a row) and wait. The modem should respond with "OK". This indicates that you have entered command mode. You then may enter AT command strings to the modem.

- 29a. Basic AT commands

Modern modems require an initialization string for configuring themselves. The most common configuration string is "ATZ", which is the reset command. Usually and this will depend on the modem, there are factory stored configurations that can be accessed by using the "ATF1" command. If there are more than one available configuration, the others can be accessed by "ATF2" and so on.

Older modems, typically 14.4kbps and earlier, had elaborate initialization strings that differed for each modem and each manufacturer. It was quite a headache to support so many different types of modems and make them work with each other.

To dial out, the "ATD" command is used. "ATDT" uses tone dialing versus rotary dialing. Immediately after the "ATDT" command, the destination telephone number is entered, for example: "ATDT555-1234". Would command the modem to use tone dialing to dial the number 555-1234. To hang up a modem, the AT command string "ATH" can be used.

A partial listing of the AT command set is available in Appendix J. The AT command set is incredibly large and is constantly growing due to the improvements and innovations by the modem manufacturers. There are two main manufacturer's of modem chipsets: Rockwell and US Robotics. Both have excellent documentation on identifying and configuring the modem chipsets that they manufacture.

- Multiplexing

Multiplexing is the transmission of multiple data communication sessions over a common wire or medium. Multiplexing reduces the number of wires or cable required to connect multiple sessions. A session is considered to be data communication between two devices: computer to computer, terminal to computer, etc..

Individual lines running from 3 terminals to one mainframe is not a problem but when the number of terminals increases to 10 and up, it becomes a problem. Imagine a mainframe computer with 1200 terminals connected and each terminal running its own wire to the mainframe. If each wire was 1/4" in diameter (typical Cat 5 cable), you would have a wiring bundle going into the computer, roughly 2 feet in diameter.

A multiplexer allows sharing of a common line to transmit the many terminal communications as in the above example. The connection between the multiplexer and the mainframe is normally a high speed data link and is not usually divided into separate lines.

The operation of multiplexers (abbreviated MUXs) is transparent to the sending and receiving computers or terminals. Transparent means that as far as everyone is concerned, they appear to be directly connected to the mainframe with individual wires. The multiplexer does not interfere with the normal flow of data and it can allow a significant reduction in the overall cost of connecting to remote sites, through the reduced cost of cable and telephone line charges.

Multiplexers are used to connect terminals located throughout a building to a central mainframe. They are also used to connect terminals located at remote locations to a central mainframe through the phone lines.

There are 3 basic techniques used for multiplexing:

- Frequency Division Multiplexing (FDM)

- Time Division Multiplexing (TDM)

- Statistical Time Division Multiplexing (STDM)

- 30a. FDM - Frequency Division Multiplexing

Frequency Division Multiplexing (FDM) is an analog technique where each communications channel is assigned a carrier frequency. To separate the channels, a guard-band would be used. This is to ensure that the channels do not interfere with each other.

For example, if we had our 3 terminals each requiring a bandwidth of 3 kHz and a 300 Hz guard-band, Terminal 1 would be assigned the lowest frequency channel 0 - 3 kHz, Terminal 2 would be assigned the next frequency channel 3.3 kHz - 6.3 kHz and Terminal 3 would be assigned the final frequency channel 6.6 kHz - 9.6 kHz.

The frequencies are stacked on top of each other and many frequencies can be sent at once. The downside is that the overall line bandwidth increases. Individual terminal requirement were 3 kHz bandwidth each, in the above example: the bandwidth to transmit all 3 terminals is now 9.6 kHz.

FDM does not require all channels to terminate at a single location. Channels can be extracted using a multi-drop technique, terminals can be stationed at different locations within a building or a city.

FDM is an analog and slightly historical multiplexing technique. It is prone to noise problems and has been overtaken by Time Division Multiplexing which is better suited for digital data.

- 30b. TDM - Time Division Multiplexing

Time Division Multiplexing is a technique where a short time sample of each channel is inserted into the multiplexed data stream. Each channel is sampled in turn and then the sequence is repeated. The sample period has to be fast enough to sample each channel according to the Nyquist Theory (2x highest frequency) and to be able to sample all the other channels within that same time period. It can be thought of as a very fast mechanical switch, selecting each channel for a very short time then going on to the next channel.

Each channel has a time slice assigned to it whether the terminal is being used or not. Again, to the send and receiving stations, it appears as if there is a single line connecting them. All lines originate in one location and end in one location. TDM is more efficient, easier to operate, less complex and less expensive than FDM.

- 30c. STDM - Statistical Time Division Multiplexing

Statistical Time Division Multiplexing uses intelligent devices capable of identifying when a terminal is idle. They allocate time only to lines when required. This means that more lines can be connected to a transmission medium as this device statistically compensates for normal idle time in data communication lines. Newer STDM units provide additional capabilities such as data compression, line priority, mixed speed lines, host port sharing, network port control, automatic speed detection and much more.

- 31. Telecommunication Multiplexing

Telecommunication multiplexing is used between switching offices on Interoffice trunks and Intertoll trunks. The Telcos (telecommunication companies such as Bell Canada, AGT, BC-Tel, etc..) share communication facilities which can be either FDM or TDM. A communication path can change in mid-stream from FDM to TDM and back again depending on where or whose communication facility is being used.

FDM is analog and is being updated to TDM throughout the world. Still today, there are locations where FDM is being used.

- 31a. FDM - Channel Groups

Telecommunications FDM is based on channel groups. The basic channel is called the Voice Channel and it has a bandwidth of 0-4 kHz. The channel groups are based on multiples of the voice channel:

| Multiplex Level |  | Voice Circuits |  | Freq Band (kHz) |  | BW (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Voice Channel |  | 1 |  | 0 - 4 |  | 4 |
| Group |  | 12 |  | 60 - 108 |  | 48 |
| Supergroup |  | 60 |  | 312 - 552 |  | 240 |
| Mastergroup |  | 600 |  | 564 - 3,084 |  | 2520 |
| Jumbogroup |  | 3600 |  | 564 - 17,548 |  | 16984 |

- The Mastergroup and Jumbogroup have guard-bands added to the bandwidth.

- A Group is made of 12 Voice Channels.

- A Supergroup (60 Voice channels) is made of 5 Groups (12 Voice Channels).

- A Mastergroup (600 Voice Channels) is made of 10 Supergroups (60 Voice Channels).

- A Jumbogroup (3600 Voice Channels) is made of 6 Mastergroups (600 Voice Channels).

- 31b. TDM - T1 Carrier System

Telecommunications TDM is based on the T1 Carrier System. It is a digital system that digitizes the analog Voice Channel into 8 bit data. This means that there are 28 or 256 levels that the 8 bit data can represent.

| 31b. TDM - T1 Carrier System (cont'd) | Next |
| --- | --- |

It samples the analog signal 8000 times a second (2x 4 kHz - makes Nyquist happy!). It is a serial data stream so we transmit the 8 bit data 1 bit at a time. This means that for a digitized Voice Channel, the data rate is:

8 bits x 8000 samples = 64 Kbps

The basic Carrier used in the T1 Carrier System is called the T1 (sometimes called DS-1) and it carries 24 Voice Channels. The Bit Rate for the T1 Carrier is 1.544 Mbps. If we multiply:

24 Voice Channels x 64 Kbps per Voice Channel = 1.536 Mbps

The missing 8 K is used to "frame" the data. It is information used for the Start Frame bytes, End Frame, Error Checking, Routing information, etc..

| Digital Circuit | Voice Channels | Bit Rate | # of T1 Circuits |
| --- | --- | --- | --- |
| T1 (DS-1) | 24 | 1.544 Mbps | 1 |
| T2 (DS-2) | 96 | 6.312 Mbps | 4 |
| T3 (DS-3) | 672 | 44.736 Mbps | 28 |
| T4 (DS-4) | 4032 | 274.176 Mbps | 168 |

Typically:

- T1 - Twisted Pair or Coax Cable

- T2 - Coax Cable

- T3 - Coax, Fibre Optics or Light Route Radio

- T4 - Coax or Fibre Optics

You can rent any quantity of a T1 line, you don't have to rent the complete circuit. You basically rent a time-slot on the line based on 64 kbps channels. This is called Fractional T-1.

- 32. Introduction to the ISO - OSI Model

The ISO (International Standards Organization) has created a layered model called the OSI (Open Systems Interconnect) model to describe defined layers in a network operating system. The purpose of the layers is to provide clearly defined functions to improve internetwork connectivity between "computer" manufacturing companies. Each layer has a standard defined input and a standard defined output.

Understanding the function of each layer is instrumental in understanding data communication within networks whether Local, Metropolitan or Wide.

- 32a. OSI Model Explained

This is a top-down explanation of the OSI Model, starting with the user's PC and what happens to the user's file as it passes though the different OSI Model layers. The top-down approach was selected specifically (as opposed to starting at the Physical Layer and working up to the Application Layer) for ease of understanding of how the user's files are transformed through the layers into a bit stream for transmission on the network.

There are 7 Layers of the OSI model:

- 7. Application Layer (Top Layer)

- 6. Presentation Layer

- 5. Session Layer

- 4. Transport Layer

- 3. Network Layer

- 2. Data Link Layer

- 1. Physical Layer (Bottom Layer)

- 32b. Layer 7 - Application Layer

Fig. 1 Basic PC Logical Flowchart

A basic PC logical flowchart is shown in Fig. 1. The Keyboard & Application are shown as inputs to the CPU that would request access to the hard-drive. The Keyboard requests accesses to the hard-drive through user enquiries such as "DIR" commands and the Application through "File Openings" and "Saves". The CPU, through the Disk Operating System, sends/receives data from the local hard-drive ("C:" in this example).

| 32b. Layer 7 - Application Layer | Next |
| --- | --- |

Fig. 2 Simple Network Redirection

A PC setup as a network workstation has a software "Network Redirector" (actual name depends on the network - we will use a generic term) placed between the CPU and DOS as in Fig 2. The Network Redirector is a TSR (Terminate and Stay Resident) program which presents the network hard-drive as another local hard-drive ("G:" in this example) to the CPU. Any CPU requests are intercepted by the "Network Redirector". The Network Redirector checks to see if a local drive is requested or a network drive. If a local drive is requested, the request is passed on to DOS. If a network drive is requested, the request is passed on to the network operating system (NOS).

Electronic mail (E-Mail), client-server databases, games played over the network, print and file servers, remote logons and network management programs or any "network aware" application are aware of the network redirector and can communicate directly with other "network applications" on the network. The "Network Aware Applications" and the "Network Redirector" make up Layer 7 - the Application layer of the OSI Model as shown in Fig 3.

Fig. 3 PC Workstation with Network Aware Software

| 32c. Layer 6 - Presentation Layer | Next |
| --- | --- |

- 32c. Layer 6 - Presentation Layer

The Network Redirector directs CPU operating system native code to the network operating system. The coding and format of the data is not recognizable by the network operating system. The data consists of file transfers and network calls by network aware programs.

As an example: when a dumb terminal is used as a workstation in a mainframe or minicomputer network, the network data is translated into and from the format that the terminal can use. The Presentation layer presents data to and from the terminal using special control characters to control the screen display (LF-linefeed, CR-carriage return, cursor movement, etc..). The presentation of data on the screen would depend on the type of terminal VT100, VT52, VT420, etc.

Similarly, the Presentation layer strips the pertinent file from the workstation operating system's file envelope. The control characters, screen formatting and workstation operating system envelope are stripped or added to the file, depending on if the workstation is receiving or transmitting data to the network. This could also include translating ASCII files characters from a PC world to EBCDIC in an IBM Mainframe world.

The Presentation Layer also controls security at the file level. This provides file locking and user security. The DOS Share program is often used for file locking. When a file is in use, it is locked from other users to prevent 2 copies of the same file to be generated. If 2 users both modified the same file and User A saved it then User B saved it - User A's changes would be erased!

At this point, the data is contiguous and complete at this point (one large data file). See Fig. 4.

| 32d. Layer 5 - Session Layer | Next |
| --- | --- |

- 32d. Layer 5 - Session Layer

The Session layer manages the communications between the workstation and network. The Session layer directs the information to the correct destination and identifies the source to the destination. The Session layer identifies the type of information as data or control. The Session layer manages the initial start-up of a session and the orderly closing of a session. The Session layer also manages Logon procedures and Password recognition. See Fig. 5.

Fig. 5 Session Layer

- 32e. Layer 4 - Transport Layer

In order for the data to be sent across the network, the file must be broken up into usable small data segments (typically 512 - 18K bytes). The Transport layer breaks up the file into segments for transport to the network and combines incoming segments into a contiguous file. The Transport layer does this logically not physically, it is done in software as opposed to hardware.

The Transport layer provides error checking at the segment level (frame control sequence). This checks that the datagrams are in the correct order and the Transport layer will correct out of order datagrams. The Transport layer guarantees an error-free host to host connection, it is not concerned with the path between machines.

| 32f. Layer 3 - Network Layer | Next |
| --- | --- |

- 32f. Layer 3 - Network Layer

The Network layer is concerned about the path through the network. It is responsible for routing, switching and controlling the flow of information between hosts. The Network layer converts the segments into smaller datagrams that the network can handle. The Network layer does not guarantee that the datagram will reach its destination. The network hardware source and destination addresses are added.

Fig. 7 Network Layer

- 32g. Layer 2 - Data Link Layer

The Data Link layer is a firmware layer of the network interface card. The Data Link layer puts the datagrams into packets (frames of bits: 1s & 0s) for transmission and assembles received packets into datagrams. The Data Link layer works at the bit level and adds start/stop flags and bit error checking (CRC or parity) to the packet frame. Error checking is at the bit level only, packets with errors are discarded and a request for re-transmission is sent out. The Data Link layer is concerned about bit sequence.

Fig. 8 Data Link Layer

- 32h. Layer 1 - Physical Layer

The Physical layer concerns itself with the transmission of bits and the network card's hardware interface to the network. The hardware interface involves the type of cabling (coax, twisted pair, etc..), frequency of operation (1 Mbps, 10Mbps, etc..), voltage levels, cable terminations, topography (star, bus, ring, etc..), etc.. Examples of Physical layer protocols are 10Base5 - Thicknet, 10Base2 - Thinnet, 10BaseT - twisted pair, ArcNet, FDDI, etc.. See Fig. 9.

Fig. 9 Physical Layer

| 32i.Layer Specific Communication | Next |
| --- | --- |

- 32i. Layer Specific Communication

Each layer may add a Header and a Trailer to its Data which consists of the next higher layer's Header, Trailer and Data as it moves through the layers. The Headers contain information that addresses layer to layer communication specifically. For example: The Transport Header (TH) contains information that only the Transport layer sees and all other layers below the Transport layer pass the Transport Header as part of their Data.

PDU - Protocol Data Unit (fancy name for Layer Frame)

| 32j. OSI Model Functional Drawing | Next |
| --- | --- |

- 32j. OSI Model Functional Drawing

- 33. Synchronous Transmission

Message Frames

Synchronous Transmission sends packets of characters at a time. Each packet is preceded by a Start Frame which is used to tell the receiving station that a new packet of characters is arriving and to synchronize the receiving station's internal clock. The packets also have End Frames to indicate the end of the packet. The packet can contain up to 64,000 bits depending on the protocol. Both Start and End Frames have a special bit sequence that the receiving station recognizes to indicate the start and end of a packet. The Start and End Frames may be only 2 bytes each.

Efficiency

Synchronous transmission is more efficient than asynchronous (character transmission) as little as only 4 bytes (2 Start Framing Bytes and 2 Stop Framing bytes) are required to transmit up to 8K bytes. Extra bytes, like the Start and Stop Frame, that are not part of the data are called overhead. Packet overhead consists of control information used to control the communication.

Efficiency example: An Ethernet frame has an overhead of 26 bytes including the "Start and Stop Frames", the maximum data size is 1500 bytes. What is the Ethernet frame's efficiency?

| 33a. Clocking: Self & Manchester Encoding | Next |
| --- | --- |

- 33a. Clocking: Self & Manchester Encoding

Synchronous transmission is more difficult and expensive to implement than asynchronous transmission. It is used with all higher transfer rates of communication: Ethernet, ArcNet, Token Ring etc... Synchronous transmission is used in fast transfer rates 100 Kbps to 100 Mbps. In order to achieve the high data rates, Manchester Line Encoding is used.

In the Manchester Code, there is a transition at the middle of each bit period. The mid-bit transition serves as a clocking mechanism and also as data: a low to high transition represents a 1 and a high to low transition represents a 0.

Manchester Encoding has no DC component and there is always a transition available for synchronizing receive and transmit clocks. Because of the continuous presence of these transitions, Manchester Encoding is also called a self clocking code.

It has the added benefit of requiring the least amount of bandwidth compared to the other Line Codes (Unipolar, Polar, etc..). Manchester coding requires 2 frequencies: the base carrier and 2 x the carrier frequency. All other types of Line Coding require a range from 0 hertz to the maximum transfer rate frequency. In other words, Manchester Encoding requires a Narrow Bandwidth

- 34. Basic Frame Structure

The Generic Packet X is used as an introduction to Synchronous Data Transmission. As we explore more standards and protocols, we find that we can expand the frame structure (packet) into better defined sections that will allow easier understanding of different frame types (Ethernet, Token Ring, SDLC, HDLC, Frame Relay, ATM, Cell Relay, etc...). It also will provide a point of reference.

Basic Frame Structure

- 34a. Preamble: Starting Delimiter/Alert Burst/Start of Header

At the beginning of each frame (packet), there will be a sequence of octets (8 bit words), called the Preamble. The Preamble is used to:

- Inform the receiving station that a new packet is arriving

- Synchronize the receive clock with the transmitted clock

The Preamble is a series of octets with a specific bit pattern that is used only by the Preamble.

Names used by other protocols for the Preamble are: Starting Delimiter, Alert Burst and Start of Header. All perform the same 2 basic functions.

- 34b. Address Field(s): Source and/or Destination

The Address Field consists of a Source Address and/or a Destination Address. The Source and Destination Addresses are hexadecimal numbers that identify the sender - Source and receiver - Destination. The Network Addresses reside in either the Network Interface Card's firmware or can be either assigned during the initialization of the NIC.

The purpose of the Source Address is to identify to the network who is sending data. The purpose of the Destination Address is to identify to the network who should be receiving the data.

Under some protocols, there may not be both Source and Destination Addresses. Only one address may be present.

- 34c. Control Field

The Control Field is used to indicate the Type of Information being sent as Data. The Type of Information can be Control information used when establishing a connection (handshaking) or it can be Data such as file transfers between clients and servers. The purpose of the Control Field is to identify what the purpose of the packet or frame is: Control or Data. It can also be used to indicate the size of the packet and Data.

- 34d. Data/Message and optional Pad

The Data Field or Message is the actual information that is being transmitted. It can contain Control Information for handshaking or actual Data used by applications. The Control Field would indicate the Data Field size. The Data field is also called the Info field by some protocols.

The optional Pad is used to pad the data field when the protocol has a fixed Data Field size. If the Data Field size is fixed at 1200 octets and only 300 octets of information is available then the Pad will fill in the remaining 900 octets with characters (e.g. 900 octets of 00h). The protocol may also use the Pad to ensure a minimum Data field size.

- 34e. CRC/ Frame Check Sequence

The CRC / Frame Check Sequence (FCS) contains an error checking number that the Destination can use to verify that the packet is okay and error-free. CRC is an abbreviation for Cyclic Redundancy Checking. The Frame Check Sequence typically incorporates a 32 Bit CRC check. Checksums work similarly but use a different algorithm.

As each packet is sent, the Source calculates a check number from the data using a predetermined algorithm (formula). The result of this calculation is appended to the packet in the Frame Check Sequence (FCS) field. At the Destination, the same calculation is performed and the result is compared to the transmitted Frame Check Sequence. If the result generated at the Destination is identical to the FCS, then it is assumed that the packet is error free at the bit level.

- 34f. End Frame Delimiter

The End Frame Delimiter is a series of octets that have a specific bit pattern that identifies the end of the packet to the Destination. Not all protocols have End Frame Delimiters fields, protocols with fixed packet size may not need the End Frame Delimiter field as the Destination may simply count the number of octets it has received.

- 35. Physical Layer

The OSI Model Physical Layer concerns itself with the transmission of bits through the communication medium. The order of the bits and importance is determined by the Protocol's packet.

- 35a. Asynchronous & Synchronous Communication

In Asynchronous Communications, the OSI Physical layer concerned itself with the RS-232D standard and the Voice Channel. The RS-232D standard stated the electrical and mechanical characteristics of the cable for the transmission of the digital signal between the DTE (PC) and DCE (modem). The Voice Channel stated the electrical and mechanical characteristics of the connection between DCE to DCE (modem to modem) through the phone lines.

The order of the bits was determined by the ASCII characters, the parity (Odd/Even/None), number of Stop Bits and the Transfer Protocol used. Examples of Transfer Protocols are:

- Kermit

- Xmodem

- Ymodem

- Zmodem

Similarly, in Synchronous Communications, the electrical and mechanical characteristics of the cable for the transmission of the signal are defined by the protocol used between Network Interface Cards.

The electrical characteristics associated with the OSI Model's Physical layer are:

- Transmission rate (bits/sec)

- Voltage levels

- Line Encoding

- Propagation delay

- Termination impedance

The mechanical characteristics associated with the OSI Model's Physical layer are:

- Connector type

- Cable type & size

- Cable Length

- Topology

- Shielding

In summary, the OSI Physical Layer is concerned with the transmission of bits on the network: the order of bits, bit level error-checking, and the electrical & mechanical characteristics.

- 36. IEEE-802.3 Protocol

The IEEE-802.3 Protocol is based on the Xerox Network Standard (XNS) called Ethernet. The IEEE-802.3 Protocol is commonly called Ethernet but it is just 1 version. There are 4 versions or flavours of the Ethernet frame:

| Ethernet_802.2 |  | Frame type used on Netware 3.12 & 4.01 |
| --- | --- | --- |
| Ethernet_802.3 |  | Frame type used on Netware 3.x & 2.x (raw) |
| Ethernet_II |  | Frame type used on DEC, TCP/IP |
| Ethernet_SNAP |  | Frame type used on Appletalk (SubNet Access Protocol) |

NOTE: The Source and Destination must have the same Ethernet Frame type in order to communicate.

- 36a. CSMA/CD (Carrier Sense Multiple Access/ Collision Detect)

Bus arbitration is performed on all versions of Ethernet using the CSMA/CD (Carrier Sense Multiple Access/ Collision Detect) protocol. Bus arbitration is another way of saying how to control who is allowed to talk on the (medium) and when. Put simply, it is used to determine who's turn it is to talk.

In CSMA/CD, all stations, on the same segment of cable, listen for the carrier signal. If they hear the carrier, then they know that someone else it talking on the wire. If they don't hear carrier then they know that they can talk. This is called the Carrier Sense portion of CSMA/CD.

All stations share the same segment of cable and can talk on it similar to a party line. This is the Multiple Access portion of CSMA/CD.

If 2 stations should attempt to talk at the same time, a collision is detected and both stations back off for a random amount of time and then try again. This is the Collision Detect portion of CSMA/CD.

- 36b. IEEE 802.3 Ethernet Media Types

IEEE 802.3 defines 5 media types of IEEE 802.3 Ethernet Types:

| IEEE 802.3 |  | 10Base5 |  | Thick Coax |  | 10Mbps |  | Baseband |  | 500m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IEEE 802.3a |  | 10Base2 |  | Thin Coax |  | 10Mbps |  | Baseband |  | 185m |
| IEEE803b |  | 10Broad36 |  | Broadband |  | 10 Mbps |  | Broadband |  | 3600m |
| IEEE802.3e |  | 1Base5 |  | StarLAN |  | 1 Mbps |  | Baseband |  | 500m |
| IEEE 802.3i |  | 10BaseT |  | Twisted Pair |  | 10Mps |  | Baseband |  | 100m |

IEEE 802.3 - 10Base5 (Thick Coax) is used only as backbones to networks. Backbones are lines that connect buildings & network equipment together such as Bridges, Routers, Brouter, Hubs, Concentrators, Gateways, etc.. 10Base5 is being replaced by either Thin Coax or fibre optics.

IEEE 802.3a - 10Base2 is commonly used in new installations as a backbone to connect buildings and network equipment together. 10Base2 (Thin Coax) is also used to connect work-stations together but the preferred choice is to use 10BaseT.

IEEE 802.3b - 10Broad36 is rarely used, it combined analog and digital signals together. Broadband means that a mixture of signals can be sent on the same medium.

IEEE 802.3e - StarLAN is a slow 1 Mbps standard that has been replaced by Thin Coax or Twisted Pair.

IEEE 802.3i - 10BaseT is commonly used to connect workstations to network hubs. The network hubs can use 10BaseT (Twisted Pair) to connect to other Hubs.

- 36c. IEEE 802.3 10Base5

10Base5 Specifications :

Coaxial Cable

Uses double shielded 0.4 inch diameter RG8 coaxial cable about the size of a garden hose. The cable is not flexible and difficult to work with. The cable has a characteristic impedance of 50 ohms.

Connection to the workstation is made with a MAU - Medium Attachment Unit or Transceiver. The MAU physically and electrically attaches to the coaxial cable by a cable tap. The cable is pierced and a connection is made by a screw to the center conductor.

The MAU is connected to the NIC (Network Interface Card) by the AUI (Attachment Unit Interface) cable. The AUI port on a NIC and a MAU is a DB15 connector. Maximum AUI cable length is 50 m.

Cable Termination and Connector

The standard termination is 50 +/-2 ohms. The end connector on the RG-8 cable is an "N" type connector. The cable is externally terminated with a resistor inside an N connector.

Grounding

To minimize noise on the segment, the cable is grounded at the termination at only one end.

Maximum Nodes on a cable segment

On any 1 cable segment, the maximum allowed number of nodes or MAUs is 100.

Minimum Distance between nodes

Minimum distance between nodes or MAUs is 2.5 m or 8 feet.

Velocity of propagation

The speed of the signal through the cable is 0.77c. ("c" is equal to the speed of light - 300,000,000 m/sec). The velocity of propagation for 10Base5 specification cable is equal to 0.77 x 300,000,000 m/sec. This is determined by cable capacitance. Maximum coaxial cable segment length 500 m

The maximum segment length is 500 m or a maximum 2.165 uSec propagation delay. Propagation delay is what actually determines the maximum length of the segment.

Propagation delay for a specific cable length in meters is calculated by:

What is the propagation delay for a 500 m length of 10Base5 cable?

Maximum Number of Segments

Maximum of 5 segments (with 4 repeaters) can be along the path between any 2 network nodes: 3 may be coax segments having a maximum delay of 2.165 uSec and 2 may be link segments having a maximum delay of 2.570 uSec.

With no link segments used 3 populated coax segments can exist on a path.

5-4-3 Rule

The 5-4-3 Rule states that you are allowed 5 segments with 4 repeaters and 3 populated segments.

Maximum Transfer Rate

The Maximum Data Transfer Rate for IEEE 802.3 is 10 Mbps (10,000,000 bits per second of data). In actual fact, data transfer is dependant on how many users are fighting for the bus and how fast the user's data can get on the bus.

Physical Bus/Logical Bus

IEEE 802.3 is a Physical Bus - the cable is physically laid out as 1 long cable with the network nodes attached to it. It is also treated as a Logical Bus - electronically and logically it appears as 1 long cable with the network nodes attached to it.

- 36d. IEEE 802.3a 10Base2

Coaxial Cable

Uses RG-58A/U coaxial cable, 0.2 inch in diameter. The cable is flexible and easy to work with. The cable has a characteristic impedance of 50 ohms.

Connection to the workstation is made with either a MAU - Medium Attachment Unit/Transceiver or directly to the NIC using a BNC TEE.

Most NICs have the MAU built-in for 10Base2. The 3C509 card in the lab have built-in MAUs for Coax (10Base2) and Twisted Pair (10BaseT). They also have a AUI connection for an external MAU such as used in 10Base5. You can buy MAUs for 10Base2 and 10BaseT if your NIC does not have them already built-in.

Cable Termination and Connector

The standard termination is 50 +/-2 ohms. The end connector is an "BNC" twist and lock type connector. The cable is externally terminated with a special terminating BNC connector. BNC stands for Bayonet Navy Connector.

Grounding

To minimize noise on the segment, the cable is floating. The IEEE 802.3a specifications calls for all BNC connectors and TEEs to be insulated. A common problem with 10Base2 is having the barrel of the BNC connector touching a heating duct or computer chassis. The shield should be floating, it is not connected to electrical ground.

Maximum Nodes on a cable segment.

On any 1 cable segment, the maximum allowed number of nodes is 30.

Minimum Distance between Nodes

Minimum distance between nodes is 0.6 m or 2 feet.

Velocity of propagation

The speed of the signal through the 10Base2 cable is 0.65c. ("c" is equal to the speed of light - 300,000,000 m/sec). The minimum velocity of propagation for 10Base2 specification cable is equal to 0.65 x 300,000,000 m/sec. This is determined by cable capacitance.

Maximum coaxial cable segment length 185 m.

The maximum segment length is 185 m (600 ft.) or a maximum 0.949 uSec propagation delay. Propagation delay not distance is what actually determines the maximum length of the segment. Propagation delay (units are seconds) is calculated by:

What is the propagation delay for a 185 m length of 10Base2 cable?

Maximum Number of Segments

Maximum of 5 segments (with 4 repeaters) can be along the path between any 2 network nodes: 3 may be coax segments having a maximum delay of 0.949 uSec and 2 may be link segments having a maximum delay of 0.949 uSec.

With no link segments used 3 populated coax segments can exist on a path.

Maximum Transfer Rate

The Maximum Data Transfer Rate for IEEE 802.3a is 10 Mbps (10,000,000 bits per second of data). In actual fact, data transfer is dependant on how many users are fighting for the bus and how fast the user's data can get on the bus.

Physical Bus/Logical Bus

IEEE 802.3a is a Physical Bus - the cable is physically laid out as 1 long cable with the network nodes attached to it.

It is also treated as a Logical Bus - electronically and logically it appears as 1 long cable with the network nodes attached to it.

- 36e. IEEE 802.3i 10BaseT

Twisted Pair Cable

10BaseT uses unshielded twisted pair (UTP) cable. The cable is flexible and easy to work with. The cable has a characteristic impedance of 100 ohms. There are 2 pairs of twisted wires used with 10BaseT. Separate Rx (receive) and Tx (transmit) pairs are used. The lines are balanced lines to minimize noise and there are a Rx+ & Rx- pair and a Tx+ & Tx- pair.

The nodes are connected to a MPR (multiport repeater) also called a Concentrator or Hub. The cables are wired as straight-through cables meaning the Node's Rx & Tx lines connect directly to the Hub's Rx & Tx lines respectively.

Two nodes can be directly connected together bypassing the Hub by using a Cross-over (X-over) cable. In a X-over cable, the Tx and Rx lines are crossed so that one node's Tx lines go to the other nodes Rx lines and vice versa.

Cable Termination and Connector

The standard termination is 100 ohms. The end connector is an "RJ45" quick disconnect connector. The cable is internally terminated at the NIC and Hub.

Grounding

To minimize noise on the segment, the cable is a balanced line with Rx- & Rx+ and Tx- & Tx+. There is no shielding and any noise that appears on the Rx+ wire will appear on the Rx- wire. When the 2 signals are combined, the noise cancels due to Rx- & Rx+ being 180 degrees out of phase.

Maximum Nodes

For 10BaseT, the maximum allowed number of nodes is 128 on one segment.

Maximum Distance between Nodes & Hub

Maximum distance between nodes & Hub is 100 m.

Velocity of propagation

The speed of the signal through the cable is 0.59c. ("c" is equal to the speed of light - 300,000,000 m/sec). The minimum velocity of propagation for 10Base5 specification cable is equal to 0.59 x 300,000,000 m/sec. This is determined by cable capacitance.

Maximum cable segment length 100 m

The maximum segment length is 100 m or a maximum 0.565 uSec propagation delay. Propagation delay not distance is what actually determines the maximum length of the segment. Propagation delay (units are seconds) is calculated by:

What is the propagation delay for a 100 m length of 10BaseT cable?

Maximum Number of Segments

Maximum of 5 segments (with 4 repeaters) can be along the path between any 2 network nodes: 3 may be coax segments having a maximum delay of 0.565 uSec and 2 may be link segments having a maximum delay of 0.565 uSec. The 5-4-3 rule will be discussed under Repeaters and its special implications for IEEE 802.3i.

Maximum Transfer Rate

The Maximum Data Transfer Rate for IEEE 802.3i is 10 Mbps (10,000,000 bits per second of data). In actual fact, data transfer is dependant on how many users are fighting for the bus and how fast the user's data can get on the bus.

Physical Star/Logical Bus

IEEE 802.3a is a Physical Star - the cable is physically laid out as star pattern with all twisted pair cables (AUIs) coming from the nodes to a central wiring closet containing the Hub (Multi-Port Repeater / Concentrator)

It is treated as a Logical Bus - electronically and logically it appears as 1 long cable with the network nodes attached to it. A node can be a client, server, workstation or other hub.

- 36f. MAC - Medium Access Control

The IEEE 802.3 Medium Access Control layer is physically located in the firmware (ROM) of the Network Interface Card. It is the link between the Data Link Layer and the Physical Layer of the OSI model and logically resides in the lower portion of the Data Link Layer. There is only 1 MAC layer for all IEEE 802.3 versions: 802.3, 802.3a, 802.3b, 802.3i, etc..

The IEEE 802.3 Medium Access Control uses CSMA/CD (Carrier Sense Multiple Access/Collision Detect) to determine Bus Arbitration. The MAC layer is concerned with the order of the bits and converting the Datagram from the Network Layer into Packets/Frames.

Preamble

The Preamble is used to synchronize the receiving station's clock. It consists of 7 bytes of 10101010.

Start Frame Delimiter (SFD)

The Start Frame Delimiter indicates the start of the frame. It consists of 1 byte of 10101011. It is an identical bit pattern to the preamble except for the last bit.

- Start Frame Delimiter (SFD)

The Start Frame Delimiter indicates the start of the frame. It consists of 1 byte of 10101011. It is an identical bit pattern to the preamble except for the last bit.

The Destination Address (DA)

Indicates the destination (receiving station) of the frame. It can be 2 or 6 octets long (16 or 48 bits), usually it is 6 octets (the 2 octet version is used for compatibility with the original Ethernet frame from XNS and is considered obsolete).

The DA field consists of

I/G stands for Individual/Group. It indicates whether the destination is for an individual or for a multicast broadcast. It is one bit long:

0 = Individual 1 = Group

A multicast broadcast can be for everyone or for a group. For a multicast broadcast to all stations, the Destination Address = FFFFFFFFFFFFh (h - hexadecimal notation). To multicast to a specific group, unique addresses must be assigned to each station by the Network Administrator.

U/L stands for Universal/Local. It allows for unique addresses. It is used to indicate whether a local naming convention is used - administered by the Network Administrator (not recommended - incredible amount of work) or the burnt-in ROM address is used (recommended).

The 46 Bit Address Field consists of 46 bits indicating the destination NIC cards address burnt into the firmware (ROM) of the card or the unique name assigned to the card during the card's initialization by the Network Administrator.

Source Address (SA)

The Source Address indicates the source or transmitting station of the frame. It is identical in format to the Destination Address but always has the I/G bit = 0 (Individual/Group Bit = Individual)

Length (L)

The Length field indicates the Length of the Information Field. It allows for variable length frames. The minimum Information Field size is 46 octets and the maximum size is 1500 octets. When the Information Field size is less than 46 octets, the Pad field is used. Due to the 802.3 MAC Frame having a Length field, there is no End Delimiter in the MAC Frame. The Length of the field is known and the receiving station counts the number of octets.

Information Field (Data)

The Information Field contains the Data from the next upper layer : Logical Link Control Layer. It is commonly referred to as the LLC Data. The minimum Information Field size is 46 octets and the maximum size is 1500 octets.

Pad

The Pad is used to add octets to bring the Information Field up to the minimum size of 46 octets if the Info Field is less than the minimum.

Frame Check Sequence (FCS)

The Frame Check Sequence is used for error-checking at the bit level. It is based on 32 bit CRC (Cyclic Redundancy Checking) and consists of 4 octets (4 x 8 = 32 bits). The FCS is calculated according to the contents of the DA, SA, L, Data and Pad fields.

- 36g. Total Length of a MAC Frame

- 36i. Packet Sniffing

A packet sniffer captures packets from the Ethernet bus. The network interface card (NIC) acts in a mode called promiscious mode. Promiscious mode means that the NIC can look at all traffic on the wire and not just to traffic addressed to itself. Normally, the NIC ignores all traffic except for packets addressed to itself, multicasts and broadcast packets.

The following captured packet is displayed in raw format. Raw format is hexadecimal numbers in rows of 16 digits.

FF  FF  FF  FF  FF  FF  00  20  AF  10  9A  C0  00  25  E0  E0

03  FF  FF  00  22  00  11  00  00  00  00  FF  FF  FF 	FF  FF

FF  04  52  00  00  00  00  00  20  AF  10  9A  C0  40  0B  00

01  00  04  00  00  00  00  00  00  00  00  00

Raw Captured Packet

Raw captured packets do not display the Preamble, Start Frame Delimiter and the Frame Check Sequence fields. These fields are used to inform the receiving station of a new frame and for error checking.

The breakdown of the packet is according to the Ethernet MAC frame and as follows:

| 1st 6 bytes: |  | FF-FF-FF-FF-FF-FF |  | Destination MAC address |
| --- | --- | --- | --- | --- |
| 2nd 6 bytes: |  | 00-20-AF-10-9A-C0 |  | Source MAC address |
| Next 2 bytes: |  | 0025 |  | Length/Type field - IEEE 802.3 frame |
| Next 37 bytes |  |  |  | Data |
| Last 9 bytes |  | all 00s |  | Pad |

The length of the data in the Info field is 0025h or 37 bytes long. The minimum Info field size is 46 bytes so the data is padded with 9 bytes of 00h.

The Length/Type field value is less than 05DCh (1500 in decimal) which indicates that it is an Ethernet_802.2 frame (IEEE 802.3) with a Logical Link Control layer (covered later) between the MAC layer and the Network layer.

If the value was 0800h, it would indicate an Ethernet_II frame used for TCP/IP.

If it were 8137, it would indicate an Ethernet_802.3 (raw) frame used by pre 3.12 Netware.

The complete listing of the Length/Type field assignments is covered in Appendix C Ethernet Type Field.

Looking at the MAC block diagram, the data from the Info field is shown broken down (up to be more exact) into the higher levels: Logical Link Control layer, Network layer and the Transport layer. Note: A thorough knowledge of each of the layers and quite a few handy reference books are required in order to determine exactly what is happening. The remaining layers will be examined as an example only.

NOTE: Modern packet sniffer will break down the raw packet's fields for you.

LLC Layer

The first 3 bytes of the data in the Ethernet frame Info field is the header of the Logical Link Control layer (LLC IEEE 802.2).

1st byte:	E0	Destination Service Access Port (DSAP)

2nd byte:	E0	Source Service Access Port (SSAP)

3rd byte:	03	Control code

E0h indicates that it is a Novell Netware stack talking (source) to a Novell Netware stack (destination). The 03h is the LLC layer's handshaking. The size of the LLC's Data field is 34 bytes. The LLC layer is covered extensively in the following chapter.

Network Layer

The data of the LLC layer becomes the header and data of the layer above it which is the Network layer. In this case, it is an IPX PDU (Protocol Data Unit) which is indicated by the first 2 bytes being FFFFh - the IPX checksum.

(Hex)

1st 2 bytes:	FFFF		IPX Checksum (always FFFFh, FCS does error checking)

Next 2 bytes:	0022		IPX PDU length allowable range 001Eh (30) to 0240h (576)

Next byte:	00		Transport control field - hop count, allowed 00 to 0Fh (15)

Next byte:	11		Packet Type 11h (17) is Netware Core Protocol (NCP)

Next 4 bytes:	00000000	Destination network address, all 0s indicate local network

Segment number in server autoexec.ncf file

Next 6 bytes:	FFFFFFFFFFFF	Destination host address (same as dest MAC address)

Next 2 bytes:	0452		Destination socket , Service Advertising Protocol

Next 4 bytes:	00000000	Source network address (all 0s indicate local network)

Next 6 bytes:	0020AF109AC0	Source host address (same as soruce MAC address)

Next 2 bytes:	400B		Source socket (arbitrarily assigned starting at 4000h)

Last 4 bytes:	Data

The following tables describe the field values for the IPX PDU's packet type and Socket numbers:

Packet Type	Field Value 	Purpose

NLSP		00h		Netware Link Services Protocol

RIP		01h		Routing Information Protocol

SAP		04h		Service Advertising Protocol

SPX		05h		Sequenced Packet Exchange

NCP		11h		Netware Core Protocol

NetBIOS		14h		NetBIOS and other propagated packets

IPX Packet Type Field

Netware Socket Numbers and Processes

Socket Number		Process

451h		Netware Core Protocol (NCP)

452h		Service Advertising Protocol (SAP)

453h		Routing Information Protocol (RIP)

455h		Novell NetBIOS

456h		Diagnostics

9001		Netware Link Services Protocol (NLSP)

9004		IPXWAN Protocol

Transport Layer

The Network layer's Data field becomes the Transport layer's PDU. In this case it is only 4 bytes long.

1st 2 bytes:	0001	Packet type (Standard Server Request)

Next 2 bytes:	0004	Service type (file server)

The following tables describe the values of the Service Advertising Protocol's Packet Type and Service Type fields:

Field Value (hex)	Packet Type

01		Standard Server Request

02		Standard Server Reply

03		Nearest Server Request

04		Nearest Server Reply

SAP Packet Types

Field Value (hex)	Service Type

0000		Unknown

0003		Print Queue

0004		File Server

0005		Job Server

0007		Print Server

0009		Archive Server

0024		Remote Bridge Server

0047		Advertising Print Server

8000		All values are reserved up to 8000

FFFF		Wildcard

Example Packet Sniffing Summary

This packet is commonly called a Standard Server Request that is broadcast (Destination FF-FF-FF-FF-FF-FF) on the local network (00-00-00-00) from a Novell Netware client. The client is looking for a file server to login in to. The server would respond with a Server Advertising Protocol PDU listing its services.

| 36j. Packet Sniffing Block Diagram | Next |
| --- | --- |

- 37. IEEE 802.2 LLC - Logical Link Control Layer

The Logical Link Control Layer resides in the upper portion of the Data Link Layer. The LLC layer performs these functions:

- Managing the data-link communication

- Link Addressing

- Defining Service Access Points (SAPs)

- Sequencing

The LLC provides a way for the upper layers to deal with any type of MAC layer (ex. Ethernet - IEEE 802.3 CSMA/CD or Token Ring IEEE 802.5 Token Passing).

The Data field of the MAC layer Frame transmits the LLC Protocol Data Unit.

LLC PDU Format

- 37a. Service Access Ports (SAPs)

SAPs are Service Access Ports. A SAP is a port (logical link) to the Network layer protocol. If we were operating a multiprotocol LAN, each Network Layer protocol would have its own SAP. This is the method that the LLC uses to identify which protocol is talking to which. For example, Unix's TCP/IP, Novell's SPX/IPX and IBM's Netbios would all have different SAPs to identify which was which.

Address		Assignment

00		Null LSAP

02		Individual LLC Sublayer Management Function

03		Group LLC Sublayer Management Function

04		IBM SNA Path Control (individual)

05		IBM SNA Path Control (group)

06		ARPANET Internet Protocol (IP)

08		SNA

0C		SNA

0E		PROWAY (IEC955) Network Management & Initialization

18		Texas Instruments

42		IEEE 802.1 Bridge Spannning Tree Protocol

4E		EIA RS-511 Manufacturing Message Service

7E		ISO 8208 (X.25 over IEEE 802.2 Type 2 LLC)

80		Xerox Network Systems (XNS)

86		Nestar

8E		PROWAY (IEC 955) Active Station List Maintenance

98		ARPANET Address Resolution Protocol (ARP)

BC		Banyan VINES

AA		SubNetwork Access Protocl (SNAP)

E0		Novell NetWare

F0		IBM NetBIOS

F4		IBM LAN Management (individual)

F5		IBM LAN Management (group)

F8		IBM Remote Program Load (RPL)

FA		Ungermann-Bass

FE		ISO Network Layer Protocol

FF		Global LSAP

DSAP stands for Destination Service Access Port and is the receiving station's logical link to the Network Layer protocol. SSAP stands for Source Service Access Port and is the transmitting station's logical link to the Network Layer Protocol.

SAPs ensure that the same Network Layer protocol at the Source talks to the same Network Layer protocol at the Destination. TCP/IP talks to TCP/IP, Netbios talks to Netbios and IPX/SPX talks to IPX/SPX.

- 37b. Types of LLC Operation

LLC defines 2 types of operation for data communication:

- Type 1: Connectionless

- Type 2: Connection Oriented

Type 1: Connectionless

Connectionless service for data communications is very similar to sending mail with the postal system (hand delivered mail). The data is sent and we hope it arrives at its destination. There is no feedback from the destination to indicate whether it arrived or not.

Type 1: Connectionless Service

Type 2: Connection Oriented

Connection Oriented service for data communications is very similar to having a phone conversation. First a connection is made and established by dialing the number, waiting for it to ring, someone picking up the line and saying hello. This establishes the connection. During the conversation, confirmation that the other person is still there (hasn't fallen asleep or died) and listening is given by hearing things like: yeah, oh really, uh huh, etc.. This is the acknowledgement of receipt of data. If the destination party did not hear something correctly, they ask to have it repeated which is called automatic repeat request (ARQ).

Connection Oriented service

NOTE: These models for connectionless and connection-oriented can be used for any protocol.

Type 2: Connection Oriented operation for the LLC layer provides 4 services:

- Connection establishment

- Confirmation and acknowledgement that data has been received.

- Error recovery by requesting received bad data to be resent.

- Sliding Windows (Modulus: 128)

Sliding Windows are a method of increasing the rate of data transfer. Type 2 Connection Oriented operation calls for every Protocol Data Unit (LLC frame) sent to be acknowledged. If we waited for every PDU to be acknowledged before we sent the next PDU, we would have a very slow data transfer rate.

For example: If we were contacting Microsoft in Sunnyvale California, it might take 2 seconds for our LLC PDU to reach Microsoft and another 2 seconds for the acknowledgement to return. This would mean that we are only sending 1 PDU every 4 seconds. If our PDU was IEEE 802.3 MAC's limit of 1500 octets (8x1500 = 12 Kbits), we would actually be transferring at 3 Kbps (12 kbits/4 seconds). This would be regardless of our actual transfer rate! Waiting for an acknowledgement is controlling the data transfer rate!

To overcome this problem, a Sliding Window system of data transmission is used. Each PDU is sequentially numbered (0 - 127). Rather than wait for an acknowledgement, the next PDU is numbered and sent out. The receive station LLC layer acknowledges with the received PDU's numbers back to the transmit station. The LLC will allow up to 128 PDUs to be sent and not acknowledged before it sounds an error alarm.

The received station LLC layer keeps track of the PDUs it is receiving and if one should be lost during transit, it requests the Source to restart transmitting at that PDU number. All PDUs since the lost PDU are discarded.

It is called a Sliding Window because the number of unacknowledged PDUs is determined by the time it takes to get to the destination and for the destination to acknowledge the receipt of the PDU. This time is dependant on the transfer rate and the physical distance the PDU must travel. It is set automatically and we do not have to worry about it.

- 37c. Classes of LLC

There are 2 Classes of Logical Link Control defined:

Class I :	Type 1 operation only (connectionless)

Class II:	Both Type 1 (connectionless) and

Type 2 (connection-oriented) operation allowed.

- 37d. LLC PDU Control Field Formats

There are 3 LLC PDU Control field formats:

- Un-numbered (U-Format PDU)

- Information Transfer (I-Format PDU)

- Supervisory (S-Format PDU)

Un-numbered (U-Format PDU)

The last 2 bits are set to 1, to indicate U-Format Control Field.

M - Modifier bits, they are set depending on the mode of operation: Command, Response or Data

P/F - Poll/Final bit, this bit is used by the Source to solicit a response from the Destination. It is used by the Destination to respond to a solicit from the Source.

The Un-numbered LLC Control field is used mainly in Type 1 (connectionless) operation. The PDUs are not numbered, they are sent out and hopefully arrive at their destination. U-Format PDUs can be commands, responses and data. There are only 8 bits in a U-Format LLC PDU. In the U-Format (Unnumbered), there are 8 commands & responses:

UI	- Unnumbered information 			(here's some data - hope it arrives)

DISC	- Disconnect 					(we're done, shut her down)

SABME	- Set Asynchronous Balanced Mode Extended 	(start now)

XID	- Exchange IDs					(Here's who I am, who are you?)

TEST	- Test the link 				(Here's a test, send me back a test)

UA	- Unnumbered Acknowledgement			(Yes, I'm still here)

DM	- Disconnect Mode 				(I'm disconnecting)

FRMR	- Frame Reject 					(Bad frame - reject)

Information Transfer (I-Format PDU)

It is used for transferring information or data between Source and Destination in a Type 2 (connection oriented) operation. It is the only LLC PDU allowed to transfer information in Type 2 operation.

I-Format Control Field Format

The last bit is set to 0, to indicate that it is an I-Format Control Field.

P/F - Poll/Final bit, this bit is used by the Source to solicit a response from the Destination. It is used by the Destination to respond to a solicit from the Source.

N(R) - PDU number received. Used with the Sliding Window and for acknowledging PDUs.

N(S) - PDU number sent. Used with the Sliding Window and for acknowledging PDUs.

The N(R) bits are commonly called "Piggyback Acknowledgment" because the response is acknowledged along with the transfer of data. The acknowledgement is piggybacked onto a data transfer.

In the I-Format (Information), there are no commands & responses but typically indicated by:

I	- Information (data transfer)

Supervisory (S-Format PDU)

Supervisory (S-Format) LLC Control fields are used for Data Link supervisory control functions (handshaking). The S-Format Control fields are used for acknowledging I-Format PDUs, requesting retransmission, requesting a temporary suspension of transmission (buffers full - wait).

S-Format LLC PDU Control Field

The last 2 bits are set to 0 1, to indicate that it is a S-Format Control Field

S - Supervisory function bits. Determines the purpose of the control field

The four 0s in a row are reserved bits and are always set to 0.

P/F - Poll/Final bit, this bit is used by the Source to solicit a response from the Destination. It is used by the Destination to respond to a solicit from the Source.

N(R) - PDU number received. Used with the Sliding Window and for acknowledging PDUs.

In the S-Format (Supervisory), there are 3 commands & responses:

RR	- Receive Ready (awake & ready to receive)

RNR 	- Receive Not Ready (got problems, hold off for awhile)

REJ 	- Reject (received a bad PDU, send the PDU with this number again)

- 38. Network Interface Cards

There are 3 configuration types of Network Interface Cards (NIC):

- jumper configurable

- software configurable

- Plug n Play (PnP)

Jumper configurable cards have physical jumpers that you use to select the IRQ, I/O address, upper memory block and transceiver type (10BaseT, 10Base2 or 10Base5). Older cards will also allow selecting DMA channel - this was used with XT and 286 PCs.

Software configurable NICs have a proprietary software program that sets the NIC's "internal jumpers". They are usually menu driven and have an auto configuration mode, where the program will attempt to determine the most suitable configuration. These programs are not foolproof, you still require a thorough knowledge of the PC's architecture.

Plug n Play NICs will attempt to auto-configure themselves during the bootup sequence immediately after installation. They also come with a proprietary software program in case that anything goes wrong and you have to manually configure them.

A combination (combo) NIC has the option of connecting to the network using either Twisted Pair (10BaseT), Coax (10Base2) or AUI (Attachment Unit Interface for 10Base5). The NIC can only connect to one medium type at a time and the configuration software allows you to select which medium interface to connect to. Newer NICs will autodetect the cabling type used.

- 38a. IRQs, DMAs and Base Addresses

When a NIC is configured, you are setting the parameters which tell the computer network software where to find the adapter (base address) and who is "tapping the CPU on the shoulder" (IRQ). The base address is the pointer to the rest of the world that says "Here I am at base address xxx!". The IRQ is the "tap on the shoulder" to the CPU that says "Hey, it's IRQx, I've got something important to say!". The Upper Memory Block is the NIC's BIOS or actual program in the NIC's ROM. It is set to a free area of memory in the PC's upper memory - to avoid conflicts with other devices (video cards, internal modems, SCSI drivers, etc..).

IRQ - Interrupt Requests

IRQ stands for Interrupt ReQuest. It is a "tap on the shoulder" to the CPU by a peripheral card plugged in an ISA slot to tell the CPU that the peripheral has something to say (also used by EISA and MCA slots). Common peripherals are modems, NICs (network interface cards), sound cards, SCSI adapters, hard-drive controllers, floppy drive controllers, COM ports and printer ports.

An IRQ is a hardware interrupt, this means that there is a physical line run to each of the ISA slots on the motherboard. There are 2 types of ISA slots: 8 bit and 16 bit. The 16 bit consists of the 8 bit slot plus a 16 bit extension slot. There are 8 IRQ (IRQ0-7) lines that run to the 8 bit ISA slot. There are 8 more (IRQ8-15) that run to the 16 bit ISA extension slot. For a total of 16 IRQs in a typical ISA bus PC.

IRQ0 has the highest priority and IRQ7 the lowest priority. IRQ8-15 have "special" priority as will be explained. When IBM introduced the AT computer, they added IRQ8-15. In order to make AT (286) PCs backward compatible with 8 bit XT (8088) PCs and to "up" the priority of the new IRQ lines, they cascaded two interrupt controllers. This results in IRQ8-15 having the same priority as IRQ2. Priority means if two IRQs are active at the same time, the one with the higher priority is serviced first.

IMPORTANT: An IRQ can be assigned to only ONE active device at a time. If 2 devices share the same IRQ, this is called a CONFLICT. This means that when the IRQ line becomes active, the CPU does not know which device needs to "talk".

For example, if a modem used IRQ5 and a NIC used IRQ5. When the modem had some information that needed to be passed on to the CPU, it would set IRQ5 active. The CPU would not know whether to talk to the NIC or modem. The computer may hang, or nothing would happen.

*** IRQ conflicts are the NUMBER 1 source of PC problems! ***

Here is a table that is used as a rule of thumb (guideline) in selecting IRQs for PCs. The IRQs are listed in order of priority. (Note that not all IRQ lines go to the card slots)

IRQ	Function		Physical Line	   ISA Bus

IRQ0	System Timer			No

IRQ1	Keyboard Controller		No		-

IRQ2	Cascaded to IRQ8-15		No		-

IRQ8	Real-time clock			No		-

IRQ9	*-Available	(IRQ2)		Yes		8/16 bit

IRQ10	NIC				Yes		16 bit

IRQ11	SCSI adapter			Yes		16 bit

IRQ12	Motherboard mouse/available	Yes		16 bit

IRQ13	Math coprocessor		No		-

IRQ14	Primary IDE controller		Yes		16 bit

IRQ15	Secondary IDE controller	Yes		16 bit

IRQ3	Com2/Com4			Yes		8 bit

IRQ4	Com1/Com3			Yes		8 bit

IRQ5	Sound card/LPT2			Yes		8 bit

IRQ6	Floppy drive controller		Yes		8 bit

IRQ7	Parallel port LPT1		Yes		8 bit

*- IRQ9 appears as if it is IRQ2. Normally not used because it can cause interesting problems to appear. Is it really IRQ9 or is it the IRQ2 cascaded to IRQ9? Which do you set it to? What if you are using an 8 bit ISA modem in a 16 bit ISA slot? See what I mean...

The preceding table is a rule of thumb or guideline to selecting IRQs for your peripherals. For example if the PC does not use a SCSI adapter than IRQ11 is available for use for another NIC card or another device. Most autodetecting software or operating systems expect to see the IRQs assigned as above.

Standard COM Port Assignment

Note that COM1 (DB9 on the back of the PC) and COM3 share IRQ4. This is allowed as long as only one device is active at a time. This means that if you are running a mouse on COM1 then you cannot use COM3 for an internal modem. You will run into a conflict.

Some communication packages will allow you to do this but most will choke or cause flaky operation. A common sympton is if you move the mouse, you see garbage on your terminal program.COM2 (DB25 on the back of the PC) and COM4 have a similar problem except that most people don’t use COM2. It is usually safe to configure an internal modem to COM4. If COM2 is used, it is typically used for an external modem or a plotter. Usually, both are not active at the same time.

Port	IRQ	Function

COM1	4	Mouse

COM2	3	Not used or plotter or external modem

COM3	4	Not used (conflicts with mouse)

COM4	3	Not used or internal modem

DMA -Direct Memory Access

DMA stands for Direct Memory Access. This is a method that allows channels to be openned by the peripheral to read/write directly to memory without going through the CPU. This off-loads some of the work from the CPU to allow it to do more important tasks.

There are 8 DMA channels available in the PC: DMA0-7. They are divided into 8 bit channels and 16 bit channels based on the 8 bit ISA slot and 16 bit ISA slot. Here is a table that is used as a rule of thumb for selecting DMA channels:

DMA	Function		Physical Line	ISA Bus	     Channel Width

DMA0	Available			Yes	16 bit		8 bits

DMA1	Sound	card			Yes	8 bit		8 bits

DMA2	Floppy Disk controller		Yes	8 bit		8 bits

DMA3	ECP Parallel Port		Yes	8 bit		8 bits

DMA4	* - Not used			No	-		16 bit

DMA5	Sound	card			Yes	16 bit		16 bit

DMA6	SCSI				Yes	16 bit		16 bit

DMA7	Available			Yes	16 bit		16 bit

* - DMA4 is cascaded to the first 8 bit DMA controller and is not available.
Note: DMA0 is on the 16 bit ISA bus but is only 8 bits wide.

*** DMA conflicts are the NUMBER 2 source of PC problems! ***

Like IRQs, you are only allowed to assign one DMA channel to an active device at a time. Otherwise you will have a conflict appear and things will not work properly. You may have one DMA channel assigned to two devices ONLY if one device is active at a time.

Base Addresses

Base addresses are also called I/O ports, I/O addresses, I/O port addresses and base ports. They are memory locations that provide an interface between the operating system and the I/O device (peripheral). The peripheral communicates with the operating system through the base address. Each peripheral must have a UNIQUE base address. Standard Base Address assignments (h - hexadecimal):

Base Address		Function

060h + 064h		Keyboard controller

170h + 376h		Secondary IDE Hard-drive controller

1F0h + 3F6h		Primary IDE Hard-drive controller

220h			Sound Card

2A0h			Token Ring NIC

300h			Ethernet NIC

330h			SCSI adapter

3F2h			Floppy Drive Controller

3F8h			COM1

2F8h			COM2

3E8h			COM3

2E8h			COM4

378h			LPT1

278h			LPT2

*** Base Address conflicts are the NUMBER 3 source of PC problems! ***

Unfortunately, the above table is only a small part of the Base Addresses used. The base addresses used will depend on what has been installed on the PC.

- 38b. Legacy NICs

Before installing a legacy (polite way of saying old) NIC, a PC diagnostic program (Checkit or MSD) should be run to determine available: IRQs, Base Addresses and UMBs. After determining which IRQs, Base Addresses and UMBs are available, you would configure the NIC hopefully to the rule of thumb tables listed previously. In the case of the Upper Memory Block, you would also allocate that memory block using EMM386.EXE in config.sys (x800 block size).

Ex: device=c:\dos\emm386.exe x=C000-C800

This would ensure that EMM386.EXE does not allow any other program, Windows or TSR from using the same memory block thus avoiding memory conflicts. This is used to be a typical job interviewer's question: "What do you do to config.sys when installing a legacy network card?".

- 38c. NIC Diagnostic Tools

NICs come with software diagnostic tools that allow you to check the operation of the NIC. They are usually called Internal Diagnostics, Loopback Test and Echo Server Test. The Internal Diagnostics checks the internal hardware on the NIC card. It usually checks about a dozen or more different aspects of the network card up to the transmit/receive circuitry.

Internal Diagnostics

Loopback Test checks to see if the NIC can transmit and receive data properly. This test is usually applicable to 10Base2 (coax) only, as a BNC TEE with 2 terminations is required. There is no 10BaseT loopback test because you can't terminate at the NIC.

Loopback Test

Note: The first two diagnostic routines are performed not connected to the physical network. This prevents faulty NICs from disruptting normal network traffic. The last diagnostic routine is the Echo Server or Network test. Two NICs are required. A known working NIC acts as an Echo Server and the NIC under test is the Echo client. The echo client sends a packet to the echo server who echoes the packet back. This is tested on the network and can be used for any cabling type not just 10Base2 as per the example.

Echo Server Test

- 38d. Network Interface Card Drivers

Network Interface Card Drivers are the software interface between the Network Card Hardware/Firmware and the Network Operating System Data Link layer. The Network Card device driver is a device driver loaded in config.sys. The Network Card consists of Firmware and Hardware.

The Firmware is the program stored on the network card's ROM (BIOS) and configuration information stored in E2ROM. The configuration information would be the IRQ, Base Memory Address, Transceiver Type, etc.. for the Network Card. The Hardware would be the physical components: ICs, connectors, etc..

There are basically 3 types of Network Card Drivers:

- NDIS

- ODI

- Packet drivers

NDIS stands for Network Driver Interface Specification. NDIS drivers are used by Microsoft based Network Operating Systems such as Microsoft LAN Manager, Windows NT, Windows for WorkGroups and IBM's OS/2.

ODI stands for Open Datalink Interface. ODI drivers are used by Novell's Network Operating System and Apple.

Packet drivers use software interrupts to interface to the network card. Many non-commercial programs (shareware and freeware) use Crnywr packet driver interfaces.

The 3 Network Driver Types are not compatible with each other but most Network Operating Systems (Novell, WFWG, etc..) can use either NDIS or ODI. The NOS (Network Operating System) determines which type of Network Driver can be used. Regardless of the Network Driver type used, all have a network device driver loaded into memory during boot up and a network protocol bound to the network card.

The purpose of the Network Drivers is to decouple the network adapter's device driver from the higher layer protocols. The higher layer protocols can be IPX/SPX for Novell, Netbios for Microsoft, TCP/IP for Unix etc..

Traditional Network Card Device Driver Problems (pre-1990)

Traditionally (in the olden days - 1990), the Network Card Device Driver and NOS' Data Link layer were generated as 1 software program specific to the computer it was generated on.

As an example, with Novell 3.11 and earlier, a special program was run, called WSGen (workstation generator), which would generate a Workstation Shell. The Workstation Shell would be a software program running as a TSR which would be a combination of the Network Card Device Driver and Novell's IPX protocol. The Workstation Shell was specific to the computer that it was generated on and could not be used on another computer. This meant that every PC in a network would have its Workstation Shell recompiled with every new version of Novell! In a small network this would not be a problem, but in large networks (100+ PCs), this becomes a logistic nightmare!

Another problem emerged, the Workstation Shells directly controlled the network card and were specific to only one NOS. This meant that only one NOS protocol could be run, in Novell's case IPX. Interconnecting Networks became a major problem.

Still another problem arose in trying to run more than 1 network card in a computer (This is done typically in bridges, routers and servers). The Workstation Shells did not have the provision to allow the NOS Protocol to "bind" to more than one Network Card easily.

The NIDS and ODI Network Card Driver specifications were implemented to address the following specific areas:

- Provide a standard separate interface between the Network Card Device Driver and Data Link Layer.

- Allow more than one NOS Protocol to access the Network Card Device Driver.

- Allow the NOS to "bind" to more than one Network Card Device Driver.

NDIS Drivers

The NDIS (Network Driver Interface Specification) standard was developed jointly by Microsoft and 3Com for implementation in Microsoft's NOS and IBM OS/2.

The Microsoft implementation of NDIS modifies the config.sys file, autoexec.bat file and makes two important initialization files: SYSTEM.INI and PROTOCOL.INI.

Microsoft loads the IFSHLP.SYS file as a device driver in the CONFIG.SYS file. The IFSHLP.SYS is the installable file system helper file and contains the network redirector for the NDIS interface. The LASTDRIVE command in the config.sys file tells the network operating system the last available drive that can be used for mapping network drives.

The SYSTEM.INI file contains information similar to the following:

[network]

sizworkbuf=1498

filesharing=no

printsharing=no

autologon=yes

computername=E237-12

lanroot=C:\NET

username=EBLANCHARD

workgroup=WORKGROUP

reconnect=yes

dospophotkey=N

lmlogon=1

logondomain=T217PROJECT

preferredredir=full

autostart=full

maxconnections=8

[network drivers]

netcard=elnk3.dos

transport=ndishlp.sys,*netbeui

devdir=C:\NET

LoadRMDrivers=yes

[Password Lists]

*Shares=C:\NET\Shares.PWL

EBLANCHARD=C:\NET\EBLANCHA.PWL

The PROTOCOL.INI file contains protocol specific information and the virtual network card interface. A typical netbeui NDIS protocol.ini looks like:

[network.setup]

version=0x3110

netcard=ms$elnk3,1,MS$ELNK3,1

transport=ms$ndishlp,MS$NDISHLP

transport=ms$netbeui,MS$NETBEUI

lana0=ms$elnk3,1,ms$netbeui

lana1=ms$elnk3,1,ms$ndishlp

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NDISHLP

[MS$ELNK3]

DriverName=ELNK3$

IOADDRESS=0x300

[MS$NDISHLP]

DriverName=ndishlp$

BINDINGS=MS$ELNK3

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=12

BINDINGS=MS$ELNK3

LANABASE=0

ODI Drivers

The Open Datalink Interface (ODI) is a software standard developed by Novell and Apple Corporation to provide a layered approach to comply with the ISO Open System Interconnect (OSI) model for the Physical, Datalink and Network layers.

The Open Datalink Interface was developed to overcome several limitations on the previous network interface card driver software. Previous to the ODI standard, each workstation was required to "compile" its own workstation's IPX.COM shell using Novell's "WSGEN" program (workstation generation program). This resulted in a single program, that contained the network card driver, Datalink interface and Network layer protocol (IPX/SPX), commonly called the "workstation shell".

This approach limited the workstation to 1 network card and only 1 Network layer protocol. Multiple network cards and Network layer protocols were not allowed under "WSGEN".

The ODI standard broke the "workstation shell" into manageable parts that permits multiple network cards and protocols. For example: This means that 1 workstation/client can have an Ethernet 10BaseT card running IPX/SPX protocols (Novell) and a Farallon Localtalk card in it for running Appletalk (Macintosh).

The ODI standard compared to the OSI Model:

OSI	= Open System Interconnect		ODI	= Open Datalink Interface

SPX 	= Sequenced Packet Exchange		IPX	= Internetwork Packet Exhange

LSL 	= Link Suppport Layer			VLM 	= Virtual Loadable Modules

MLID 	= Multiple Link Interface Driver	MSM 	= Media Support Module

HSM 	= Hardware Support Module

Novell Lite (very old - defunct) is Novell's Peer to Peer Network Operating system. Peer to Peer Networks use DOS's File Allocation Table (FAT) and Novell Lite is no exception (Novell Netware has its own high performance disk operating system). Novell Lite follows Novell's Netware structure for the Network, Datalink and Physical layers and it is an excellent example of an ODI compliant NOS (Network Operating System). At the Transport layer it uses Peer to Peer Client and Server software instead of Novell's Netware Transport layer software - SPX (VLM).

A typical Novell client is loaded from the DOS prompt or from a STARTNET.BAT file:

SET NWLANGUAGE= ENGLISH

LSL.COM				Link Support Layer Software

3C509.COM			3C509 Network Interface Card Driver (MLID) ODI Compliant

IPXODI				IPX Network layer protocol driver

VLM				Loads client software

NET.CFG is the network configuration file used by the above files. It is a text file and contains the following basic section:

Link Driver 3C5X9		(NIC drivername)

INT 10			(IRQ #)

PORT 300		(Base memory address in hexadecimal)

FRAME Ethernet_802.2	(Frame type on Netware 3.12 & newer)

FRAME Ethernet_802.3	(Frame type on Netware 3.11 and older)

FRAME Ethernet_II		(Frame type used by UNIX)

FRAME Ethernet_SNAP 	(Frame type used by Appletalk)

NetWare DOS Requester

FIRST NETWORK DRIVE = F

USE DEFAULTS = OFF

VLM = CONN.VLM

VLM = IPXNCP.VLM

VLM = TRAN.VLM

VLM = SECURITY.VLM

;  VLM = NDS.VLM	(used for Netware 4.11 NDS services)

VLM = BIND.VLM

VLM = NWP.VLM

VLM = FIO.VLM

VLM = GENERAL.VLM

VLM = REDIR.VLM

VLM = PRINT.VLM

VLM = NETX.VLM

Packet Drivers

Packet drivers use software interrupts to identify the network cards to the data link layer. Packet drivers are free software drivers that were developed to address the problems of running multiple protocols over one network card. NDIS and ODI are proprietary schemes that have been developed by 3COM/Microsoft and Novell/Apple respectively to address this problem.

The Crynwr Software collection of packet drivers are available throughout the Internet and they are free to use unlike shareware and commercial products.

Advantages:

- Run multiple applications across the same board: TCP/IP, NetBIOS, Netware

- One board fits all, no buying different boards for different applications.

- No more reconfiguring and rebooting to change applications.

- Connect to a Novell File Server (or servers) and still run TCP/IP or PC-NFS or with the Novell systems remaining active and available for file serving and printing.

The Packet Driver acts as a fast smart secretary, bothering clients only when packets arrive specifically for them.

Software Interrupts

Software interrupts are interrupts generated by software unlike hardware interrupts that are physical lines that run to each device. Software interrupts that are available are 0x60 to 0x66. Table xx-1 lists the software interrupts and their assignments.

The packet drivers are assigned software interrupts to the network interface card during the bootup process usually in the autoexec.bat file. For a 3c503 card the autoexec.bat file would have this line:

3c503 0x60 5 0x300

where:

- 3c509 calls up the packet driver 3c509.com

- 0x60 is the software interrupt assigned to the NIC

- 5 is the hardware interrupt of the NIC

- 0x300 is the I/O address of the NIC

Any network traffic received or transmitted from the NIC would be addressed by the software interrupt 0x60. Complete documentation is available from the Crynwr collection under the files. Important files to read are install.doc and packet.doc.

Software Interrupts Assignments

60 -- -- reserved for user interrupt

61 -- -- reserved for user interrupt

62 -- -- reserved for user interrupt

63 -- -- reserved for user interrupt

64 -- -- reserved for user interrupt

65 -- -- reserved for user interrupt

66 -- -- reserved for user interrupt

67 -- -- LIM EMS

68 01 -- APPC/PC

69 -- -- unused

6A -- -- unused

6B -- -- unused

6C -- -- DOS 3.2 Realtime Clock update

6D -- -- VGA - internal

6E -- -- unused

6F -- -- Novell NetWare

70 -- -- IRQ8 - AT/XT286/PS50+ - REAL-TIME CLOCK

71 -- -- IRQ9 - AT/XT286/PS50+ - LAN ADAPTER 1

72 -- -- IRQ10 - AT/XT286/PS50+ - RESERVED

73 -- -- IRQ11 - AT/XT286/PS50+ - RESERVED

74 -- -- IRQ12 - PS50+ - MOUSE INTERRUPT

75 -- -- IRQ13 - AT/XT286/PS50+ - 80287 ERROR

76 -- -- IRQ14 - AT/XT286/PS50+ - FIXED DISK

77 -- -- IRQ15 - AT/XT286/PS50+ - RESERVED

78 -- -- not used

79 -- -- not used

7A -- -- Novell NetWare - LOW-LEVEL API

7A -- -- AutoCAD Device Interface

7B -- -- not used

7C -- -- not used

7D -- -- not used

7E -- -- not used

7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE

80 -- -- reserved for BASIC

- 39. Repeaters

Repeaters are physical hardware devices that have a primary function to regenerate the electrical signal by:

- Reshaping the waveform

- Amplifying the waveform

- Retiming the signal

- 39a. Purpose of a Repeater

The purpose of a repeater is to extend the LAN Segment beyond its physical limits as defined by the Physical Layer's Standards (e.g. Ethernet is 500m for 10Base5). A LAN Segment is a logical path such as the logical bus used by all 802.3 Ethernet types. A LAN Segment is given an identification number called a Segment Number or Network Number to differentiate it from other segments.

Typically, repeaters are used to connect 2 physically close buildings together that are too far apart to just extend the segment. Can be used to connect floors of a building together that would surpass the maximum allowable segment length. Note: for large extensions as in the above example, 2 Repeaters are required. For shorter extensions, only 1 Repeater may be required.

- 39b. Repeater's OSI Operating Layer

Repeaters operate at the OSI Model Physical Layer.

- 39c. Repeater's Segment to Segment Characteristics

Repeaters do not "de-segment" a network. All traffic that appears on one side of the repeater appears on both sides. Repeaters handle only the electrical and physical characteristics of the signal.

Repeaters work only on the same type of Physical Layer: Ethernet to Ethernet or Token Ring to Token Ring. They can connect 10Base5 to 10BaseT because they both use the same 802.3 MAC layer.

You can run into problems connecting 1Base5 to 10BaseT with the transfer rate (1 Mbps vs. 10 Mbps). A repeater cannot connect Token Ring to Ethernet because the Physical Layer is different for each.

- 39d. Repeater Addressing: MAC Layer and Network Segment

The MAC Layer Address is used to identify the Network Card to the Network. The Repeater is transparent to both sides of the segment and both sides can "see" all the Mac Addresses regardless on which side they are on. This means that any network traffic on Floor 1 will appear on Floor 5 and vice versa.

Nodes A & B could be furiously exchanging files and this network traffic would also appear on Floor 1. Repeaters provide no isolation between segments, there is only one collision domain.

Because Repeaters provide no isolation between segments and the repeater is transparent to both sides of the segment, both sides of the repeater appear as 1 long segment. The Network Number or Segment Number is the same on both sides of the Repeater.

When using repeaters, make sure that the overall propagation delay does not exceed the Physical Layer Standard being used. Repeaters will add a propagation delay to the signal that is being repeated also. Check that rules such as the 5-4-3 Rule for IEEE 802.3 are not broken or for XNS Ethernet that a maximum of only 2 Repeaters are between any 2 nodes.

You are allowed to parallel Segments using multiport repeaters. Multiport repeaters have several inputs/outputs. Notice that all floors have the same Segment Number. You are not allowed to create a loop between two segments by using two repeaters.

Fibre Optic Repeaters join 2 segments together with a fibre optic link. The Transfer rate is not changed through the fibre. The advantage is noise immunity and longer distances. Segments can be joined up to 3000m apart and still be within the propagation delay specification for the Physical Layer. Two fibre optic repeaters are required: one at each end of the fibre.

Fibre Optic Repeater

- 40. Hubs

Hubs are also called Multiport Repeaters or Concentrators. They are physical hardware devices.

Some Hubs are basic hubs with minimum intelligence - no microprocessors. Intelligent Hubs can perform basic diagnostics and test the nodes to see if they are operating correctly. If they are not, the Smart Hubs or Intelligent Hubs will remove the node from the network. Some Smart Hubs can be polled and managed remotely.

- 40a. Purpose of Hubs

Hubs are used to provide a Physical Star Topology. The Logical Topology is dependant on the Medium Access Control Protocol. At the center of the star is the Hub with the network nodes on the tips of the star.

Star Topology

The Hub is installed in a central wiring closet with all the cables extending to the network nodes. The advantage of having a central wiring location is that it is easier to maintain and troubleshoot large networks. All of the network cables come to the central hub, it is especially easy to detect and fix cable problems. You can easily move a workstation in a star topology by changing the connection to the hub at the central wiring closet.

The disadvantages to a star topology are:

- failure of the Hub can disable a major section of the network

- The Star Topology requires more cabling than does the ring or the bus topology because all stations must be connected to the hub, not to the next station.

- 40b. Hub's OSI Operating Layer

Hubs are multiport repeaters and as such obey the same rules as repeaters (See previous section OSI Operating Layer). They operate at the OSI Model Physical Layer.

- 40c. Hub's Segment to Segment Characteristics

To understand the Ethernet segment to segment characteristics of a hub, the first thing to do with Ethernet Hubs is to determine how they operate. Logically, they appear as a Bus Topology and physically as a Star Topology. Looking inside an Ethernet Hub, we can see that it consists of a electronic printed circuit board which doesn't tell us much. If we form a functional drawing, we can clearly see how the Physical and Star Topology appears:

Understanding that inside the Hub is only more repeaters, we can draw the conclusion that all connections attached to a Hub are on the same Segment and have the same Segment Number. It is considered one repeater from any port to any port even though it is indicated as a path of 2 repeaters.

The 5-4-3 Rule for Ethernet Hubs:

Cascaded Hub Network

Cascading Hubs means to connect the Hubs together through the RJ45 ports. One Master Hub (Level 1) is connected to many Level 2 (Slave) Hubs who are masters to Level 3 (slave) Hubs in a hierarchical tree or clustered star. The maximum number of stations in a Cascaded Hub Network is limited to 128.

Backbone Networks

In a Backbone Network, there is no Master Hub. The Level 1 Hubs are connected through their AUI port to a Coax Backbone. For Thin Coax, up to 30 Hubs can be connected together. For Thick Coax, up to 100 Hubs can be connected to the backbone. The Backbone is considered to be a populated segment.

Level 2 Hubs are allowed to be connected to the Level 1 Hubs' 10BaseT ports. This connection between the 2 Hubs is considered an unpopulated segment or link segment. Up to 1024 stations or nodes can be attached to the Level 2 Hubs' 10BaseT ports.

All stations and segments would appear as 1 Logical segment with 1 Network Number. In the real world, you would never attach 1024 stations to 1 segment, the resulting traffic would slow the network to a crawl.

- 40d. Hub's Addressing

Again, because a Hub is just many repeaters in the same box, any network traffic between nodes is heard over the complete network. As far as the stations are concerned, they are connected on 1 long logical bus (wire).

- 40e. Half-Duplex & Full-Duplex Ethernet Hubs

Normal Ethernet operation is Half-Duplex: only 1 station or node talking at a time. The stations take turns talking on the bus (CSMA/CD -bus arbitration).

Full-Duplex Ethernet Hubs are Hubs which allow 2 way communication between Hubs thus doubling the available bandwidth from 10 Mbps to 20 Mbps. Full duplex Hubs are proprietary products and normally only work within their own manufacturer's line.

If A wanted to talk to C, a direct 10 Mbps line would be connected through the 2 switching hubs. Simultaneously, if D wanted to talk to B, another direct 10 Mbps line in the opposite direction would be connected through the 2 switching Hubs thus doubling the available bandwidth to 20 Mbps.

There are no official standards for Full-Duplex Ethernet just proprietary ones.

- 40f. Switching Hubs

Switching hubs are hubs that will directly switch ports to each other. They are similar to full duplex hubs except that they allow dedicated 10 Mbps channels between ports.

If A wanted to communicate with B, a dedicated 10 Mbps connection would be established between the two. If C wanted to communicate with D, another dedicated 10 Mbps connection would be established.

- 41. Bridges

Bridges are both hardware and software devices. They can be standalone devices - separate boxes specifically designed for bridging applications, or they can be dedicated PCs with 2 NICs and bridging software. Most servers software will automatically act as a bridge when a second NIC card is installed.

- 41a. Bridge OSI Operating Layer

Bridges operate on the OSI Model Data Link Layer. They look at the MAC addresses for Ethernet and Token Ring to determine whether or not to forward or ignore a packet.

- 41b. Purpose of a Bridge

The purposes of a Bridge are:

- Isolates networks by MAC addresses

- Manages network traffic by filtering packets

- Translate from one protocol to another

Isolates networks by MAC addresses

For example, you have 1 segment called Segment 100 with 50 users in several departments using this network segment. The Engineering Dept. is CAD (Computer Aided Design) oriented and the Accounting Dept. is into heavy number crunching: year end reports, month end statements etc..

On this network, any traffic between Client A, B or C and the Accounting File Server in the Accounting Dept. will be heard across the Segment 100. Likewise any traffic between the Engineering Dept.'s Clients G, H or I to the CAD File Server will be heard throughout the Network Segment. The result is that the "Other" Departments access to the Generic File Server is incredibly slow because of the unnecessary traffic occurring due to other departments: Engineering & Accounting.

Note: The designations A, B, and C are used instead of MAC addresses for brevity. The actual MAC addresses would be hexadecimal numbers such as 08-00-EF-45-DC-01.

The solution is to use a Bridge to isolate the Accounting Dept. and another bridge to isolate the Engineering Department. The Bridges will only allow packets to pass through that are not on the local segment. The bridge will first check its "routing" table to see if the packet is on the local segment, if it is, it will ignore the packet and not forward it to the remote segment. If Client A sent a packet to the Accounting File Server, Bridge #1 will check its routing table, to see if the Accounting File Server is on the local port. If it is on the local port, Bridge #1 will not forward the packet to the other segments.

If Client A sent a packet to the Generic File Server, again Bridge #1 will check its routing table to see if the Generic File Server is on the local port. If it is not, then Bridge #1 will forward the packet to the remote port.

Note: The terms local and remote ports are abitrarily chosen to distinguish between the two network ports available on a bridge.

In this manner the network is segmented and the local department traffic is isolated from the rest of the network. Overall network bandwidth increases because the Accounting Dept. does not have to fight with the Engineering Dept. for access to the segment. Each segment has reduced the amount of traffic on it and the result is faster access. Each department still has complete access to the other segments but only when required.

Manages network traffic by filtering packets

Bridges listen to the network traffic and build an image of the network on each side of the bridge. This image of the network indicates the location of each node and the bridge's port that accesses it. With this information, a bridge can make a decision whether to forward the packet across the bridge if the destination address is not on the same port or it can decide to not forward the packet if the destination is on the same port.

This process of deciding whether or not to forward a packet is termed filtering packets. Network traffic is managed by deciding which packets can pass through the bridge. The bridge filters packets.

Translate from one protocol to another

The MAC layer also contains the bus arbitration method used by the network. This can be CSMA/CD as used in Ethernet or Token Passing as used in Token Ring. Bridges are aware of the Bus Arbitration and special translation bridges can be used to translate between Ethernet and Token Ring.

- 41c. Bridge Segment to Segment Characteristics

Bridges physically separate a network segments by managing the traffic based on the MAC address.

Bridges are store and forward devices. They receive a packet on the local segment, store it and wait for the remote segment's to be clear before forwarding the packet.

There are 2 physical types of bridges: Local and Remote Bridges.

Local Bridges are used as in the previous examples where the network is being locally (talking physical location now) segmented. The 2 segments are physically close together: same building, same floor, etc... Only 1 bridge is required.

Remote Bridges are used in pairs and where the network is remotely segmented (again talking physical locations). The 2 segments are physically far apart: different buildings, different floors, etc... 2 x Half Bridges are required: one at each segment. The Remote bridges are 1/2 of a normal bridge and may use several different communications media inbetween.

- 41d. Bridge Methodologies

There are 3 primary bridging methodologies used by bridges for connecting local area networks:

- Transparent bridges

- Spanning Tree Protocol

- Source Routing

Transparent Bridges were originally developed to support the connection of Ethernet networks. The spanning tree protocol was developed to improve upon transparent bridging. Source Routing Bridges are used by Token Ring. Source routing bridges require a solid understanding of Token Ring concepts and as such will be covered under the section discussing Token Ring.

Transparent Bridges

Transparent Bridges examine the MAC address of the frames to determine whether the packet is on the local Segment or on the distant Segment. Early bridges required the system administrator to manually build the routing table to tell a bridge which addresses were on which side of the bridge. Manually building a routing table is called fixed or static routing. Modern bridges are self-learning, they listen to the network frame source addresses to determine which side of the bridge the node is on and build a routing table that way.

The following network will be used as an example of a self-learning transparent bridge's routing table construction.

As frames flow on Bridge #1's local port, Bridge #1 examines the source address of each frame. Eventually after all nodes on the local port, have become active, Bridge #1 associates their address as being on the local port. Any frames with a destination address other than the nodes on the local port are forwarded to the remote port. As far as Bridge #1 is concerned, nodes on Bridge #2's local port appear as if they were on Bridge #1's remote port.

Bridge #2 builds its routing table in a similar manner to Bridge #1. Note the differences.

Advantages to Transparent Bridges

- Self learning: requires no manual configuration, considered plug and work.

- Independent of higher level protocols (TCP/IP, IPX/SPX, Netbeui, etc..)

Disadvantages of Transparent Bridges

- Can only work with 1 path between segments: Loops are not allowed. A loop would confuse the bridge as to which side of the bridge a node was really on: local or remote?

Transparent Bridges are not acceptable for use on MANs or WANs, as many paths can be taken to reach a destination. In the above example, it is simple to determine that a loop occurs but in a large corporate network with several hundred bridges, it may be next to impossible to determine. As such, Bridges are most commonly used in LAN to LAN connectivity and not in MANs or WANs.

Spanning Tree Protocol - IEEE 802.1D

The Spanning Tree Protocol was developed to address the problems of loops in Transparent Bridging. The IEEE 802.1D (Institute of Electrical and Electronic Engineers) committee formed the Spanning Tree Protocol.

The Spanning Tree Protocol (STP) converts a loop into a tree topology by disabling a bridge link. This action ensures there is a unique path from any node to every other node in a MAN or WAN. Disabled bridges are kept in stand-by mode of operation until a network failure occurs. At that time, the Spanning Tree Protocol will attempt to construct a new tree using any of the previously disabled links.

The Spanning Tree Protocol is a Bridge to Bridge communication where all bridges cooperate to form the overall bridge topology. The Spanning Tree algorithm is dynamic and periodically checks every 1 to 4 seconds to see if the bridge topology has changed.

Bridge #3 & #5 are stand-by bridges and have their links disabled. This results in only 1 path to each network segment.

Each bridge is assigned an arbitrary number to assign priority to the bridge in the internetwork. The number is concatenated with the bridge MAC address. The MAC address is used as a tie breaker mechanism if 2 bridges have the same priority. The lower the assigned number the higher the bridge priority.

During initial power-up, a Bridge Protocol Data Unit (BPDU) is flooded out each network port of the bridge. The BPDU contains the current spanning tree root, the distance to the root (measured in hops through other bridges), the bridge address information and the age of the information in the BPDU. Bridge priorities are usually manually controlled so as to configure the traffic flow over the internetwork over a preferred path.

Problems can arise where the Spanning Tree Algorithm may select a path from Los Angeles to New York City and back to San Francisco rather than the preferred route of Los Angeles to San Francisco.

- 41e. Reasons to use a Bridge

There are four basic reasons to use a bridge:

- Security: Stops networks from forwarding sensitive data

- Bandwidth: Reduce traffic by segmentation

- Reliability: If 1 segment goes down, it does not take down the complete LAN

- Translation: Translate different Data Link protocols such as Token Ring to Ethernet

- 41f. Bridge Addressing

Bridges work at the Data Link Layer and recognize the MAC addresses. Spanning Tree Protocol adds a Bridge Protocol Data Unit (BPDU) for Bridge to Bridge communications. Source Route Bridges and Token Ring provide special Data Link layer communication and will be discussed later.

- 41g. Collapsed Backbones

Collapsed Backbones take the network backbone and electronically collapse it into a high speed electronic card cage. Usually Collapsed Backbones operate at 100 Mbps. The card cage holds plug-in cards for repeaters, hubs, bridges, routers, brouters and gateways.

Software is provided to remotely configure all plug-in cards using SNMP. SNMP is a network management protocol that stands for Simple Network Management Protocol. It is a standard for intelligent network devices to communicate their configuration to administrators operating from remote workstations. The workstations can be thousands of miles away!

- 42. Routers

Routers are hardware and software devices. They can be cards that plug into a collapsed backbone, stand-alone devices (rack mount or desktop) or software that would run on a file server with 2 NICs.

- 42a. Purpose of Routers

The purpose of a router is to connect nodes across an internetwork regardless of the Physical Layer and Data Link Layer protocol used. Routers are hardware and topology independent. Routers are not aware of the type of medium or frame used (Ethernet, Token Ring, FDDI, X.25, etc...). Routers are aware of the Network Layer protocol used: Novell's IPX, Unix's IP, XNS, Apples DDP, etc..

- 42b. Router OSI Operating Layer

Routers operate on the OSI Model's Network Layer. The internetwork must use the same Network Layer protocol. Routers allow the transportation of the Network Layer PDU through the internetwork even though the Physical and Data Link Frame size and addressing scheme may change.

- 42c. Router Segment to Segment Characteristics

Routers that only know Novell IPX (Internetwork Packet Exchange) will not forward Unix's IP (Internetwork Packet) PDUs and vice versa. Routers only see the Network Layer protocol that they have been configured for. This means that a network can have multiple protocols running on it: SPX/IPX, TCP/IP, Appletalk, XNS, etc..

In the following network, Router #3 is a Novell SPX/IPX router, it only sees the Network Layer protocol IPX. This means that any TCP/IP PDUs will not pass through, the router does not recognize the PDUs and doesn't know what to do with them.

Routers #1 & #2 are TCP/IP routers, they recognize only IP protocols. This keeps SPX/IPX traffic off of "Segment 300". This is in quotations because TCP/IP has a different network numbering scheme than IPX.

Important Point: Routers allow network traffic to be isolated or segmented based on the Network Layer Protocol. This provides a functional segmentation of the network.

Routers that only can see 1 protocol are called Protocol Dependent Routers. Routers that can see many different protocols (2 or more) are called Multiprotocol Routers.

- 42d. Router Addressing

Routers combine the Network Number and the Node Address to make Source and Destination addresses in routing Network Layer PDUs across an network. Routers have to know the name of the segment that they are on and the segment name or number where the PDU is going to. They also have to know the Node Address: MAC Address for Novell and the IP address for TCP/IP.

For Novell's SPX/IPX (Sequential Packet eXchange/Internetwork Packet eXchange), the Network Layer PDUs address is composed of the Network Address (32 bit number) and the Host address (48 bit - MAC address).

- 42e. Routing Protocols

Routing Protocols are a "sub-protocol" of the Network Layer Protocol that deal specifically with routing of packets from the source to the destination across an internetwork. Examples of Routing Protocols are: RIP, IGRP and OSPF.

- 42f. RIP - Routing Information Protocol

RIP was one of the first routing protocols to gain widespread acceptance. It is described in RFC1058 which is an Internet standard. RFC stands for request for comment and the RFC1058 is the 1,058 RFC standard published. Commercial NOS such as Novell, Apple, Banyan Vines and 3Com, use RIP as the base routing algorithm for their respective protocol suites.

RIP is a distance vector algorithm. Routers maintain a detailed view of locally attached network segments and a partial view of the remainder of the routing table. The routers contain information on the number of hop counts to each segment. A hop is considered to be one transverse through a router. Pass through a router and the Hop count increases by 1.

The routers are updated every 30 seconds, each router sending out a RIP broadcast. This advertisement process is what enables RIP routing to be dynamic. Dynamic routers can change routing tables on the fly as the network configuration changes. By using the Hop Count information from their routing tables, routers can select the shortest path - the least number of hops to the destination.

Apple uses RTMP (routing table maintenance protocol) which adds a route status indicator: good, bad or suspect depending on the age of the route information.

Novell adds ticks to the RIP algorithm, Ticks are dynamically assigned values that represent the delay associated with a given route. Each tick is considered 1/18 of a second.

LAN segments are typically assigned a value of 1 tick, a T1 link may have a value of 5 to 6 ticks and a 56 Kbps line may have a value of 20 ticks. Larger number of ticks indicate a slower routing path.

Three commonest problems that can occur with RIP are:

- Routing loops: the router indicates that the shortest path is back the way the packet came from.

- Slow Route Convergence: routers have delay timers that start counting after the RIP advertising packet is broadcasted. This gives the routers time to receive and formulate a proper routing table from the other routers. If the delay timer is too short, the routing table can be implemented with incomplete data causing routing loops

- Hop Count Exceeded: the maximum number of hop counts is 15 for RIP. A hop count of 15 is classified as unreachable which makes RIP unsuitable for large networks where hop counts of 15 and above are normal.

- 42g. EGRP - Exterior Gateway Routing Protocol

EGRP was created to solve many of the problems with RIP and has become the default routing protocol across the Internet. EGRP is an enhanced distance vectoring protocol, it uses up to 5 metrics (conditions) to determine the best route:

- Bandwidth

- Hop Count (Delay) - maximum of 255

- Maximum Packet size

- Reliability

- Traffic (Load)

These routing metrics are much more realistic indicators of the best routes compared to simple hop counts.

- 42h. OSPF - Open Shortest Path First

OSPF is a link state premise, this means that it has several states of routers linked together in a hierarchical routing model:

The top of the root is the Autonomous Router, it connects to other autonomous systems (the Internet). The next is the Backbone Routers, which is the highest area in the OSPF system. Border routers are attached to multiple areas and run multiple copies of the routing algorithm. Last is internal routers which run a single routing database for one area.

Basically, by dividing the network into a routing hierarchy, substantial reduction of routing update traffic and faster route convergence results on a local basis. Each level has a smaller routing table and less to update.

- 43. Brouters (Bridge/Routers)

Brouters are protocol dependant devices. When a brouter receives a frame to be forwarded to the remote segment, it checks to see if it recognizes the Network layer protocol. If the Brouter does, it acts like a router and finds the shortest path. If it doesn't recognize the Network layer protocol, it acts like a bridge and forwards the frame to the next segment.

The key advantage to Brouters is the ability to act as both a bridge and a router. It can replace separate bridges and routers, saving money. This is, of course, provided that the Brouter can accomplish both functions satisfactorily.

- 44. Gateways

One definition of a Gateway is the Hardware/Software device that is used to interconnect LANs & WANs with mainframe computers such as DECnet and IBM's SNA.

Often the router that is used to connect a LAN to the Internet will be called a gateway. It will have added capability to direct and filter higher layer protocols (layer 4 and up) to specific devices such as web servers, ftp servers and e-mail servers.

- 44a. Gateway's OSI Operating Layer

A Gateway operates at the Transport Layer and above. Typically translating each source layer protocol into the appropriate destination layer protocol. A mainframe gateway may translate all OSI Model layers. For example, IBM's SNA (System Network Architecture) does not readily conform to the OSI Model and requires a gateway to tranlate between the two architectures.

- 44b. Gateway Segment to Segment Characteristics

There can be major differences between "local" and "distance" segments. As can be seen from the above diagram, the 2 Networks appear as if they are from other planets. Mainframes are based on a central number crunching CPU with terminals connected. All information displayed on the terminals is controlled by the central CPU.

LANs consist of distributed CPUs that share data and files. This leads to a unique problem in connecting the two architectures that requires a gateway.

- 44c. Gateway Addressing

The gateway addressing depends on which OSI layers are translated. It could be all layers!

# 45. Token Ring

### STOP - You are now leaving Ethernet IEEE 802.3 
Please fasten your seatbelts and place your trays in the fully upright position

Token Ring is a token passing bus arbitration topology for the Physical and Data Link Layers. It is a logical ring and a physical star topology.

Token Ring uses a token passing scheme for bus arbitration. A special packet is passed around the ring called a token. When a node requires access to the ring, the node claims the token and then passes its information packet around the ring. All nodes read the destination address and if it is not addressed for them, the information packet is then passed on to the next node. When the destination node reads the packet, it marks it as read and passes it on to the next node. When the information packet completely circulates the ring and arrives back at the source node, the source node releases the token back on to the ring.

Token Rings are not usually drawn as the above drawing indicates: a separate line between each node. They are usually represented as understood that separate paths exist between nodes and are drawn as in the figure to the right.

## 45a. IBM Token Ring

Token Ring was originally developed by IBM for their PC LAN networks. It started out in 1969 as the Newhall Network, named after the originator of the token ring concept. IBM's Token Ring is the basis for the IEEE 802.5 standard Token Ring. They are very similar and have minor differences which we will cover.

- 45b. IEEE 802.4 Token Bus

An industrial version of Token Ring is standardized under IEEE 802.4 Token Bus. It is used in manufacturing process equipment for plant operation. It is used in automobile plants for computerized assembly. It uses a Logical Ring and a Physical Bus topology.

- 45c. IEEE 802.5 Token Ring

IEEE 802.5 Token Ring standard is based on the IBM Token Ring network. Token Ring has been used mainly in large corporations and was considered in the past to be the only way to handle data communications in large networks (1000+) nodes.

Token Ring equipment is more expensive than Ethernet and is one of the reasons that Ethernet is more popular. The other reason is that Token Ring is much more complex bus arbitration method than CSMA/CD and few network personnel understand the full capabilities of Token Ring.

- 45d. IEEE 802.5 Bus Arbitration

Token Ring is a token passing bus arbitration. A token is circulated on the ring. If a node on the ring needs to access the ring (transfer information), it claims the token.

The token is a special packet, that is circulated around the ring. It is read from one node than passed to the next node until it arrives at a node that needs to access the ring (transfer information/data). When a node receives the token, the node is allowed to send out its information packet.

Example: The token is circulating the ring, Node B needs to send some data to Node G. Node B waits for the token to come by. There is only one token allowed on the ring. When it receives the token, it can then send out its information packet. Node G is the destination address.

Node C receives the packet, reads the destination address and passes it on to the next node. Node D, E & F do likewise.

When the packet arrives at node G, node G reads the destination address and reads the information. Node G marks the information packet as read and passes it on.

Note: the Source and Destination addresses remain unchanged after passing through Node G. Node B is still the Source address and Node G is still the Destination address.

The packet continues around the ring, until it reaches the source address Node B. Node B checks to make sure that the packet has been read - this indicates that Node G is actually present. The information packet is erased. Node B then releases the token onto the ring.

Information marked READ is passed through the ring back to the Source - Node B

The information packet is called the Token Frame. The token is called the Token (sometimes referred to as the free token). This can be confusing. Remember, when we talk about a frame, we are talking about data/information. When talking about a token, we are talking about bus arbitration and permission to use the bus.

- 45e. 4 / 16 Mbps Transfer Rate

The transfer rate for Token Ring is 4 Mbps for older systems or 16 Mbps for newer systems (1990 and newer). There are several products in development and available that will increase Token Ring's transfer rate using Switching Hubs and even faster transfer rates over existing cabling.

NOTE: 		16 Mbps NIC cards will operate at both 16 and 4 Mbps speeds.

4 Mbps NIC cards will only operate at 4 Mbps.

To identify the speed of an unknown card, exam the integrated circuits on the card. There is only 1 chipset that implements IEEE 802.5's 4 Mbps standard for Token Ring. It was developed jointly by Texas Instruments and IBM. It is a 5 chip set and consists of:

- TMS38051 Ring Interface Transceiver

- TMS38052 Ring Interface Controller

- TMS38010 Communications Protocol Processor

- TMS38021 Protocol Handler for 802.5 Functions

- TMS38030 DMA Controller between NIC and PC Bus

4 Mbps Token Ring NICs are usually full length expansion cards.

16 Mbps NICs have typically 1 large IC with 132 pins and several small ones. They are typically 1/2 length cards. The IC number is TMS380C16 for the Texas Instrument version or TROPIC for the IBM version or DP8025 for the National version.

- 45f. IEEE 802.5 Topology

Token Ring is a Logical Ring / Physical Star topology. So far we've been only discussing the logical portion. Nodes on the network are physically connected via their NICs to a central concentrator or hub. The concentrator is called a MAU or MSAU both stand for MultiStation Access Unit. To avoid confusion with Ethernet MAUs, we will refer to a Token Ring hub as a MSAU (pronounced "M sow") or as a concentrator.

- 45g. MSAUs

A Token Ring MSAU has connections to connect to the nodes and it also has special connections called Ring In and Ring Out to connect to other MSAUs.

The Ring In connector is abbreviated RI and the Ring Out connector is abbreviated RO. The nodes (PCs) would be attached to connectors 1 to 8 for this 8 node MSAU.

The MSAU logical connection would be drawn as indicated below:

The connection from the Node to the MSAU is called the Lobe. The connection to the ring is via the Ring In and Ring Out connectors.

MSAUs are passive devices, there isn't any "intelligence" built-in. MSAUs come in 2 flavours:

- Unpowered - The unpowered MSAUs receive their power through the NIC cards.

- Powered. - The powered MSAUs plug in the wall outlet and have their own power supply built-in.

Token Ring connectors

The wiring between the NIC card and MSAU consists of 2 pairs of wires:

- Receive Pair - This pair receives packets from its upstream neighbour

- Transmit Pair - This pair transmits packets to its downstream neighbour

There are 4 types of connectors used with Token Ring:

- RJ11 connectors are used with older 4 Mbps systems

- RJ45 connectors with both 4/16Mbps systems

- Hermaphroditic connectors with IBM Cat 1 cabling

- The DB9 is used to connect the NIC card to the Hermaphroditic cable.

Signal Lead	Hermaphroditic	RJ45	RJ11	DB9

Tx+		Orange (O)	3	2	9

Rx+		Red (R)		4	3	1

Rx-		Green (G)	5	4	6

Tx-		Black (B)	6	5	5

UTP wiring pinouts:

Note: The receive pair (Rx) is the center pair of wires. The transmit pair (Tx) the outside pair.

MSAU Relay

When a Token Ring NIC is first turned on, it goes through a process called Ring Insertion. It checks the Lobe to see if the wiring is okay and then applies a DC voltage on the Transmit pair of wires. The DC voltage is often called phantom power.

This voltage energizes a relay in the MSAU and attaches the Lobe to the ring. If you disconnect a cable at the MSAU, the relay will de-energize and automatically disconnect the lobe from the ring. You can actually hear the relays clicking in and out.

Ring In/ Ring Out

On a MSAU are 2 connectors called Ring In (RI) and Ring Out (RO). These are used for connecting MSAUs together. Two pairs of wires are run between MSAUs to connect them together, one pair is used for the Main Ring and one is used for the Backup Ring.

The following figure indicates the Main Ring and the Backup Ring. Notice that the Backup Ring runs in parallel with the Main Ring and is not normally used. Also notice that the direction of data flow on the Backup Ring is opposite to the Main Ring.

Wrapping

If the Main Ring fails due to cable faults or MSAU problems, the Main Ring can be wrapped to the Backup Ring. Wrapping is a term that is used to indicate that the Backup Ring is being used in addition to the Main Ring.

The Backup Ring is connected to the Main Ring. The Main Ring or a portion of the Main Ring is still being used. Wrapping is only associated with the Ring In and Ring Out connectors on the MSAUs.

Main Ring wrapped to Backup ring

This can be done either of 3 ways:

- Passive Hermaphroditic Style MSAUs - remove the suspected RI or RO Hermaphroditic connector. The connector will automatically short and wrap the Main Ring to the Backup Ring

- Passive RJ11 & RJ45 Style MSAUs - Manually switch the suspected RI or RO connector with the available switches

- Active MSAUs - They will automatically wrap if there is a problem.

Physical Star/ Logical Ring

With an understanding of how an MSAU works, it is easier to see how we get a Logical Ring for Token Ring. The Physical Star results from the Lobe cabling fanning out to the Nodes.

- 45h. IEEE 802.5 and the OSI Model

| 45h. Token Ring OSI Model (cont'd) | Next |
| --- | --- |

- 45i. Token Ring Cabling

There are 2 basic types of Token Ring cabling:

- Shielded Twisted Pair (STP)

- Unshielded Twisted Pair (UTP)

Shielded Twisted Pair

STP or Shielded Twisted Pair is balanced shielded twisted pair cable, 150 +/-15 ohms impedance. It is used typically with the Hermaphroditic connectors. It is referred to as IBM Type 1, 1A, 2 or 6 cabling. It is the most expensive cabling to use. The cable is expensive and the connectors are expensive.

Max Lobe Distance	# Stations per ring	Concentrator

4 Mbps		1000 ft/ 305 m	 		250		Passive

16 Mbps		550 ft/ 168 m			250		Passive

4/16 Mbps	1000 ft/ 305 m			250		Active

Unshielded Twisted Pair - Type 3

UTP or Unshielded Twisted Pair is used with phone style connectors: RJ11 or RJ45. It is 100 +/-15 ohms impedance typically 22 to 24 AWG wire. It is categorized into the following categories:

Max Lobe Distance	# Stations per ring	Concentrator

4 Mbps		328 ft/ 100 m	 	72/54		Passive (old/new)

16 Mbps		328 ft/ 100 m		250		Passive

4 Mbps		328 ft/ 100 m		54		Active

16 Mbps		328 ft/ 100 m		250		Active

IBM Cabling System

Type 1

Two shielded, solid wire, twisted pairs, 22 AWG. Available for plenum or nonplenum interior use and underground or aerial exterior use. Use of Type 1 permits transmission at 16 Mbps and a maximum of 260 stations on the network.

Note: Plenum is heating ducts and air returns. To be qualified for plenum installation means that it must meet certain standards for releasing hazardous fumes and temperature ratings.

Type 2

Two shielded, solid wire twisted pairs, 22 AWG plus four twisted pairs of solid 26 AWG wires added between the shield and the insulating cable sheath. Type 2 supports 16 Mbps transmission.

Type 3 (RJ11 and RJ45)

Unshielded, telephone grade (22 or 24 AWG) twisted pairs, typically found inside a building. Basically equivalent to Cat 5 cable. See previous section on Unshielded Twisted Pair cabling.

Type 5

100/50 micron fibre-optic cable, used to connect distant MSAUs with fibre optic repeaters.

Type 6

Patch cables consisting of data-grade, stranded, shielded twisted pairs, 26 AWG. the distant limits are 66% of Type 1 cable.

Type 8

Under carpet cable, data-grade twisted pair cable, 26 AWG. The distance limits are 50 percent of Type 1.

Type 9

Shielded twisted pair, 26 AWG approved for plenum installations. The distance limits are 66% of Type 1 cable.

- 45j. Ring Insertion

When any node or host wishes to attach to the ring, it initiates the Ring Insertion process. The Ring Insertion process has 5 phases:

Phase 0 Lobe Media Check

The Lobe Media Check is performed by the NIC and it verifies the Lobe cable by looping the station transmit signal to the station receiver at the MSAU. A Lobe Media test MAC frame is issued. The relay in the MSAU is not energized at this time. A special packet is sent from the NIC to the de-energized MSAU lobe relay. The packet loops back from the MSAU and returns to the NIC. The integrity of the wiring that makes up the lobe is be checked.

The NIC applies Phantom Power (DC voltage) on the Transmit pair to activate the relay at the MSAU port. The NIC is now physically connected to the Ring.

Phase 1: Monitor Check

The ring station waits for an Active Monitor Present frame, Standby Monitor Present frame or Ring Purge MAC frame. If the ring station does not receive one of these frames before the T(attach) timer runs out, the ring station initiates Token Claiming to re-elect an Active Monitor.

Phase 2: Address Verification

The station verifies that its MAC address is unique with the Ring. It sends a Duplicate Address Test MAC frame onto the ring. The Duplicate Address Test frame has the source and destination address set to its own MAC address. If the frame returns marked read, then the station knows that there is another node with an identical address.

Phase 3: Neighbour Notification

The station learns its Nearest Active Upstream Neighbour (NAUN) address and informs its downstream neighbour of its own address through the Neighbour Notification process.

Phase 4: Request Initialization

The workstation sends the Request Initialization MAC frame to the Ring Parameter Server (RPS), which responds with an Initialize Ring Station MAC frame containing the station's parameters, such as the local ring number, ring parameter timer values, etc.. If no RPS is available, the station will insert with its default parameters.

When Phase 4 is complete the station is physically and logically attached to the ring.

- 45k. CAUs & LAMs

Smart concentrators or Hubs are called CAUs (pronounced cows) in Token Ring. CAU stands for Control Access Unit. It has a CPU built in and the smarts to control and determine when a Node is operating incorrectly. It can determine if the RI or RO main ring is operating properly. CAUs can make decisions on disconnecting nodes or wrapping the Main Ring to the Backup Ring. They are also able to be controlled and programmed from a remote station - SNMP compliant (Simple Network Management Protocol). Nodes can be remotely disconnected from the Ring. CAUs controls LAMs.

A CAU can control up to 4 LAMs (pronounced lambs). LAM stands for Lobe Access Module and LAMs have the Lobe connections. The CAU is connected to the LAMs by a Power Connection and a Data Connection. A LAM has 20 lobe connections. A LAM is an active concentrator.

Active Concentrators

An active concentrator is a concentrator that retimes and regenerates the data signal. It does the job of a repeater. Since it retimes and regenerates the data signal it is not used in Ring Length calculations.

- 45l. Ring Calculations

Maximum Ring Length

The ring length of a Token Ring network is based on the length of the cable used in concentrator-to-concentrator connections, and in the longest concentrator-to-node connection. It is based on Type 1 cabling.

Ring Speed		Maximum Ring Length

4 Mbps			1200 ft/ 360 m

16 Mbps			550 ft/ 168 m

Ring Length Calculations

The following ring has Type 1 cabling, 4 passive concentrators and the lobe with the Maximum Lobe Length (MLL) indicated. The cable length for this ring is calculated by adding all the cable lengths between the passive concentrator's Ring In and Ring Out connectors together plus the Maximum Lobe Length (MLL).

Total Cable Length = MML + RI/RO cable lengths

Total Cable Length = 40 ft + 6 ft + 75 ft + 230 ft + 80 ft = 431 ft

The passive concentrators (MSAUs) will have an effect on the ring length also. Each passive concentrators will appear as 25 ft of Type 1 cable. The Ring Length has to be adjusted for the presence of each of the MSAUs:

Ring Length = Cable Length + (number of MSAUs x 25 ft)

Ring Length = 431 ft + 4 x 25 ft = 531 ft

If you check the maximum ring length parameters mentioned earlier, you will see that this ring would function within the specifications for both a 4 Mbps and a 16 Mbps Token Ring.

Mixing Cable Types and Ring Length

The Ring Length is always calculated based on Type 1 cable. All other types of cable used in the network are converted to Type 1 cable first before determining the Ring Length. The conversion factors for other cable types is indicated in the following table:

Cable Type		Conversion Factor

Type 1, 1A or 2 STP		1.0	(this is the reference)

Type 6 STP			1.3

Cat 5 UTP			1.7

Cat 3 UTP			3.0

For example, in the following Token Ring, there is a mixture of cable types. The first step is to convert the cable lengths to their equivalent Type 1 cable length.

Cable	Type		Length	Conversion Factor	Type 1 equivalent length

A	Cat 5 UTP	80 ft		1.7		136.0 ft

B	Type 6 STP	12 ft		1.3		15.6 ft

C	Cat 5 UTP	65 ft		1.7		110.5 ft

D	Cat 3 UTP	127 ft		3.0		381.0 ft

E	Type 2 STP	185 ft		1.0		185.0 ft

Total Cable Length =		828.1 ft

Ring Length = Total Cable Length + (number of MSAUs x 25 ft)

Ring Length = 828.1 ft + (4 x 25 ft) = 928.1 ft

If you check the maximum ring length parameters, you will see that this ring would function within the specifications for a 4 Mbps but not for a 16 Mbps Token Ring.

Active Concentrators and Ring Length

Active concentrators and the cables connecting the Ring In/Ring Out connectors are not counted in determining the Ring Length. Since it retimes and regenerates the data signal it is not used in Ring Length calculations.

In the following 16 Mbps Token Ring, all cable types are Type 1. If the active concentrator is ignored, a quick addition of cable lengths indicates that the ring length is over the maximum allowable ring length for 16 Mbps Token Ring. In fact, the active concentrator allows ring lengths over the maximum allowed ring length by discounting the cables attached to it.

Total Cable Length = MML + RI/RO cable lengths

Total Cable Length = 75 ft + 12 ft + 210 ft = 297 ft

The 165 ft and 95 ft cables are not counted because they are attached to the active concentrator's Ring In/Ring Out connectors.

Ring Length = Cable Length + (number of MSAUs x 25 ft)

Ring Length = 297 ft + 3 x 25 ft = 372 ft

The active concentrator is not counted in the calculation of Ring Length so only 3 MSAUs are used. The ring length appears to be 372 ft which is within the limits of 16 Mbps Token Ring (550 ft). In fact, the use of an active concentrator allows this 16 Mbps to operate with a total cable length that is actually greater than the allowed ring length for 16 Mbps Type 1 cabling.

- 45m. Token Ring Monitors and Servers

Active Monitor (AM)

The Active Monitor (AM) is the active node with the highest address: it wins active monitor status by a token claiming process that takes place between all active nodes. Any node can become the AM, all other nodes become Standby Monitors in case the Active Monitor fails or turns off.

The duties of the Active Monitor are:

- Maintaining the Master Clock

- Ensuring Proper Ring Delay

- Initiating Neighbour Notification

- Monitoring Neighbour Notification

- Monitoring Token and Frame Transmission

- Detecting Lost Tokens and Frames

- Purging the Ring

Maintaining the Master Clock

The Active Monitor maintains the ring's master clock which controls timing and ensures that all other clocks on the ring are synchronized. The AM beats the drum for the other nodes to follow.

Ensuring Proper Ring Delay

It inserts a latency buffer (delay) to guarantee a minimum ring length. The delay is 24 bits long for 4 Mbps Token Ring and 32 bits long for 16 Mbps Token Ring.

Initiating Neighbour Notification

The Active Monitor periodically broadcasts the Active Monitor Present MAC frame to all ring stations on its ring, allowing each to acquire the address of its Nearest Active Upstream Neighbour (NAUN). The NAUN address is used during error isolation to determine if there is a failing component in a given ring station's fault domain (next node over).

The nodes on the ring are aware of the MAC addresses of their Nearest Active Upstream Neighbour. Notice the word "active", the neighbour must be connected to the ring.

Monitoring Neighbour Notification

At any time during the neighbour notification cycle, certain events could happen that could affect the Neighbour Notification process. The Active Monitor checks for these conditions and takes appropriate action:

1. The Active Monitor's Neighbour Notification Timer runs out. The ring is taking too long to complete the Neighbour Notification process. The Active Monitor restarts the Neighbour Notification process and reports a Neighbour Notification Incomplete MAC frame to the Ring Error Monitor (REM - just another node on the ring that has the job of monitoring ring errors).

2. The Active Monitor Present Frame takes too long to circle the Ring. The AM initiates token claiming so it can retransmit an Active Monitor Present frame.

3. If a Standby Monitor Present Frame is received after Neighbour Notification is complete, the Neighbour Notification is ignored and restarted. Another station has just connected to the ring and by inserting into the ring, has changed the NAUN order.

4. If another Active Monitor Present Frame is received with a source address different from its own. This means that there is another Active Monitor on the Ring. The receiving Active Monitor shuts down and becomes a Standby Monitor.

5. A hard error (cable fault, student playing with RI and RO ports) causes the ring to go down. After the hard error is fixed, the Neighbour Notification process is restarted.

Monitoring Token and Frame Transmission

The AM monitors the ring to make sure that Tokens and Frames only circle the ring once. There is a Monitor bit in the MAC frame and whenever a MAC frame is repeated by the AM, the AM sets the Monitor bit to "1". All frames that are received with the Monitor bit set to 1 are not repeated - this means that the frame has already circulated the ring once.

Detecting Lost Tokens and Frames

The AM has a timer to check that there are Tokens and Token Frames circulating the ring. The timer is set for the absolute longest time that it would take for a Token or Frame to circulate the ring. If the timer times out before a new Token or frame is received. The ring is purged and a new Token is released.

Purging the Ring

The AM broadcasts the Ring Purge MAC frame to all ring stations on its ring before originating a new Token. Receipt of the returned frame indicates to the AM that a frame can circulate the ring without incident. The Ring Purge Frame resets the ring stations to Normal Repeat mode.

Standby Monitor (SM)

There is only 1 Active Monitor allowed on the ring at a time, all other stations become Standby Monitors. Standby Monitors determine whether the Active Monitor is functioning properly. If a Standby Monitor determines that the Active Monitor is not operating properly, the Standby Monitor initiates the Token Claiming process.

Duties of the Standby Monitor

a. The Standby Monitor checks to see if a Token is circulating the ring. It has a Timer called the "Good_token" timer and knows that a Token has to circulate within that time. If the Token does not go by within this designated period, the Standby Monitor knows that Active Monitor is not doing its job. The Standby Monitor then initiates the Token Claiming process to re-elect an Active Monitor.

b. The Standby Monitor restarts another Timer called "receive notification" whenever an Active Monitor Present frame comes by. If the Timer runs out before another Active Monitor Present frame arrives, the SM assumes that the Active Monitor is not present or has malfunctioned. The SM initiates the Token Claiming process to re-elect an Active Monitor.

Ring Parameter Server (RPS)

The Ring Parameter Server provides 3 main services to the ring:

1. Assigns operational parameters to the station at the time of insertion onto the ring. These are parameters such as: Ring Number, Physical Location and Soft Error Report Timer Value. If there is no RPS present, the ring station uses its default values.

2. Ensures that all stations on the ring have the same operational values.

3. Forwards registration information to the LAN Managers from stations attaching to the ring.

Configuration Report Server (CRS)

A Configuration Report Server accepts commands from the network management software to get station information, set station parameters and remove stations from the ring. It also collects and forwards configuration reports generated by stations on its ring to the LAN manager.

The network management software is a program that monitors the Network and is used by the System Administrator. It can monitor many Rings and may include Ethernet segments and connections to WANs.

Ring Error Monitor

The Ring Error Monitor observes, collects and analyses hard-error and soft-error reports sent by ring stations on a single ring and assists in fault isolation and correction.

Hard Error Processing Function

Hard errors are detected, isolated and bypassed through the use of a Beacon MAC frame. Hard errors are broken cables, failed equipment, improper signal timing, incorrect voltage levels.

Any ring station that detects a hard error can generate a Beacon MAC frame. The frame is addressed to all other stations on the ring. A Beacon Frame contains the address of the station that discovered the Hard Error, its NAUN and a physical location (the RPS gives this information to the NIC during initialization).

1. Station G hasn't received any frame for a while. Station G starts a Beacon Frame with Station F as its NAUN

2. When Station F receives the Beacon Frame and reads that it is the NAUN of the Fault Domain. It disconnects from the ring and re-attaches to the ring using the Ring Insertion process.

Note: The cable between Station F and Station G is called the Fault Domain. 3. If the fault still remains on the ring, the Beacon Frame originator Station G, disconnects from the ring and re-attaches using the Ring Insertion process.

A fault that can be cured in this manner is called a Temporary Fault. If the fault cannot be cured than it is called a Permanent Fault.

The Ring Error Monitor monitors the Beacon Frame. It reports the location, NAUN, Beacon Originator address and whether it is a Temporary or Permanent Fault to the network management program.

Soft Error Processing

Soft Errors are errors in the Bit pattern or encoding. There are 5 Soft Error types:

1. Line Error:

A code violation between the starting and ending delimiters in the MAC frame or a Frame Check Sequence error. The FCS doesn't add up!

2. Internal Error:

The ring station recognizes a recoverable internal error. This can be used for detecting a ring station in marginal operating condition.

3. Burst Error:

The absence of transitions for 5 half-bit times. Manchester encoding is used for Token Ring and a low to high transition is a 1 and a high to low transition is a 0.

4. A/C Error:

The Token Ring Frame has 2 bits called Address-Recognized (A) and Frame-Copied (C). During Neighbour Notification, there should not be 2 Standby Monitor Present frames with AC=00 in a row. This would indicate a copy or framing error.

5. Abort Delimiter Error

A station sends out a special frame called an Abort Sequence or Abort Delimiter when it discovers an error (soft or hard) while transmitting a frame.

The Ring Error Monitor keeps track of the number of Soft Errors, who reported them and the NAUN. The REM has thresholds of acceptable Soft Error levels (can be adjusted) and reports excessive Soft Errors to the LAN Manager.

There are two categories of Soft Errors: Non-Isolating and Isolating.

- Non-Isolating means that the Soft Error can only be isolated to the ring.

- Isolating means that the Soft Error can be isolated to a station.

The REM keeps track of all stations and the Soft Errors associated with them.

Where are these Monitors?

Active Monitor

Any station can be the Active Monitor - first station on with the highest MAC address.

Standby Monitor

All other stations besides Active Monitor

Ring Error Monitor

Usually in a Bridge/Router - something that is always on.

Configuration Report Server

Usually in the same Bridge/Router as the Ring Error Monitor.

Ring Parameter Server

Usually in the same Bridge/Router as the Ring Error Monitor

- 45n. Token Ring Hierarchy

NOTE: The LAN Manager discussed here is a Network Management program running on OS/2 and is not Microsoft or IBM's Lan Manager network operating system!

- 45o. IEEE 802.5 Frames

IEEE 802.5 Token Ring Standard has 3 frames specified:

- Abort Sequence

- Token

- Token Frame

Abort Sequence

The Abort Sequence is used to indicate to stations on the ring that a temporary internal error has occurred. The Source NIC has discovered a soft error during transmission and is aborting the transmission. It is a broadcast frame that goes to every station on the local ring.

The Abort Sequence consists of

- Start Delimiter (SD)

- End Delimiter (ED)

Token

The Token, sometimes called the free token, is used for Bus Arbitration in Token Ring. Whichever station claims the Token has the right to transmit its information/data (Token Frame) on the ring. The Token consists of 24 bits:

- Start Delimiter (SD)

- Access Control Field (AC)

- End Delimiter (ED)

It is a broadcast frame that goes to every station on the local ring.

Token Frame

The Token Frame is used for transmitting information/data on the ring. It is the only frame with Source and Destination addresses. The Token Frame consists of the following fields:

- Start Delimiter (SD)

- Access Control (AC)

- Frame Control (FC)

- Destination Address (DA)

- Source Address (SA)

- Information Field (INFO)

- Frame Check Sequence (FCS)

- End Delimiter (ED)

- Frame Status (FS)

IEEE 802.5 Frame Fields

Start Delimiter (SD)

The Start Delimiter (SD) is used to:

- Inform the receiving station that a new frame is arriving.

- Synchronize the transmit and recieve clocks

The SD is 8 bits long and consists of :

J K 0 J K 0 0 0

Where J & K are Manchester coding violations and have no change of state during the half bit. The J code violation is a steady high state and the K code violation is a steady low state.

Access Control (AC)

The Access Control (AC) field is used to:

- Set priority of Token

- Indicate if it is a Token or Information Frame

- To stop frames from continuously circulating the ring

The AC field consists of 8 bits:

P P P T M r r r

Where:

- P = Priority Bits

- T = Token Bit (0 - Token , 1 - Information)

- M = Monitor Bit

- R = Reservation Bits

The Priority Bits are used to indicate the priority of the Token. Each workstation is assigned a priority for their transmissions: 000 is the lowest and 111 is the highest (7 levels of priorities). The Lan administrator sets the priority levels. For a workstation to claim a Token, it must have a priority equal to or greater than the priority of the Token.

It is the responsibility of the node when finished transmitting data, to release the Token and to return the priority bits to the Reservation Bits.

The Reservation Bits are used to negotiate the priority of the next token as a transmission passes by. When a Token or Token Frame goes by, a node is allowed to reserve the priority of the next Token to be released by placing its priority in the Reservation Bits. In order to change the Reservation Bits, the node's priority must be greater than the existing Reservation bits.

The Token Bit is used to indicate whether the frame is a Token or Token Frame (information frame). T = 0 indicates a Token, T=1 indicates a Token Frame.

The Monitor Bit is used by the Active Monitor (AM) to stop frames from continuously circulating the ring. The Monitor bit is set to M=0 by the Source and when a frame passes by the AM, the monitor bit is set to M=1. If the AM receives a frame with the monitor bit set to M=1. The frame is removed from the ring, purges the ring and issues a new token.

Frame Control (FC)

The Frame Control (FC) field is used to:

- Indicate frame type (MAC or LLC)

- Provides communication at the MAC level between stations

The FC field consists of 8 bits:

F F Z Z Z Z Z Z

Where:

- F = Frame Type (00 = MAC, 01 = LLC, 1x = not defined)

- Z = Control Bits

The Frame Type indicates whether it is a MAC layer addressed communication or an LLC layer addressed communication.

The Control bits indicate the type of MAC level communication:

- 00 - Normal Buffer

- 01 - Express Buffered

- 02 - Beacon

- 03 - Claim Token

- 04 - Ring Purge

- 05 - Active Monitor

- 06 - Standby Monitor

Destination Address (DA)

The Destination Address (DA) is use to indicate the destination address of the Token Frame (information frame). It consists of 48 address bits.

The first bit is the Individual/Group (I/G) bit - this is used to indicate an individual or broadcast (to everyone). The 2nd bit is used to indicate whether a Universal or Local naming convention. A Universal naming convention would use the MAC address burnt into the NIC's ROM. The Local naming convention would be set by the Lan Administrator. The remaining 46 bits are the stations unique address.

Source Address (SA)

The Source Address is identical to the Destination Address field except that the Individual/Group bit is always set to Individual (0) for IEEE 802.5. For IBM Token Ring, the first bit is the Routing Bit which instructs bridges to pass the Token Frame (1) or to ignore it (0).

IEEE 802.5

IBM Token Ring

This difference is very important. Only IBM Token Ring allows Source Route Bridging.

Functional Addresses

Token Ring provides MAC addresses that are reserved for special functions:

Null Address:			0000 0000 0000

Broadcast Address:		FFFF FFFF FFFF or C000 FFFF FFFF

Active Monitor Address:		C000 0000 0001

Ring Parameter Server:		C000 0000 0002

Ring Error Monitor		C000 0000 0008

Configuration Report Server	C000 0000 0010

Netbios Address:		C000 0000 0080

Bridge Address:			C000 0000 0100

Lan Manager Address:		C000 0000 2000

User Defined addresses:		C000 0008 0000 to C000 4000 0000

Information Field (INFO)

The Information Field is used to:

- Carry data communication to the MAC layer (IEEE 802.5)

- Carry data communication to the LLC layer (IEEE 802.2)

The Frame Control bits FF determine whether the information is for the MAC (FF=00) or LLC (FF=01) layer. The LLC information is processed identical to Ethernet as covered earlier under IEEE 802.2 and will not be covered here.

The Routing Bit determines whether the frame uses normal MAC layer communication or if Source Routing is used. Only, IBM Token Ring uses Source Routing, IEEE 802.5 does not.

If the Routing Bit is set, then the MAC Frame INFO field contains routing information used during Source Routing. The Routing Bit instructs the bridge to pass the Token Frame or ignore.

If the Routing Bit is not set than normal MAC layer local ring communication is active. This includes specific subcommands called MVID (Major Vector IDs) that work with the Frame Control control bits to: Beacon, Claim Token , Ring Purge, etc.. (See Token Ring MVID Table)

Frame Check Sequence (FCS)

The Frame Check Sequence (FCS) is used for error checking - uses 32 bit CRC (cyclic redundancy checking). It checks the FC, DA, SA and INFO fields. It is 4 bytes long.

End Delimiter (ED)

The End Delimiter is used to indicate the end of the frame. It consists of 8 bits in a "J K 1" sequence where J & K are Manchester coding violations.

J K 1 J K 1 I E

Where:

- I = Intermediate bit, indicates that a frame is part of a multiframe transmission

- E = Error bit indicates that there is an error in the frame such as bad FCS, Manchester code violation, non-integral number of bytes.

Frame Status (FS)

The Frame Status (FS) field is used to indicate whether the address was recognized by the destination and if the frame was copied. This acts as an indication to the source that the destination station is present and accepting data. This FS field consists of 8 bits:

A C r r A C r r

Where:

- A - address recognized bit

- C - Frame copied bit

- r - Reserved bit

Note: The A/C bits are provided twice for redundancy.

- 46. Linux on Token Ring

I decided to implement Token Ring on one of my Linux servers because I had some time on my hands, a few MSAUs and a box of 3Com 3C619B Token Ring network cards. Not to mention a burning desire to run a Token Ring network for the past few years.

This article will deal with:

- installing and configuring a 3C619B Token Ring network card in Linux

- simple routing from Token Ring LAN to an Ethernet LAN through a Linux server.

- 46a. Installing the NIC

The first step was installing the NIC. This required opening the computer and finding a spare 16 bit ISA slot. No problem. In it went and I was one step closer to completion.

The next step required testing the card. Unfortunately, most diagnostic programs that come with PC hardware run in DOS, so as a rule, I always allocate one 20 MB partition to DOS for storing them. Reboot to DOS and run the 3C619B configuration program called 3tokdiag.exe.

At this point the card  should be connected to a MSAU (multistation access unit - sometimes referred to as a MAU) for proper testing. The MSAU can have either the original IBM hermaphroditic connectors, RJ45 or RJ11 connectors. I used an IBM 8228 with hermaphroditic connectors. I connected my RJ45 cable to it using a Token Ring balun (small impedance matching transformer) which matches the 150 ohm impedance of STP to the 100 ohm impedance of UTP.

I ran the diagnostic tests and bang, the MMIO test failed with an error about a memory conflict. So much for right out of the box luck. This meant that I would have to set the card's IRQs, base address and memory address (which I should normally have to do anyway).  A quick check of the Token Ring Howto and voila, it says that the cards with the Tropic chipset (IC has Tropic written right on it) uses the ibmtr driver. The card's chipset was indeed the Tropic and away I went. Now for the configuration parameters.... here was were the problems started.

The 3C619B card could be run in either 3Com mode or 100% IBM compatibility mode. To make a long story short, use the 100% IBM compatibility mode. Even though the settings are not clear, in my case the choices were for "primary or secondary" card which actually means which base address to use. The configuration parameters that Linux is looking for are:

Config mode:			IBM

I/O Base Address:		Primary	(means using 0xA20)

Int Req:			2 (9)		(16 bit cards use IRQ 9)

Ring Speed:			16 Mbps

Bios/MMIO Base Add:		D4000h

shared RAM Address range:	D0000h

Mem mode:			16 bit

I/O mode:			16 bit

IRQ Driver type:		Edge triggered

Auto Switch:			Enabled

I am not sure what the MMIO address does but I know that with these values, the card passed all diagnostic tests fine. The big problem I had was in confusion between MMIO and Memory address. I had set MMIO address to 0xD0000 and this failed miserably.

The first few tests check the internals of the NIC and the last test checks the lobe connection (between NIC and MSAU). The last test takes quite a long time to perform so be patient.

NOTE: Now as far as I can tell, the ibmtr.c source code only allows the above settings (someone correct me if I'm wrong!). Unfortunately, the comment header of ibmtr.c doesn't indicate any configuration settings (oversight?). From what I can tell from ibmtr.c and testing that was performed over a period of 3 weeks (yes that is right - I was on the verge of giving up), these are the only values that will work.

- 46b. The Kernel and Token Ring

The Linux kernel must be recompiled for Token Ring support. You can compile it in directly or as a module both methods work admirably. To compile the kernel, you change directories to /usr/src/linux and run either:

- "make config" - for command line text based line by line prompt

- "make menuconfig" - for a command line configuration menu

- "make xconfig" - for an X windows configuration menu

I suggest that you use either menuconfig or xconfig. The "make config" method can be extremely unforgiving if you should make a mistake - you have to start all over again.

The assumption at this point is that you have a working recompiled kernel and are only adding support for a Token Ring card. This means that the only change should be to add Token Ring support to the kernel. Go to Network Device Support section and select Token Ring Driver Support as either as compiled as part of the kernel (Y) or as a  module (M). I selected compiled as part of the kernel. Next select "IBM Tropic chipset based adapter support" (again Y or M - your choice). Save and exit and you're now ready to recompile the kernel.

make clean ; make dep ; make zImage

make modules

make modules_install

I copied the zImage file to the root directory (I'm using slackware - you may need to copy it to /boot directory for other distributions):

cp /usr/src/linux/arch/i386/boot/zImage /token-ring

Now the new kernel was in place, it's time to add a new lilo entry.

- 46c. LILO and T.R. Kernel

Since I wasn't sure how Linux would work with the new Token Ring card, I wanted to be able to boot to the old working kernel (non Token Ring). I added another entry into /etc/lilo.conf that would address the new kernel. At the lilo boot prompt I would have a new choice of which kernel to boot to. I modified /etc/lilo.conf with a simple text editor for the new kernel:

# LILO configuration file

#

# Start LILO global section

# location of boot device

boot = /dev/hda

# how long (1/10 of seconds) will the LILO prompt appear before booting to the first listed kernel

delay = 50

vga = normal

# End LILO global section

# Linux bootable partition configuration begins

# Original kernel config starts here

image = /vmlinuz	# name and path to kernel to boot to

root = /dev/hda2	# which partition does it reside on

label = linux		# the name that the LILO prompt will display

read-only		# let fsck check the drive before doing anything with it - mandatory

# End of original kernel

# Token Ring kernel starts here

image = /token-ring

root = /dev/hda2	# which partition does it reside on

label = token-ring	# the name that the LILO prompt will display

read-only		# let fsck check the drive before doing anything with it - mandatory

# End of Token Ring kernel

# DOS partition starts here

other = /dev/hda1	# which partition does it reside on

label = dos		# the name that the LILO prompt will display

table = /dev/hda

# End of DOS partition

My DOS partition is on /dev/hda1 and Linux on /dev/hda2 with a swap partition on /dev/hda3 which is not mentionned in the lilo.conf file.

After saving and exiting the /etc/lilo.conf. You must run lilo to enter the setttings. All that is required is to type "lilo" at the command prompt with root privilege. If everything was entered properly, you should see:

ashley:~# lilo

Added	linux *

Added	token-ring

Added	dos

ashley:~#

This indicates that everything went okay (ashley is the name of my server). The asterick indicates that linux is the default boot selection (first entry in lilo.conf).

- 46d. Token Ring Kernel and Boot Messages

Since I compiled Token Ring support directly into the kernel, I didn't have to modify (usually just uncomment) or add support for the ibmtr driver in the /etc/conf.modules file. When I rebooted the machine, I hit the CTRL key at the LILO prompt and then the TAB key. This displayed the lilo boot choices. I typed "token-ring" at the LILO prompt:

LILO: token-ring

I then closely watched for the following messages to scroll across the screen:

<snip>

tr0: ISA 16/4 Adapter| 16/4 Adapter /A (long) found

tr0: using IRQ 9, PIO Addr a20, 16 k Shared RAM

tr0: Hardware address: 00:20:AF:0E:C7:2E

tr0: Maximum MTU 16 Mbps: 4056, 4 Mbps: 4568

<snip>

tr0: Initial interrupt: 16 Mbps, Shared Ram base 000d0000

<snip>

tr0: New Ring Status: 20

tr0: New Ring Status: 20

tr0: New Ring Status: 20

tr0: New Ring Status: 20

<snip>

And its up.. It's quite stable and if you have a passive msau, you shoud bew able to hear the relay click in for the ring insertion phase .

If you see either of these error messages:

arrgh! Transmitter busy

Unknown command 08h in arb

Then you have the wrong Shared Ram Address range configured on your card. Set it to 0xD0000h.

- 46e. Configuring the Interface

Now that there was support for the Token Ring card in the kernel, the interface had to be configured. This means that the IP address, mask, broadcast address and default route must be set. In Slackware, the /etc/rc.d/rc.inet1 file is modified to add the following parameters. If you are just testing, you can type in the following parameters at the command prompt:

/sbin/ifconfig tr0 192.168.2.1 broadcast 192.168.2.255 netmask 255.255.255.0

where:

tr0 is the first Token Ring adapter found

192.168.2.1 is the IP address of the interface

192.168.2.255 is the broadcast address of the interface

255.255.255.0 is the subnet mask

At this point, you should type "ifconfig" by itself on the command line interface and you should see something like this:

eth0      Link encap:Ethernet  HWaddr 00:A0:24:CC:12:6F

inet addr:192.168.1.3 Bcast:192.168.1.255  Mask:255.255.255

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:

RX packets:53775 errors:0 dropped:0 overruns:0 frame:

TX packets:7489 errors:0 dropped:0 overruns:0 carrier:

collisions:0 txqueuelen:100

Interrupt:10 Base address:0xe800

tr0       Link encap:Token Ring  HWaddr 00:20:AF:0E:C7:2E

inet addr:192.168.2.1 Bcast:192.168.2.255  Mask:255.255.255.0

UP BROADCAST RUNNING MULTICAST  MTU:4500  Metric:1

RX packets:0 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:100

Interrupt:9 Base address:0xa20

lo        Link encap:Local Loopback

inet addr:127.0.0.1 Mask:255.0.0.0

UP LOOPBACK RUNNING MTU:3924  Metric:1

RX packets:235 errors:0 dropped:0 overruns:0 frame:0

TX packets:235 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:0

Notice that both the ethernet, loopback and token ring interfaces are listed. It is very important to make sure that the Ethernet and Token Ring adapters are on separate IP networks. In this example, eth0 is on subnet 192.168.1.0 and tr0 is on subnet 192.168.2.0.

At this point you should be able to ping your linux box from the token ring network. Symptoms of a wrong NIC configuration is if you can ping localhost and the linux network card address (like 192.168.2.1) from within the Linux server fine but when you ping anything outside of the linux server (such as other LAN hosts) you get the error messages listed above.

- 46f. Routing from Token Ring to Ethernet

There are two methods that can be used to connect Ethernet networks to Token Ring networks. The first method uses the Data Link layer of the OSI model and is called a translation bridge. There are several major differences between the two MAC frames, one of the most significant is the tranmission of most significant bits (MSB) of a byte. Token Ring transmits the least significant bit (LSB) first while ethernet transmits it in reverse order with the MSB first (or vice versa depending on if you are a Token Ring guy or Ethernet guy). Unfortunately, Linux doesn't support translation bridging for a very good reason (see next paragraph).

The second method uses the Network layer (IP layer) and is called routing. Both Ethernet and Token Ring protocol stacks already deliver their data to the Network layer in the proper order and in a common format - IP datagram. This means that all that needs to be done to connect the two LAN arbitration methods is to add a route to our routing table (too easy!).

Since our ethernet routing is already working including default gateway. I only had to add the following line to /etc/rc.d/rc.inet1. To test type at the command line:

/sbin/route  add - net  192.168.2.0 netmask 255.255.255.0

Any packet not addressed to the Token Ring network 192.168.2.0 is forwarded to the Ethernet network. I used a similar route on the Ethernet side and everything not addressed to the Ethernet network 192.168.1.0 was sent to the Token Ring network.

To verify that everything still works from the Linux box:

- ping an Ethernet host

- ping a Token Ring host

To verify that routing is working, try to ping across the Linux server from an Ethernet host to a Token Ring host and vice versa.

NOTE: This is a very simple routing example. Only two LANs are being used: 192.168.1.0 and 192.168.2.0. Your situation will most likely be more complicated. Please see the man pages on routed for further information.

- 46g. Token Ring Problems

While Linux ran beautifully with Token Ring, I can't say the same about Win95. The biggest problem that I ran into was the fact that Win95 performs a software reboot whenever its configuration is changed or when most new software is installed. While this isn't a problem with Ethernet, it is a problem with Token Ring. Token Ring has many maintenance and administration duties implemented in the network card itself. The network card requires a hard boot to reset not a soft boot.

The results were that the Win95 clients would lose their network connections (specifically the network stack to the NIC) and hang during soft boots - very frustrating. Add any new software especially if it is a network install and bam, down goes Win95 - hung again. I would have to shut off the PC and reboot. I never realized how often you have to reboot Win95 until I implemented Token Ring on it. I would not want to administrate a Token Ring network on Win95 for a living.

This is not a Token Ring fault but a Win95 fault as far as I can tell. I was using Win95a so perhaps later versions have addressed this problem and corrected it. Linux did not have any problems of this nature.

- 47. Source Routing

IEEE 802.5 does NOT provide Source Routing as part of the IEEE standard. Only IBM Token Ring uses Source Routing.

In Source Routing, the Source is responsible for determining the best path to the Destination. The Source sends out a XID (Exchange ID) or TEST frame which picks up routing information along the way to the Destination. As the XID frame goes through bridges/routers, the bridges/routers add their route information into the MAC Info field's Routing Information. The route information contains the Bridge # and the Segment #.

Because multiple routes may exist between rings, the Bridges check the route information to see if the XID frame has already passed into the ring. If it has, the frame is ignored. If it has not been into the new ring, the frame is passed on.

If Station X on Ring 100 wanted to send information to Station Y on Ring 500. Station X would first send out a local ring MAC Token Frame.

When the Frame returns with A/C = 00 then Station X would send a XID Frame (sometimes called a Discovery Frame) to see if Station Y was on another ring. The XID Frame would go to Bridges A, B & C and each would forward the XID Frame to Rings 200, 300 and 400 respectively.

There would be 3 new XID Frames generated now on Segments 200, 300 & 400. Each XID Frame would have the Bridge # and Segment # added to the Routing Information field.

Because Station Y is not on Segments 200, 300 or 400, Bridges D, E & F forward the XID frame to Segment 500. There are now 3 XID Frames forwarded to Segment 500 - Note: only 1 XID Frame exists on the ring at any 1 time.

Station Y acknowledges each of the XID Frames in turn, changes the direction bit in the Routing Information field and they are returned to the Source in the same order.

Station X receives the first XID frame and uses its routing information for all subsequent frame transmission to Station Y. Station X ignores all other XID frames. The assumption is made that the first XID frame received back is the quickest and best route. This method of Source Routing is called All Routes Broadcast (ARB).

If a Spanning Tree Algorithm is used then only 1 path would exist between Station X and Station Y. When Source Routing is used with a Spanning Tree Algorithm, it is called a Single Route Broadcast (SRB).

Note: In IEEE 802.5 and IBM Token Ring, it is the responsibility of the MAC layer to find the best route. Ethernet relies on the Network layer to find the best route.

- 48. ISDN - Integrated Services Digital Network

The Integrated Services Digital Network (ISDN) is a method used to bridge "the last mile" between the Central Office and the premise connection (home). ISDN uses the existing wiring so no new cabling is required.

There are two basic services offered:

- Basic Rate Interface (BRI) consists of 2B + D channels. Which stands for 2 Bearer channels of 64 kbps each for data and one D channel of 16 kbps for handshaking and control. Having a separate channel for handshaking and control is called "out of band" signalling. The 2B channels can be bonded together for a single data channel with a 128 kbps transfer rate.

- Primary Rate Interface (PRI) consists of 23B + D channels. Which stands for 23 Bearer channels of 64 kbps each for data and one D channel of 64 kbps for handshaking and control. The Bearer channels can be bonded in any combination as required.

ISDN lines can be dedicated lines that are always up and connected or they can be dial on demand (DOD) lines. When the line is required the connection is dialed up and made. The connection time for an ISDN line is very quick, in the order of 0.5 second or so. This can result in a substantial cost saving if used over long distance or paying by the minute. The line charges are only for when data is being transferred and not when it is sitting idle.

ISDN - OSI Layers

The D Channel uses up to the OSI Network Layer while the B channel uses both the Data Link and Physical layers.

ISDN - OSI Model

The line encoding is used by ISDN is standard telecommunication 2B1Q which stands for 2 Binary elements encoded in 1 quaternary. A dibit (digital bit) is represents two binary elements for each voltage change. The following table illustrates the 2B1Q encoding used by ISDN:

| Dibit | Voltage |
| --- | --- |
| 10 | +3 |
| 11 | +1 |
| 01 | -1 |
| 00 | -3 |

ISDN Premise Connection

The following diagram illustrates a basic ISDN connection from the Central Office to the premise:

ISDN Premise Connection

The Central Office must have ISDN capabilities in the switch in order to connect to an ISDN premise. The connection from the CO to the premise uses the existing analog phone linges. At the BRI premise, a Network Termination 1 (NT-1) device converts the 2 wire analog line to a 4 wire system called the S/T interface. A PRI rate interface is The S/T interface is a bus topology that terminates in a 100W termination.

A total of 8 ISDN Terminal Equipment 1 (TE) devices are allowed to connect to the S/T Interface. If access is required to traditional analog devices such as a plain old telephone set (POTS) which are called Terminal Equipment 2 (TE2) devices, a Terminal Adapter (TA) can be used to provide access.

ISDN Advantages

- ISDN is a mature technology, it has been around since the late 1980s. It is has been tried, tested and works.

- It is governed by a world-wide set of standards.

- It provides symmetrical transfer rates: the transmit rate is the same as the recieve rate.

- It has consistent transfer rates. If you have a 64kbps bearer channel then that's the speed that you transfer at.

- It is competitive priced compared to other technologies.

ISDN Disadvantages

- An external power supply is required. The telco's don't supply power for ISDN lines. If the power fails, the phones won't work.

- Special digital phones are required or a Terminal Adapter to talk to the existing POTS devices.

- It is very expensive to upgrade a central office switch ($500,000+) to ISDN

- If the ISDN fails - the phone fails.

- 49. ADSL - Asymmetrical Digital Subscriber Line

Asymmetrical Digital Subscriber Line (ADSL) is a method to use the existing analog local loop lines for digital data transfer to and from the home. It is asymmetrical in that the upstream transfer rate is slower than the downstream data rate. This means that the data transfer from the premise (home) to the CO is a different rate than the data transfer from the CO to the home.

The data transfer is rate adaptive. This means that depending on the condition of the local loop lines, ADSL will automatically compensate and find the fastest transfer rate possible. The range for upstream data transfer is 64 kbps to 768 kbps. The range for downstream data transfers is 1.5 Mbps to 8 Mbps. The reasoning for the asymmetrical transfer rate is that most users will be surfing the Internet, upstream requests tend to be small webpage addresses. The downstream data consists of downloads of large graphic intensive webpages. Small upstream requests, larger downstream response.

The data transfer rate depends on the distance from the central office, the quality of the line and the wire gauge. If the distance from the central office is 15,000 to 18,000 ft, then the maximum transfer rate is 1.5 Mbps. If the distance is 9,000 ft or less, the maximum transfer rate is 8 Mbps.

ADSL Standards

At the time of this writing, there are 3 competing standards for ADSL:

- Carrierless Phase Modulation ADSL,

- Splitterless ADSL

- Discrete Multitone ADSL.

Carrierless Phase Modulation (CAP) ADSL is a modulation technique similar to Quadrature Amplitude Modulation. It provides Echo Cancellation and overlaps upstream and downstream signals.

Splitterless ADSL (also called ADSL Lite, G.Lite, PnP ADSL, Universal ADSL) has a lower transmitting rate and is easier to implement.

DMT - Discrete Multitone is an ANSI T1.413 standard which uses a broadband modem that covers the 4 kHz to 2.2 MHz range. It has 256 channels of 4 kHz, each channel is assigned 15 bits of data to transfer. In addition each channel is checked for signal quality and bits assigned accordingly. A poor responding channel may less bits assigned or none at all. DMT adjusts for the local loop line conditions and attempts to make the fastest transfer rate possible.

ADSL OSI Model

ADSL is a Physical layer protocol which covers the transmission of data, and cabling requirements.

ADSL Premise Equipment

ADSL Premise Equipment

ADSL shares the bandwidth of the local loop with the existing phone system. It does not require modification to the central office switch. Instead a splitter combines the ADSL information with the POTS switch's analog information. At the central office end, the ADSL signal is sent to the Digital Subscriber Line Access Module (DSLAM) and then to a communication server.

At the premise end, another splitter separates the ADSL information from the analog information. An ADSL modem called an ATU-R device decodes the ADSL information and sends it to the Service Module (SM). The Service Module translates it to Ethernet. In plain network terms, in comes ADSL and out comes an Ethernet signal for connection to a network interface card (NIC).

ADSL Advantages

- No expensive modification is required to CO switch.

- Simple splitter splits ADSL signal from the existing analog line.

- High bandwidth is available.

- The POTS works regardless of ADSL.

- ADSL has competitive pricing versus other technologies

ADSL Disadvantages

- The transfer rate depends on distance from the central office.

- The presence of bridged taps and load coils on the local loop affect the transfer rate.

- ADSL must be installed to test if it will work.

- 25% of existing local loops will not work with ADSL

- There is an 18,000 ft distance limit from the central office.

- There can be a bottleneck at the communication server at central office.

- 50. Cable Modems

The cable modem technology is a competitive technology to bridge the last mile. Cable television companies are battling head to head with the telephone companies to provide high speed bandwidth to the homes. The telephone companies have the digital equipment backbone starting at the central office but are crippled by the existing local loop cable.

The cable television companies have the high speed bandwidth to the homes but don't have the digital equipment backbones at the head end (the head end is where all the television signals in a cable TV line originate from). Cable modems use the existing cable TV line to provide the high speed bandwidth.

It is an asymmetrical transfer rates with the upstream data transfer rate at 2 Mbps. The downstream data transfer rate is a maximum of 30 Mbps. Most users connect the cable modem to their 10 Mbps ethernet NIC and don't utilize the cable modems full bandwidth.. Switching to a 100 Mbps ethernet NIC would give them full bandwidth.

The actual transfer speed depends on number of users that are on-line. The cable line is shared with the other subscribers in the local neighborhood. Most cable companies use dynamic IP addressing, each time the user connects, the user is assigned a new IP address. For a fee, permanent static IP addresses can be assigned.

Most cable TV companies are placing high performance web proxy servers at the head end. These servers store the most commonly accessed webpages and files locally at the head end. The user's web browser first checks the proxy server to see if the file has been downloaded there. If it hasn't then it goes out on the Internet to download it. The storing of the webpages and files on the local proxy server reduces the load on the communication servers to the Internet and gives the impression of extremely fast Internet access.

Cable Modems Standards

There are three competing standards for cable modems at the time of this writing:

- the European standard DVB/DAVIC

- the American standard MCNS

- the Geneva standard IEEE 802.14.

Only the Geneva standard guarantees the transfer rate.

Cable Modems Premise Equipment

Cable Modems Premise Equipment

The cable modem is connected to the existing cable TV RG59 coax line using a standard RF connector. The output of the cable modem is a 10BaseT or 100BaseT ethernet connection to your NIC.

Cable Modems Security Issues

Cable modems have some security issues. Users can see others on network neighborhood in Windows. Some systems have each cable modem connection is encrypted.

The assignment of IP addresses is based on the MAC address of the ethernet card. Hackers can access the network if they know another users MAC address.

Cable Modem Advantages

- Fast data transfers, up to 30 Mbps if using a 100BaseT NIC

- Competitive pricing against competing technologies.

- Easy to install - home prewired.

Cable Modem Disadvantages

- The available bandwidth depends on the number of users on the local cable TV line segment.

- There is an asymmetrical transfer rate. Upstream is slower than downstream.

- There can be a bottleneck at the communication server at the head end.

- 51. Quick Introduction to UNIX

UNIX is an operating system similar to DOS. It can run on IBM PCs, Sun Workstations, HP computers, etc.. It has been ported to many environments. It is also a multiuser environment. Several users can access the same machine simultaneously.

The purpose of this section is as a basic introduction to Unix and the reader should be aware that there are many in-depth books written on Unix that are available on-line and off-line.

History

The name Unix is not an acronym but a pun on an early operating system called Multics. Unix was original thought of as an emasculated version of Multics (called Unics).

Unix was developed in the early 70s by Bell Telephone Laboratories. Unix was developed using the C language and is easily ported to other platforms. Unix is nonpropriertary - it is not tied to a specific software vendor or tied to a specific hardware platform.

Design of the Unix System

The Unix system consists of the kernel system layer atop the hardware.

The essential core of the Unix operating system is called the kernel. This is the software layer that interacts most closely with the computer hardware. The command interpreter which implements the users commands is called the shell - this is similar to DOS's command line. The shell can also be a GUI (Graphical User Interface) like X Windows.

Also on top of the kernel would run user applications and utilities. Utilities are print managing programs, format commands, etc...

One of the main reasons that Unix has become so popular is the layered approach that the Unix operating system has taken. This has made it very easy to port to other hardware systems.

Unix Variants

Because Unix is nonpropriertary, and has been modified by thousands of programmers at Bell Systems, universities and research organizations around the world, there are may variants of Unix.

The most popular flavours of Unix are (in no particular order):

- Linux

- Xenix

- Sun OS

- Novell UnixWare

- Berkley Unix (BSD)

- SCO Unix

These Unix variants are descendants of the original AT&T Unix code.

Other operating systems are Unix-like in that they have been written from scratch to emulate the behaviour of some versions of Unix. Examples are: Coherent and QNX.

Personal Unix Systems

Unix can be run on IBM PCs, the most common implementation is LINUX. Linux is available off the Internet for free from various distributions. All distributions have the same basic kernel called Linux. The distributions package the Linux kernel with the programs that they feel will provide the best overall package. The programs that they add are custom installation programs, office packages, programming suites, server software, networking software, games, etc..

The most common distributions are (in no particular order):

- Slackware

- Red Hat

- Debian

- SuSe

- TurboLinux

- Caldera

Command Line Operation

The Command Line tells Unix what you want to do. Unix's command line tends to be cryptic and most users use some sort of GUI.

GUIs

GUIs are graphical user interfaces, they all have a look and feel similar to the Windows environment which is based on the MAC Desktop and X Windows.

A graphical user interface for Unix consists of 2 major parts: the X Windows System (often called X) and a Window Manager. An X server on a computer system manages the screen, keyboard and mouse and their interactions with client applications that reside either on the same system or on another computer on a network.

The window manager, a client application, controls the window decorations and behaviours such as resizing, stacking order of multiple windows.

XFree86 is the free X Windows system that is available with Linux. There is a wide choice of free Window Managers available:

- KDE

- Gnome

- WindowMaker

- KVWM

- TVWM

- Lesstif

OSF/Motif is the most popular commerical window manager for Unix vendors other than Sun. Motif is based on the work done by members of the Open Software Foundation (OSF). Motif is also the interface for Open Desktop (ODT) from the Santa Cruz Operations (SCO).

Case Sensitive

Unix's command line is case sensitive. Most commands and responses are presented in lower case letters. Unlike DOS and many other systems, you cannot use upper case letters when lower case letters are expected. Thus typing "LS" to list a directory instead of "ls" will result in an error (the command won't be found). This is especially true of passwords - make sure that when you enter a new password that you remember whether it is upper case or lower case!

Multi-User Operating System

Unix systems run comfortably in a variety of situations: single-user, host systems with users on local or remote terminals and networked arrangements of workstations and multiuser systems.

Multiple Operating Systems

One of the most attractive features of the personal Unix systems is their ability to run programs designed for other operating systems. Versions of Unix designed for IBM PCs lets you load DOS and Windows applications using progrmas (such as dosemu and wine respectively) and run them in windows along side Unix specific programs.

Unix is a Pre-emptive Multi-tasking environment. You can have several programs or processes running simultaneously. This also allows Unix to be a multiuser environment.

File Naming Conventions

Once again Unix is case sensitive which is a difficult area to get used to when you migrate from the DOS environment.

File Name Length

Unix allows up to 255 characters in the file name as opposed to DOS's 8.3 convention. This allows for unique and informative file descriptions rather than the encryptic file names used in DOS.

There is a problem with having long file names if you are working from the command line. Typing in a file name with 255 characters and keeping the case correct can be very frustrating. It is recommended that you keep the file names short and sweet and also use DOS's naming convention if you are going to transfer files between DOS and Unix (very common occurrence).

Allowable Characters in Filename

Only the "/" (forward slash) is not allowed in filenames because it is used as the pathname separator (DOS uses the back slash "\").

Characters to avoid: ? @ # $ ^ * ( ) ` [ ] \ | ; ' " < >

You can use spaces or tabs in filenames if you enclose the names in quotation marks on the command line but they are hard to work with. Use underscores or periods to get visual separation.

Ex. "this is my file" or my_file_is_this_one or here.is.another.file

Don't use - or + as the first character of a filename. Many commands use the - or + to introduce options or switches.

Filenames starting with "." are used by the system to make names invisible to normal directory listings. Typically, preferences or configuration files are "hidden" using a "." prefix. An example is ".signature" used for your electronic signature in E-mail.

Pathnames (/)

Unix uses the forward slash "/" as the pathname separator. Unix's top directory is called the root directory and is indicated by "/".

Compatibility with Other Systems

If you are going to use Unix with other systems such as DOS, make sure that you follow a file naming convention that is compatible with both systems. It is possible that characters that are allowed in one system may not be allowed or reserved in the other system.

DOS only allows 8 characters followed by a 3 character extension while Unix allows up to 255 characters. When transferring from DOS to Unix there is no problem. But when transferring from Unix to DOS, the Unix filename becomes truncated and converted to the 8.3 format.

Ex. Unix: "this_file_is_ver01_of_pkzip" becomes "this_fil.e_i" in the DOS world.

Wild Cards

Unix allows wild card characters in the file names similar to DOS. The Wild cards allowed are the asterick "*" and the question mark "?".

For example:

ls *xt 	will list all files ending with xt regardless of filename length.Such as:

cat_text  readme.txt

ls ?xt 	will only list 3 character long filenames that end with xt. Such as:

txt  cxt

- 51a. Basic Unix Commands

The Basic Unix commands required to navigate through Unix are:

- ls

- pwd

- mkdir

- rmdir

- cat

- cp

- mv

- rm

- cd

ls

ls stands for list directory. It is the equivalent of DOS's dir command. Available options are:

-a	lists all files including hidden files

-l	gives a long listing including rights

"ls" by itself will not display hidden files. There are many other options available but these are the most commonly used ones.

Ex. 	ls

.	..	readme.txt	Krustys_revenge	more_stuff

Ex.	ls -a

.	..	readme.txt	Krustys_revenge	more_stuff

.signature	.profile

Ex.	ls -l

total 956

drwxr-xr-x	6 arh	other	1024	Dec 16 09:44	.

drwxrwxr--x	5 root	sys	96	Dec 12 09:05	..

-rw-r--r--	1 arh	other	681	Jan 28 04:56	.profile

etc...

pwd

pwd stands for Print Working Directory. pwd displays on the screen the current directory that you are in. Before CRTs, all communication with mainframes was printed out on "teletype" style terminals thus the origins of the name pwd.

ex		pwd

/home/bart

mkdir

mkdir stands for make directory. It is similar to DOS's "md" command. In actual fact, originally DOS's make directory command was mkdir for pre-DOS 3.x.

ex.	mkdir homer		makes the directory "homer"

in the current working directory.

rmdir

rmdir stands for remove directory. It is similar to DOS's "rd" command. In actual fact, originally DOS's remove directory command was rmdir for pre-DOS 3.x.

ex. 	rmdir homer		removes or erases the directory "homer"

from the current directory.

cat

cat stands for catalog and is used for displaying files to the screen similar to DOS's "type" command. "cat" allows single screen paging, it waits for a response before displaying the next screen of information.

Ex. 	cat readme.txt			this will display the file readme.txt onto the

screen one page at a time.

cp

cp stands for copy. It is used to copy files similar to DOS's "copy" command.

Ex. 	cp readme.txt springfield.txt	copies the file readme.txt and

names the new file springfield.txt

mv

mv stands for move. It is used to move files from one directory to another and it is also used to rename files.

Ex. 	mv readme.txt /home/x-files	moves readme.txt from current

directory to directory called /home/x-files

Ex.	mv agent.sculley agent.muldar	renames file agent.sculley to new

name agent.muldar

rm

rm stands for remove. It is used to delete files similar to the DOS "del" command. It will verify that you want to delete the file. Wildcards can be used with rm.

Ex.	rm dana.sculley			deletes file named dana.sculley

cd

cd stands for change directory. It is used to change the current directory similar to DOS's "cd" command. Note: to use the double dots requires a space - cd ..

Ex.		cd /x-files/fox.muldar		Makes /x-files/fox.muldar the

current directory

- 51b. Access and Permissions

The owner of the file or directory determines who can access the file and for what purposes. The type of access can be read, write or execute the file privileges.

User/Group/Other

With respect to file and directory access, the user community is divided into 3 categories: user (or owner), group and other.

user (u)	The owner of the files or directories

group (g)	Group members. Groups are users who agree to share certain files and

directories. Groups are usually formed along project or business

organizational lines.

other (o)	All other users of the system.

Each file has a set of values stored in its inode that specifies its permissions. An inode is an entry in the table of inodes that describes the file or directory. The Table of Inodes is Unix's version of DOS's File Allocation Table.

Read/Write/Execute

The permissions indicate, for each category of user, the kind of access allowed. Permission is also called the file's protection mode or simple mode.

Type		File Action					Directory Access

read (r)	Allows file to be viewed		Allows directory to be listed

copied and printed

write (w)	Allows file to be moved,		Allows files to be created in

removed and modified			directory

execute (x)	Allows file to be run as a		Allows directory to be searched

command

Unix displays a file's permissions in the following order:

rwx	where "r" is read, "w" is write and "x" is execute

It is also expressed in Octal Code:

Mode	Octal	Binary	Descriptions

---	0	000	No permissions allowed

--x	1	001	Execute only

-w-	2	010	Write only

-wx	3	011	Write and execute only

r--	4	100	Read only

r-x	5	101	Read and execute only

rw-	6	110	Read and write only

rwx	7	111	Read, Write and Execute

It is important to know the octal code or how to figure out the octal code if you need to change permissions.

When listing a directory using the "ls -l" long directory listing, the files permissions will appear:

ls -l

total 8

drwxr-xr-x		2	rocky		other	96 	Dec 26 	23:16	.

drwxrwxr-x	7	root		sys	96	Dec 24	07:40 	..

-rw-r--r--		1	rocky		other	613	Nov 2	12:30		readme.txt

drwxr-xr-x		2	rocky		other	234	Feb 28	03:40		x-files

The long directory lists all of the rights associated with the file or directory. The mode bits are organized as follows:

The default permissions when you create a file are 777 which is 111 111 111 in binary or (rwxrwxrwx). When a directory is created, the default permissions are 666 which is 110 110 110 or (rw-rw-rw-).

Changing Permissions

When a file is first created, it is created with the default permissions 777 (rwxrwxrwx). This means that anyone can read, write or execute the new file. Unix provides a command to modify the default permissions: umask. "umask" works by deselecting the permissions that you do not want from the default permissions. "umask" by itself reports what the current mask is.

Ex	umask		will report current mask

000		000 indicates no mask and default permissions exist

Ex	umask 027	will set the mask to 0278 or 000 010 1112

000  010  111

777 (default permission)	rwx  rwx  rwx

027 corresponds to 		---  -w-  rwx	permissions deselected

Resulting permissions:		rwx  r-x  ---

user has rwx	(all permissions) 000

group has r-x	(read and execute only) 010

other has ---	(no permissions) 111

The umask command is used during your shell startup script or login script. You set it once during logging into the system and normally won't have to use it again.

chmod

If you need to change a resource (file or program rights) , use chmod to alter the permissions. chmod is the more common method of changing permissions. You can alter the permissions 2 ways: use the read/write/execute switches or use octal coding.

Ex.	chmod +r index.txt	changes the permission for user, group &

other to read.

chmod u +r index.txt	changes the permission for only the user

chmod 755 index.txt	changes the permission to rwxr-xr-x

Notice that chmod works the opposite of unmask. You set the permissions that you want.

Changing Ownership & Group

You can change the owner of a file by using the chown (change owner) command. You must be the current owner of the file to change its owner.

Ex. chown bullwinkle help.rocky.txt	This changes the owner of the file help.rocky.txt to

"bullwinkle"

Similarly, you can change the group identification of the file by running chgrp (change group). You must be the owner of the files or a user who has group write privileges to change the group ownership of files.

Ex. chgrp brains mr.peabody.doc		This changes the group of the file

mr.peabody.doc to the group "brains".

Note: You can remove a file that you don't own if it is in a directory in which you have write permission

- 51c. Links, Instances & Processes

Links

Links are aliases that point to other files that can be on the same filing system (Unix) or across several filing systems. Linking a file is a cross between renaming the file and copying the file to your Home directory. When you link a file, you add a second name to the file, so that, to the operating system it looks like there are two files. A link is actually in a directory, like a file is, but whenever a program tries to get at the file the link represents, Unix sends the program to find the "real" file.

A Soft Link can span file systems (two different servers). If you delete the original file, you delete the Link.

A Hard Link is restricted to two items on the same file system (same server). If you delete either name, the original item is still there under the remaining name only.

You can use a link to shorten typing, linking a file with a long path name to your current directory. For example: if you want to run the file /usr/local/scripts/startup.ksh from your home directory without typing the entire name, you can create a link to it in your home directory called "Startup".

ln -s  /usr/local/scripts/startup.ksh Startup

"ln" by itself is the command to create a hard link, the "-s" option makes it a soft link.

To delete a link, remove it the way you would an ordinary file using "rm". You are actually just removing the link, not the original file.

Instances & Processes

A process is an execution environment set up by the operating system kernel. A process consists of 3 major components:

- system data

- user data

- program instruction.

Unix is a multi-tasking environment, this means that multiple processes can be run simultaneously. (sometimes processes are called sessions). If a program is being run by more than 1 user at a time, each process that is running the program is called an instance.

Example: If the E-mail reader program "pine" is run, it is considered a process under Unix. If another person runs "pine" at the same time, we say that there are 2 instances of "pine" being operated.

Processes are also called "jobs".

ps - Process Status

In order to see which processes are operating under your account, type "ps". "ps" stands for Process Status and will display all processes that are currently operating.

ps

PID	TTY	TIME	COMMAND

16852	1a	0:02	ksh

16889	1a	0:00	ps

When the "ps" command is entered, a display similar to the above will be displayed.

- PID - Process ID number: used by the kernel to keep track of the process.

- TTY - The terminal with which the process is associated with.

- TIME - CPU time spent running the process (not wall-clock time).

- COMMAND - The name of the process running.

The "ps" command is very useful in determining if you have safely exited a program. Quite often in Unix, you may get bumped from a process (such as ftp) and end up disconnected. The ftp process may still be running. When you log back on, check to see what processes you have open using "ps".

kill

If you were bumped out or locked up in a process, you can usually escape back to the command line by doing one or all of the following key stroke commands:

ctlr d		End input (End of file)

ctrl z		Pause  Job

ctrl c		Kill job

ctrl x		Quit program

In order to terminate the process, you would first use the "ps" command to find out if the process is still running and what PID is associated with it.

Example:

While using the E-mail package "pine", it locked up and the above key combinations were pressed in a mad frenzy. Fortunately, we were returned to the command line prompt. "ps" was typed in to see what the process status was:

ps

PID	TTY	TIME	COMMAND

16852	1a	0:02	ksh

16889	1a	0:12	pine

16957	1a	0:12	ps

This indicated that "pine" was still running. In order to terminate or stop "pine", the kill command is used with the PID of the process that we want to terminate:

kill 16889			This will kill "pine"

Unfortunately, sometimes the kill command by itself will not work on all flavours of UNIX. Sometimes you may have to use some extra options such as:

kill -kill 16889

You can escalate the "degree" of kill by using the option "-9" and if that doesn't work then perform the kill on steroids "-15":

kill -9 16889

kill -15 16889

- 51d. Background Processing

All Unix systems have the ability to run multiple commands simultaneously. The process that is currently displayed on the screen is said to be running in the foreground. Other processes by the same user are said to be running in the background (not currently displayed on the terminal).

Job Control is used to manage multiple processes and allows users to manipulate foreground and background processes.

To run a process in the background, add "&" to the end of the normal command:

sleep 120&

In the above example, the sleep command causes the UNIX terminal to do nothing for 120 seconds. The "&" indicates that it is performed in a background process.

Moving a Foreground Process into the Background

In order for a foreground process to be moved into the background, it first must be running in the foreground. Then the foreground process must be stopped using "ctrl z" which is the pause command. Once stopped, the bg (background) command can be used to start the process running again.

Example:

pr index.txt			(prints the file index.txt to standard output)

ctrl z				(stops process - pauses)

stopped				(Unix replies with "stopped" message)

bg				(instructs process to move to the background)

[1] 	pr index.txt		(displays process moved to background)

To display currently running jobs, type:

jobs

[1]	RUNNING	pr index.txt

Moving a Background Job to the Foreground

To move a background job or process to the foreground, you must know the job number. The number in square brackets [ ] indicates the job number.

jobs

[1]	RUNNING	pr index.txt

To move the background job to the foreground, use the fg (foreground) command:

fg	OR

fg1 	OR

fg %1		(1 indicates job number)

This will move job #1 to the foreground. The fg command will be implemented just a little bit different depending on the shell or flavour of Unix run.

- 51e. Shell Programs

Unix allows each individual user to select a customizable command line interpretor. The standard Unix shell, called the Bourne shell after its author, is called sh and it resides in the /usr/bin directory.

The shell is a program that, like any other program, can be replaced. The shell is a command interpreter and a command programming language. It prompts for, reads and executes commands. The commands can come directly from a terminal or from a file.

csh /ksh

All Unix operating systems have the standard Unix shell sh. Two alternative shells csh and ksh are available and widely used. "csh" was developed by the University of California at Berkley. "ksh" was developed by AT&T and the "k" stands for Korn for the developer David Korn. Linux uses a shell called bash for Bourne again shell. It has added refinements from both csh/ksh shells.

Features of the "csh"

- Uses .cshrc and .login files for initialization at startup. These files are used for configuring your environment.

- Has a configuration file called .logout that is read when you end your session.

- Has a command history list.

- Allows job control as discussed earlier.

- Provides directory management. Additional directory commands: cd, dirs, pushd, popd. (Some csh versions don't provide this feature)

- Offers a more C-like syntax than "sh".

- When enabled by "filec", csh can complete partially typed unambiguous names for you.

- A list of aliases is kept for you (in .cshrc) that contain frequently used or complicated commands.

The Korn shell ksh uses the best features of the csh with the following enhancements:

ksh enhancements:

- Command-line editing using editors from hell worse than DOS's edlin. (Don't know where this came from - it's been many years since I wrote this ;-)

- Improved cd (change directory) command.

- Improved shell programming.

- Allows pathname to appear in the prompt similar to DOS.

The ksh uses the .profile file for startup configuration.

Aliases

Depending on the shell that you are using, you will have a startup file called either .cshrc (csh) or .profile (ksh). These files contain startup information for the user. Each user has a startup file in their home directory which can be modified to suit their individual needs.

Aliases are a means of individualizing a user's account. The user can use aliases as a macro to shorten command-line entries. In the startup file, there is an area with the heading "# handy alias". The "#" indicates that this is a comment and is ignored by the operating system. Following this heading is a list of aliases:

alias vt100="set term = vt100"

This line substitutes "set term = vt100" for when you type "vt100" at the command-line. "set term = vt100" is how you tell the Unix system what type of terminal you are emulating. Depending on the flavour of Unix that you are running, you may or may not need the "=" sign after "vt100".

Notice: There is an alias for "help", it is the Unix command "man" (manual).

- 51f. Communicating with Other Users

You can communicate with other users within Unix by two methods: write and talk.

The write command communicates on the command-line. To finish communicating use "ctrl d".

The talk command splits the screen in two. Top of the screen is the destination's half of the conversation. The bottom is the source's half. To finish communicating use "ctrl d" or break.

To initiate either method, type the command followed by the person's username that you want to communicate with. For example:

talk blanchas

To stop users from communicating with you, type:

mesg -n

To allow access:

mesg -y

To see current status:

mesg

- 51g. Creating Users and Groups

password file

User information is contained in a text file called password that is normally located in the /etc directory. The file is often modified by a program called adduser or something similar. The text file password consists of the following information for each user:

eblanchard:1v2B3uWaA.8iA:501:237:Eugene Blanchard:/home/eblanchard:/bin/bash

"eblanchard" 		is the username.

"1v2B3uWaA.8iA" 	is the user's encrypted password.

"501" 			is the user's numerical ID (must be unique)

"237" 			is the group numerical ID that the user belongs to.

"Eugene Blanchard"	is the full name of the user.

"/home/eblanchard" 	is the home directory of the user.

"/bin/bash" 		is the shell program that the user uses.

The password file is available to all users to view. This is because some programs require access to the password file.

Note: usernames longer than 8 characters can cause problems in some programs. For example, the username "eblanchard" cannot telnet into a Linux server from a Win95 telnet client. The username is truncated to "eblancha" and the Linux server doesn't recognize it as a valid username.

Changing your password

To change your password, you type the password command. Depending on the flavour of Unix that you are using, it may be one of the following:

- password

- passwd

- passwrd

- ypasswd (used with NIS)

The password command will prompt you for your existing password (old) then ask you to type in your new password once and then once again to confirm the password. Choose a password that is at least 8 characters long, use alphanumeric characters and a combination of upper and lower case letters. Write down you password in a safe place that you will remember where it is!

group file

The group file is a text file that contains information about the groups that are allowed to use the system. It is located in the directory /etc. Groups are added using a program called addgrp or something similar. Each line in a group file contains four fields:

instructors:NONE:237:eblanchard,dspurgeon,hsylvan

"instructors"		is the group name

"NONE"			indicates that there is no password with this group.

It is limited to the listed users.

"237"			is the numerical group ID

"eblanchard,..		is the list of group members which can be users or other groups

shadow file

Unix passwords are a maximum of 13 characters long and can be encrypted in 4,096 different ways. There are hacking programs that are able to decode the passwords in the password file. In order to fix this security leak, Unix operating systems that are System V compatible have another file called the shadow file that works with the password file.

The password file does not contain the encrypted passwords, instead it points to the shadow file, which contains the encrypted login passwords. The password file will have a blank space where the encrypted password usually resides.

The password file is still available for all users to view but the shadow file is restricted and only the system administrator can view it. In this manner the security loop-hole has been fixed.

- SAMBA, Win95, NT and HP Jetdirect

I am running a computer routing lab that is used to teach routing fundamentals on proprietary equipment. It consists of an 18 seat lab with 9 PCs, 1 server and 1 HP LaserJet 4050N with a HP Jetdirect  print server card installed. The server is running Slackware 4.0 with Linux 2.2.6 on it. Eight of the PCs are running WinNT 4.0 SP5 and one PC is running Win95a.

My requirements for the Linux server are as follows:

- run as a workgroup server not as a primary domain controller

- run as master browser

- share each user's home directory

- share a common directory among users called public

- share the server's CD ROM drive

- share the HP LaserJet printer

There was a choice of using NFS and configure each client to connect to the Linux server or to use SAMBA and only configure the server. During the normal operation of the lab, the clients are regularly rebuilt, rebooted and reconfigured. It was felt that by running SAMBA services, the Linux server would be transparent to the clients and allow the simplest client install.

This article will describe how I used SAMBA to:

- setup SAMBA to run on a Slackware Linux server

- share drives

- connect and logon from Win95

- connect and logon from WinNT using encrypted passwords

- how to connect Linux to HP Jetdirect print server

- how to share a Linux printer using SAMBA

NOTE: This is not a "howto" type of article but an example of a working configuration and the process used to configure SAMBA

- Installing SAMBA

The installation process will vary depending on which distribution of Linux you are running. Under Slackware, select SAMBA during the installation process or if you are adding SAMBA to an existing system, use the pkgtool program.

Change to the Slackware CD, cd to /slakware/N11. Type pkgtool and "Install packages from current directory". For all other distributions, this article will assume that you have SAMBA properly installed on your system.

SAMBA is started under Slackware by the rc script "/etc/rc.d/rc.samba":

#

# rc.samba: Start the samba server

#

if [ -x /usr/sbin/smbd -a -x /usr/sbin/nmbd ]; then

echo "Starting Samba..."

/usr/sbin/smbd -D

/usr/sbin/nmbd -D

fi

The smbd program provides SMB/CIFS services to clients. SMB (Server Message Block) is the services that Win95 and NT clients use to connect over networks. The new name for SMB is the Common Internet File System (CIFS).

The nmbd program is a NETBIOS name server to allow NETBIOS over IP naming services to clients.

Typing "ps -aux" at the command prompt allows us to view the processes that are running and to see if smbd and nmbd are actually present:

USER       PID %CPU %MEM   VSZ  RSS TTY      STAT START   TIME COMMAND

root         1  0.0  0.2   220  128 ?        S    Oct21   0:02 init

root         2  0.0  0.0     0    0 ?        SW   Oct21   0:00 [kflushd]

root         3  0.0  0.0     0    0 ?        SW   Oct21   0:00 [kpiod]

root         4  0.0  0.0     0    0 ?        SW   Oct21   0:00 [kswapd]

root       101  0.0  0.5  1544  380 ?        S    Oct21   0:00 /usr/sbin/smbd -D

root       103  0.0  0.9  1196  584 ?        S    Oct21   0:03 /usr/sbin/nmbd -D

root      8113  0.4  0.9  1164  616 ttyp0    S    11:14   0:00 -bash

root      8120  0.0  1.1  2272  744 ttyp0    R    11:14   0:00 ps -aux

- SAMBA Configuration File

The configuration file for SAMBA is /etc/smb.conf and there are many examples configurations available in /usr/doc/samba-2.0.3/examples.

The /etc/smb.conf can be divided into 3 general sections:

- Global

- Shares

- Printers

The Global section deals with global parameters such as workgroup name, netbios name, IP interface used. For example:

# Global parameters

workgroup = E328		# workgroup name

netbios name = E328-00		# Linux server's netbios name

server string = %h - Linux Samba server 	# comment shown in Win's Network Neighborhood detail view

interfaces = 192.168.1.3/24	# NICs + subnet mask (24 = 255.255.255.0)

encrypt passwords = Yes		# Required for NT (Win95 will work with encrypted or not)

null passwords = No 		# Must have a password

log file = /var/log/samba.	# location of samba log files (many!)

max log size = 50		# maximum size of each log file

socket options = TCP_NODELAY	# Speeds up convergence of netbios

os level = 33			# Gives a higher browse master "priority"

preferred master = Yes		# This server is the browsemaster

guest account = pcguest		# guest account name

hosts allow = 192.168.1. 127.	# networks allowed to access this server using SMB

The Shares section deals with sharing file directories. For example:

[homes]

comment = Home Directories	# comment shown in Win's Network Neighborhood detail view

path = %H			# automatically display user's home directory as SMB share

valid users = %S		# Only user is allowed to access this directory

read only = No			# can read/write

create mask = 0750		# permissions given when creating new files

browseable = No			# only show user's home directory not "homes" folder

[public]

comment = Public Files		# comment shown in Win's Network Neighborhood detail view

path = /home/ftp/pub		# path to public directory

guest ok = Yes			# anyone can access this directory

[cdrom]

comment = Cdrom on E328-00	# comment shown in Win's Network Neighborhood detail view

path = /cdrom			# path to cdrom drive

guest ok = Yes			# anyone can access cdrom drive, public share

The Printers section deals with sharing printers. For example:

[lp]

comment = E328-Laser		# comment shown in Win's Network Neighborhood detail view

path = /var/spool/samba		# path to spool directory

print ok = Yes			# allowed to open, write to and submit to spool directory

You can manually create the /etc/smb.conf file if you know what each of the entries mean or you can use the web GUI called SWAT (SAMBA Web Administration Tool). An added bonus of using SWAT was the online help files that described each of the choices available. I understand that SWAT is installed automatically with all versions of SAMBA from 2.0 and up.

- Running SWAT

The following instructions are taken directly from the /usr/doc/samba-2.0.3/swat/README file:

Running via inetd

-----------------

You then need to edit your /etc/inetd.conf and /etc/services to enable

SWAT to be launched via inetd.

In /etc/services you need to add a line like this:

swat	901/tcp

the choice of port number isn't really important except that it should

be less than 1024 and not currently used (using a number above 1024

presents an obscure security hole depending on the implementation

details of your inetd daemon).

In /etc/inetd.conf you should add a line like this:

swat	stream	tcp	nowait.400	root	/usr/local/samba/bin/swat swat

One you have edited /etc/services and /etc/inetd.conf you need to send

a HUP signal to inetd. On many systems "killall -1 inetd" will do this

on others you will need to use "kill -1 PID" where PID is the process

ID of the inetd daemon.

Launching

---------

To launch SWAT just run your favourite web browser and point it at

http://localhost:901

Note that you can attach to SWAT from any IP connected machine but

connecting from a remote machine leaves your connection open to

password sniffing as passwords will be sent in the clear over the

wire.

You should be prompted for a username/password when you connect. You

will need to provide the username "root" and the correct root

password.

Once SWAT is up and running, you should see the following:

The menu buttons are pretty self-explanatory and there are excellent help screens available. A quick break down of the menus:

- Home: Takes you to the main page

- Globals: Allows you to configure the global parameters

- Shares: Allows you to configure directory shares

- Printers: Allows you to configure printers based on the /etc/printcap file

- Status: Allows you to start and stop the smbd and nmbd server and show the status.

- View: Views the /etc/smb.conf file

- Password: Allows you to change the server password and account.

Whenever changes are made to the configuration in the Global, Shares and Printer section, the changes must be committed using the commit button/icon on the respective page. Otherwise the /etc/smb.conf file is not modified.

Once the changes are committed (/etc/smb.conf modified), the smbd and nmbd server should be restarted. The Status menu has options that allow the servers to be stopped and restarted.

I found that a good way of understanding the process that was going on was to view the /etc/smb.conf file as I made changes using the View button in SWAT.

- Usernames

It is very important that the usernames and passwords are the same for both the Windows and Linux environments. The synchronization of the Linux passwords with the SMB encrypted passwords is done using the shell script mksmbpasswd.sh which is found in the /usr/lib/samba/private.

Note: For Slackware, the directory for SAMBA is /usr/lib not the standard /usr/local directory.

The following information is taken from the /usr/doc/samba-2.0.3/docs/textdocs/ENCRYPTION.txt file:

The smbpasswd file.

-------------------

In order for Samba to participate in the above protocol it must

be able to look up the 16 byte hashed values given a user name.

Unfortunately, as the UNIX password value is also a one way hash

function (ie. it is impossible to retrieve the cleartext of the users

password given the UNIX hash of it) then a separate password file

containing this 16 byte value must be kept. To minimise problems with

these two password files, getting out of sync, the UNIX /etc/passwd and

the smbpasswd file, a utility, mksmbpasswd.sh, is provided to generate

a smbpasswd file from a UNIX /etc/passwd file.

To generate the smbpasswd file from your /etc/passwd file use the

following command :-

cat /etc/passwd | mksmbpasswd.sh >/usr/local/samba/private/smbpasswd

The problem that I found with this step was that I expected that it would automatically recognize shadowed passwords and place them in the smbpasswd file. Unfortunately, it didn't and I had to manually enter in the passwords using the smbpasswd command. Luckly, I had only only about 10 passwords to enter in. There is probably a method of doing this automatically and I am just not aware of it.

Once completed, I was able to use Network Neighborhood and point and click on the Linux directory shares without being prompted for a username and password.

- Configuring the HP JetDirect Card using Linux

Getting Linux and the HP JetDirect card to work was surprisingly easy. The JetDirect card is a print server card that fits into the HP 4050N printer. The first step is to configure the HP JetDirect card and printer. The standard install disk does not contain support for Linux but there is a WebAdmin tool that you can download from HP's website: http://www.hp.com/support/net_printing. I chose to do it manually by using telnet and the built-in webserver of the JetDirect card.

Telneting to the JetDirect Card

In order to telnet to the JetDirect card, you need to configure the printer's IP address. The default IP address is 192.0.0.192 which most likely will not be a valid address on your network. The HP 4050N printer allows you to to configure the IP address through the printer's status window. Select "JetDirect Menu" from the Menu button and then follow the directions for configuring the network. After the IP address is set, configure the subnet mask in a similar manner.

Telnet to your printer's IP address. You have two choices when telnetting in, you can view the current settings of the printer by typing "/" or viewing the help menu using "?" as shown by the following:

Please type "?" for HELP, or "/" for current settings

>/

===JetDirect Telnet Configuration===

Firmware Rev.   : G.07.20

MAC Address     : 00:10:83:1b:41:c7

Config By       : USER SPECIFIED

IP Address      : 192.168.1.10

Subnet Mask     : 255.255.255.0

Default Gateway : 192.168.1.1

Syslog Server   : Not Specified

Idle Timeout    : 120 Seconds

Set Cmnty Name  : notachance

Host Name       : E328-LASER

DHCP Config     : Disabled

Passwd          : Enabled

IPX/SPX         : Disabled

DLC/LLC         : Enabled

Ethertalk       : Disabled

Banner page     : Disabled

>?

To Change/Configure Parameters Enter:

Parameter-name: value

Parameter-name  Type of value

ip:             IP-address in dotted notation

subnet-mask:    address in dotted notation

default-gw:     address in dotted notation

syslog-svr:     address in dotted notation

idle-timeout:   seconds in integers

set-cmnty-name: alpha-numeric string (32 chars max)

host-name:      alpha-numeric string (upper case only, 32 chars max)

dhcp-config:    0 to disable, 1 to enable

ipx/spx:        0 to disable, 1 to enable

dlc/llc:        0 to disable, 1 to enable

ethertalk:      0 to disable, 1 to enable

banner:         0 to disable, 1 to enable

Type passwd to change the password.

Type "?" for HELP, "/" for current settings or "quit" to save-and-exit.

Or type "exit" to exit without saving configuration parameter entries

The first thing that you should do is type "passwd" and add an administrator password to the printer. Next configure the default gateway and then the host name. The rest will be configured using the printer's built-in webserver.

HP JetDirect Webtool

The HP JetDirect webtool has 6 menu tabs available:

- Status Tab

Displays current status of printer including network stats

- Identity

Displays current software/hardware revisions, host name, IP address, etc..

- Configuration

Allows configuration of TCP/IP (default protocol), IPX/SPX, DLC/LLC, Ethertalk and SNMP.

- Security

Allows changing of the administrator password and SNMP community name.

- Diagnostics

Displays statistics and information on TCP/IP, IPX/SPX, DLC/LLC, Ethertalk, printer and Jetdirect.

- Support

Takes you to the HP support website.

- Printing from Linux to JetDirect

In order to print from Linux to the JetDirect print server, an entry was made in the /etc/printcap file. I made a new spool directory called /usr/spool/lj4050n but the default /usr/spool/lpd should really be used. The directory /usr/spool is a softlink to /var/spool.

The following is a listing of the /etc/printcap file that was used to communicate with the HP JetDirect print server:

# HP Laserjet 4050n

lp|lj4050n:\

:lp=/dev/null:sh:\

:mx#0:\

:sd=/usr/spool/lj4050n:\

:rm=e328-laser.domainname.com:rp=text:

Where:

- lp|lj4050n:\

indicates the default default printer "lp" with an alias/description of "lj4050n". If there was a space in the alias, it would automatically be detected as a description.

- :lp=/dev/null:sh:\

indicates that the printer is not connected to a physical port

- :mx#0:\

indicates that there is no maximum file size to send to the printer

- :sd=/usr/spool/lj4050n:\

indicates the path to the spool directory

- :rm=e328-laser.domainname.com:rp=text:

indicates the domain name of the printer to send print jobs to and what format to send it in. The choices were text or raw for HP printers. I found that the printer was intelligent enough that it automatically detected whether it was a text file, postscript file or PCL file.

- Configuring Windows for Linux Shared Printer

From Network Neighborhood, double-click on the Linux server's shared printer icon. Windows will ask you to configure the printer. I shared the printer's configuration CD on the Linux box and went to the disk1 folder to find the INF file. The printer configuration/installation will stop and display a message something to the tune that it "can't find disk2" just go up a directory to find the disk2 folder. It will finish the installation and you are done. I usually run a Print Testpage to ensure that it works properly.

The normal installation procedure is to run the setup utility from the CD. This installs megabytes of data on to the client which was not what I wanted. I only wanted the print driver and found that the above method gave me a quick, clean and simple printer configuration.

- Summary

It was surprisingly easy to configure SAMBA and have it meet the lab's objectives. When I first ran SAMBA, it took less than 10 minutes to communicate with Win95. This was amazing as I had no prior experience with it.

In configuring the lab environment, I ran into a few problems, some annoying and some took a bit of work to sort but all were solved.

An example of one of the annoying problems was having the [homes] folder show up as a share on a client. It was identical to the client's home directory. Selecting "Browseable = No" in the Global section of /etc/smb.conf solved that.

The most frustrating problem was finding out the the smbpasswd file did not automatically convert passwords from shadow files. I kept getting asked for a username and password whenever I tried to connect to a network share. All the documentation indicated that I was doing everything correct. Manually entering each username's password using the smbpasswd program solved this. I am sure that there is an automatic process for this, as this would not be acceptable if there more than my 10 generic user accounts.

All in all, I was able to configure the network quicker and easier than if I used an NT server and the Linux server is totally transparent to the user. Here's an interesting point: this article has taken longer to write than it did to configure the network.

- 53. The Suite of TCP/IP Protocols

Unix and the suite of TCP/IP protocols go hand in hand. It is not possible to separate the two. TCP/IP refers to a suite of protocols not just the TCP and IP protocols. TCP/IP is the network portion of Unix. The following figure relates the Dept. of Defense (DoD) model of TCP/IP with the OSI model. The DoD model is also called the ARPA model (Advanced Research Projects Agency).

OSI Model and the DoD Model of TCP/IP

It is not a perfect matching between the OSI Model and the DoD model, it is close enough in principle. Note: Only a few of the major Application layer protocols are displayed, a complete listing is presented in Appendix I: TCP/IP Well Known Ports.

Network Devices are network interface cards (NIC) and their software drivers. Typically, they are Ethernet cards, Token Ring cards, WAN links such as ISDN or Frame Relay and they can also be modems and serial ports. The most common protocol used is Ethernet which uses an address burned into the NIC to identify itself to the local network. A typical Ethernet MAC (media access control) address is a 48 bit number and would look like 00-02-AF-97-F2-03. Note: the MAC address is always represented by hexadecimal numbers.

IP stands for Internet Protocol and its main job is to find the best route through the Internet to the destination. IP uses IP addresses to identify the host machine and the network. A typical IP address is a 32 bit number and looks like 142.110.237.1 where in this case 142.110.237.0 identifies the network address and 0.0.0.1 identifies the host machine. IP addresses are always represented by decimal numbers. IP protocol data units (PDUs) are called datagrams and provide a connectionless service (send and pray).

ARP stands for Address Resolution Protocol and it is used to map IP addresses to MAC addresses. This is needed because the Network layer is not aware of the Data Link layer's addresses and vice versa.

ICMP stands for Internet Control Message Protocol and is used mainly for troubleshooting TCP/IP network connections. Two common programs, ping and traceroute, are part of ICMP.

TCP stands for Transmission Control Protocol and is used to guarantee end to end delivery of segments of data, to put out of order segments in order and to check for transmission errors. TCP is a connection-oriented service.

UDP stands for User Datagram Protocol and is a connectionless service. This results in a low overhead, fast transfer service that relies on the upper layer protocols to provide error checking and delivery of data.

In the Application layer lies many hundreds of network aware programs and services such as:

HTTP (80) 	- HyperText Transport Protocol which is used for transferring webpages.

SNMP (161/162)	- Simple Network Management Protocol which is used for managing network devices.

FTP (20/21)	- File Transfer Protocol which is used for transferring files across the network.

TFTP (69)	- Trivial File Transfer Protocol which is a low overhead fast transfer FTP protocol.

SMTP (25)	- Simple Mail Tranfer Protocol which is used for transferring email across the Internet.

Telnet (23)	- An application for remotely logging into a server across the network.

NNTP (119)	- Network News Transfer Protocol which is used for transferring news.

The numbers, shown in brackets next to the protocols, are called the Well Known Port Numbers, TCP and UDP use these port numbers to indicate where the segments should be sent to. For example, webservers use Port 80 to indicate that the HTTP protocol is used. ASocket is another name for a Well Known Port. A complete listing of the ports is presented in Appendix I: TCP/IP Well Known Ports.

- 54. Internet Protocol The Network Layer protocol for TCP/IP is the Internet Protocol (IP). It uses IP addresses and the subnet mask to determine whether the datagram is on the local or a remote network. If it is on the remote network, the datagram is forwarded to the default gateway which is a router that links to another network. IP keeps track of the number of transverses through each router that the datagram goes through to reach its destination. Each transvers is called a hop. If the hop count exceeds 255 hops, the datagram is removed and the destination considered unreachable. IP's name for the hop count is called Time To Live (TTL). - IP Addresses IP addresses consist of a 32 bit number and is represented by the dot-decimal format. for example: 142.110.237.1 is an IP address. There are 4 decimal digits separated by three dots. Each digit is allowed the range of 0 to 255 which corresponds to 8 bits (one byte) of information. A portion of an IP address represents the network address and the remaining portion the host address. For example: 142.110.237.1 is the IP address of a firewall. The network that the firewall resided on is 142.110.237.0 (Note: IP addresses that end in a 0 represent network addresses). The host address of the firewall is 0.0.0.1 (Note: the network portion of the IP address is represented by 0s). Each host on the network and Internet must have a unique IP address. There are ways around having each host a unique IP address and they are discussed under firewalls. The Network Information Center (NIC) assigns network addresses to the Internet. You must apply to receive a IP network address. Depending on the class (more on this later) of the IP address, you can then assign as many host IP addresses as allowed. An alternative is to "rent" IP addresses from your local Internet Service Provider (ISP). They usually own the rights to a block of IP addresses and will rent them out for a fee. - 54b. IP Address Classifications There is a formal structure to the assignment of IP addresses. IP addresses are assigned by the Network Information Center (NIC) who is a central authority with the responsibility of assigning network addresses. There are several classifications of IP addresses. They include network addresses and special purpose addresses. Class A addresses IP address range 1.0.0.0 to 127.0.0.0 Number of networks available: 125 (see special addresses below) Number of hosts per network: 16,777,214 Net Mask: 255.0.0.0 (first 8 bits are ones) Special Addresses: 10.0.0.0 is used for networks not connected to the Internet 127.0.0.0 is the loopback address for testing (see ping) Class A addresses always have bit 0 set to 0, bits 1-7 are used as the network ID. Bits 8-31 are used as the host ID. Class A networks are used by very large companies such as IBM, US Dept of Defense and AT&T. Appendix E: IP Protocol Address Space lists the IP addresses and the organizations that use them. Class B addresses IP address range 128.0.0.0 to 191.0.0.0 Number of networks available: 16,382 (see special addresses below) Number of hosts per network: 65,534 Net Mask: 255.255.0.0 (first 16 bits are ones) Special Addresses: 172.16.0.0 to 172.31.0.0 are used for networks not connected to the Internet Class B addresses always have bit 0 and 1 set to 10, bits 2-15 are used as the network ID. Bits 16-31 are used as the host ID. Class B networks are assigned to large companies and universities. Class C addresses IP address range 192.0.0.0 to 223.0.0.0 Number of networks available: 2,097,150 (see special addresses below) Number of hosts per network: 254 Net Mask: 255.255.255.0 (first 24 bits are ones) Special Addresses: 192.168.1.0 to 192.168.255.0 are used for networks not connected to the Internet Class C addresses always have bits 0-2 set to 110, bits 3-24 are used as the network ID. Bits 25-31 are used as the host ID. Class C network addresses are assigned to small companies and local Internet providers. Class D Addresses IP address range 224.0.0.0 to 239.0.0.0 Use: Multicasting addresses Class D addresses always have bits 0-3 set to 1110, bits 4-31 are used as the Multicast address. Class D network addresses are used by multicasting. Multicasting is a method of reducing network traffic. Rather than send a separate datagram to each host if multiple host require the same information. A special multicast address can be used where one datagram is read by many hosts. Appendix F: IP Multicast Addresses lists the assigned IP multicast address space. Class E Addresses IP address range 240.0.0.0 to 255.0.0.0 Use: Reserved by the Internet for its own use. If you try to ping a Class E address, you should get the error message that says that it is an invalid IP address. - Reserved IP Addresses The following IP addresses are reserved: 127.0.0.0 Network addresses used for localhost mode (testing IP stack) 255.255.255.255 An IP address consisting of all 1s in binary (255). Broadcast address x.x.x.0 An IP address with the host portion consisting of 0s. Used to indicate the network address. Newer routers have the option of allowing these addresses. 224.0.0.0 - 255.0.0.0 Class D addresses. - 54c. Network Masking The subnet mask is used to determine which portion of the IP address is the network address and which is the host address. This means that the portions of network to host in an IP address can change. The most common subnet mask is 255.255.255.0. The simple explanation is that wherever there is a 255, this indicates that it is the network portion. Wherever there is a 0, this indicates the host portion. Later on, subnet masking will be explained more thoroughly, for now this explanation will suffice. If we examine our IP address of 142.110.237.1, and use a subnet mask of 255.255.255.0. It can be seen that the network portion of the IP address is 142.110.237 and the host portion is 1. The network address is typically written 142.110.237.0 and the host is sometimes written 0.0.0.1. Now if host 142.110.237.1 wanted to send a datagram to 142.110.237.21. It would look at the network portion of the IP address of the destination and determine that it is on the local network. It would then send the datagram out. If host 142.110.237.1 wanted to send a datagram to 142.110.150.108. It would look at the network portion of the IP address of the destination and determine that it is not on the same network. It is on 142.110.150.0 network and it would send it to the default gateway. The default gateway is a router that knows how to reach the other networks. Class Masking Class A, B and C networks use masks and not subnet masks. Masks are similar to subnet masks except that usually they are used in routers and not workstations. A Class A network has a mask of 255.0.0.0 which allows approximately 16.7 million host addresses. Also, a Class B network has a mask of 255.255.0.0 which allows approximately 65 thousand host addresses. Both classes of networks have too many hosts for one network to handle. Imagine 65,000 users trying to access a network service at the same time. The network would be swamped with requests and slow down to a crawl. The solution is to divide the network up into smaller workable networks called subnets. This is most commonly done by fooling the host machine into believing it is on a Class C network (only 254 hosts) by using a Class C mask: 255.255.255.0. This mask is called the subnet mask. Thus for a Class A network using a subnet mask of 255.255.255.0, you can have roughly 65 thousand subnets of 254 hosts. On a Class B network using a subnet mask of 255.255.255.0, you can have roughly 254 subnets of 254 hosts. Subnetting a network Subnet masks can divide networks into smaller networks than the 254 hosts discussed previously. In order to understand this process, a discussion on binary to decimal number conversion is required. The typical subnet mask 255.255.255.0 represents 4 bytes of data. Each number represents 1 byte and is displayed as a decimal number. One byte of information can represent a range of 0 - 255. One byte consists of 8 bits where 0000 0000 represents 0 in decimal and 1111 1111 represents 255 in decimal. Note: The convention for displaying bits is to group in nibbles (4 bits) to make it easier to read. Each bit position has a weighting, where the weighting is equal to 2 to the power of the position starting at position 0 on the right. The easiest way to determine the decimal weighting is to start on the right with the number 1 (which is 2^0) and double it at each bit position. The weighting for each position is follows: Each position has its weighting multiplied by the binary bit value (0 or 1). For example, if bit position 23 had its bit set to 0, its decimal value would be 0 x 8 = 0. If bit position 25 had its bit set to 1, its decimal value would be 1 x 32 = 32. To determine the decimal value of a binary number, add up all the decimal weighting values where ever there is a 1 in the binary number. For the following binary number 1111 1111, the decimal value would equal 255: For the binary number 0000 0000 the decimal value would equal 0: For the binary number 1010 1001 the decimal value would equal 169: The significance of the decimal weighting to network routing becomes more evident when the method of rolling over the binary count is examined. For example, the decimal number 63 compared to 64 in binary produces an interesting observation: Decimal 63 = 0011 1111 Decimal 64 = 0100 0000 The decimal number 63 is represented by all 1s in the first 6 bit locations. The decimal number 64 is represented by only bit 6 being a logical 1. If the count was further increased, similar relationships would occur at Decimal 127 = 0111 1111 Decimal 128 = 1000 0000 and Decimal 191 = 1011 1111 Decimal 192 = 1100 0000 and Decimal 255 = 1111 1111 Decimal 0 = 0000 0000 Notice that bit 7 and 6 are the only bits that are changing from the initial example of 63 and 64. What this means is that the network can be subdivided into 4 logical networks of 64 hosts each. In actual fact the number is 62 hosts due to address 0 not being allowed (network address) and address 63 not being allowed (broadcast address). In the introduction to subnetting, the portion of the IP address that corresponded to the network portion was easily identified as being the portion of the subnet mask that corresponded to the decimal number 255. This is really only for convenience for the dot decimal format of the IP address. In actual fact, the IP address is a 32 bit address and doesn't have byte "boundaries" as implied by the dot decimal notation. For example: 192.168.1.0 = 1100 0000 1010 1000 0000 0001 0000 0000 This means that the portion of the subnet mask that corresponds to the network address can be further broken down on the host bit positions. A Class C network address of 192.168.1.0 has 254 hosts available to it. If your network consisted of 4 different physical locations each with a maximum of 50 hosts, then subnetting the network would be required. The locations could be different buildings or cities. Bit 7 and bit 6 of the host portion can be used to describe the network portion of the subnet. The subnet masking would be: 1111 1111 1111 1111 1111 1111 1100 0000 = 255.255.255.192 The 4 subnets for Class C address 192.168.1.0 would be 192.168.1.0, 192.168.1.64, 192.168.1.128 and 192.168.1.192 with the following range of IP addresses: Subnet Host Range Broadcast address 192.168.1.0 192.168.1.1 to 192.168.1.62 192.168.1.63 192.168.1.64 192.168.1.65 to 192.168.1.126 192.168.1.127 192.168.1.128 192.168.1.129 to 192.168.1.190 192.168.1.191 192.168.1.192 192.168.1.193 to 192.168.1.254 192.168.1.255 In this manner, a router with 4 interfaces could be configured with subnet masks of 255.255.255.192 to allow subdividing the Class C network into 4 smaller networks. Theoretically, all of the host bits up to bit 1 and 0 can be used to make up to 64 subnets of 2 hosts each. In this case, 128 IP addresses would be lost to the network IP address and the broadcast IP address. The following table lists the number of hosts and networks that can implemented using subnet masking for a Class C network: Subnet mask Number of subnets Number of hosts per subnet 255.255.255.128 2 126 255.255.255.192 4 62 255.255.255.224 8 30 255.255.255.240 16 14 255.255.255.248 32 6 255.255.255.252 64 2 The above example is based on subnetting a Class C network. Subnetting can get extremely complicated if you are subnetting assigned IP addresses that are in the middle of a Class C network such as when rented from an ISP. Fortunately, there are many Subnet Mask Calculators available for download off the Internet that are designed to determine the correct subnet mask for your network. - 54e. Domain Names IP addresses are difficult for humans to remember, they're great for PCs! Domain names were invented to make it easier to navigate the Internet. A domain name is a vaguely descriptive name separated by dots. For example: www.linuxhq.org Every machine that runs TCP/IP has a text file called hosts. It is a simple lookup table that the network stack (IP) checks to see if it has a match between a domain name and an IP address. It is easily modified with a text editor and the contents look like the following: 127.0.0.1 localhost 142.110.237.1 e237-firewall.tech.el.sait.ab.ca 142.110.237.2 e237-bridge.tech.el.sait.ab.ca 142.110.237.3 ashley.tech.el.sait.ab.ca 142.110.237.4 mariah mariah.tech.el.sait.ab.ca The IP address is listed on the left and the domain name is listed on the right. The actual registered domain name is sait.ab.ca (Southern Alberta Institute of Technology). The domain name el.sait.ab.ca (electronics dept.) is a subnet of sait.ab.ca. The domain name tech.el.sait.ab.ca (technical) is a subnet of el.sait.ab.ca. The machine names are e237-firewall, e237-bridge, ashley and mariah. Mariah's entry is unique in that both the domain name mariah and mariah.tech.el.sait.ab.ca would be recognized by the IP stack as 142.110.237.4. The problem with the hosts file is that each machine must have a current up to date copy of the network. For a small network (25 or less) not connected to the Internet this is not a problem to manage. If the network is larger, than problems can occur trying to keep everyone updated. Another solution is Unix's Network Information Service (NIS) (formerly called Yellow Pages until there was a copyright conflict with the Telcos). A central NIS server shares a master hosts file to all the clients. In this way, only one file exists and is updated. This works well for a network not connected to the Internet. If you are connected to the Internet then a Domain Name Server (DNS) is used. A DNS is a special server that communicates with other servers and keeps an up-to-date look-up table that matches IP addresses to domain names for the complete Internet. It is a hierarchical system where each DNS is authorative for the domain underneath it. This means that each server knows the domain name to IP address mapping of the network underneath it. Domain Name Structure Domain names tend to follow a loose structure that gives a description of the network. For example, sait.ab.ca uses the Canada extension ca, preceded by the province of Alberta extension ab and then the abbreviation SAIT for the Southern Alberta Institute of Technology. This is a geographical designed domain name that follows the ISO-3166 country code structure as listed in Appendix K: ISO 3166 Country Codes. Example of country codes are: br Brazil ca Canada fi Finland gb United Kingdom na Nambia nz New Zealand tw Taiwan us United States There are top level domain (TLD) names extensions that attempt to describe the purpose of the domain. It is broken down into seven basic categories: com - Commercial domains that are a business. edu - Educational institutes net - This is for computers of network providers such as Internet Service Providers org - Standard organizations or non profit organizations int - Organizations that have been established by international treaties. gov - Municipal, federal, provincial, state governments. mil - United States military All in all, it is often quite difficult to establish where a domain is physically located or what it actually does from the domain name. But it makes remembering locations easier than attempting to remembering an IP address. Domain Name Look-up Procedure When a domain name is used, the IP stack doesn't understand domain names. It says "what is this? Is not an IP address!". The only thing the IP stack understands is IP addresses. The look-up order is as follows: - The IP stack checks the hosts file to see if there is a domain name match to IP address. If there is, the IP address is used. - If there is no match, the IP stack will look for a NIS server with its host file shared. If this service is not installed, the IP stack will jump to the next step. - If there is still no match, the IP stack will send out a request to the domain name server configured during the network configuration to see if it knows whose IP address belongs to the domain name. - If the domain name server doesn't know, it may make an enquiry to the next level up domain name server to see if it knows whose IP address belongs to the domain name and so on. On the Internet, there are 13 top level root domain name servers. The current addresses and domain names are found at ftp://internic.net/domain/named.cache (also called named.ca and named.root) Domain Name IP Address Description A.ROOT-SERVERS.NET 198.41.0.4 formerly NS.INTERNIC.NET B.ROOT-SERVERS.NET 128.9.0.107 formerly NS1.ISI.EDU C.ROOT-SERVERS.NET 192.33.4.12 formerly C.PSI.NET D.ROOT-SERVERS.NET 128.8.10.90 formerly TERP.UMD.EDU E.ROOT-SERVERS.NET 192.203.230.10 formerly NS.NASA.GOV F.ROOT-SERVERS.NET 192.5.5.241 formerly NS.ISC.ORG G.ROOT-SERVERS.NET 192.112.36.4 formerly NS.NIC.DDN.MIL H.ROOT-SERVERS.NET 128.63.2.53 formerly AOS.ARL.ARMY.MIL I.ROOT-SERVERS.NET 192.36.148.17 formerly NIC.NORDU.NET J.ROOT-SERVERS.NET 198.41.0.10 temporarily housed at NSI (InterNIC) K.ROOT-SERVERS.NET 193.0.14.129 housed in LINX, operated by RIPE NCC L.ROOT-SERVERS.NET 198.32.64.12 temporarily housed at ISI (IANA) M.ROOT-SERVERS.NET 202.12.27.33 housed in Japan, operated by WIDE These are controlled by InterNIC which is the primary agency responsible for registering domain names. At the time of this writing, there are several new agencies that are taking over the domain registration process for different parts of the world. - 54f. IP Header The IP datagram is traditional represented by many rows of 32 bits (4 octets or bytes). Each 32 bit word is stacked on top of each other as per the following diagram for the IP header: Field Description Version (4 bits) The IP version, currently it is version 4 0 Reserved 1-3 Unassigned 4 IP Internet Protocol 5 ST ST Datagram Mode 6 SIP Simple Internet Protocol 7 TP/IX TP/IX: The Next Internet 8 PIP The P Internet Protocol 9 TUBA TUBA 10-14 Unassigned 15 Reserved IHL (4 bits) Internet Header Length, the length of the IP header in 32 bit words. Type of Service Flags to indicate precedence, delay, throughput and (8 bits) reliability parameters. Bit 0-2: Precedence 000 - Routine 001 - Priority 010 - Immediate 011 - Flash 100 - Flash override 101 - CRITIC / ECP 110 - Internet control 111 - Network control Bit 3: 0 = Normal delay, 1 = Low delay Bit 4: 0 = Normal throughput, 1 = High throughput Bit 5: 0 = Normal reliability, 1 = High reliability Bit 6-7: Reserved for future use. Total Length Total length in bytes of the IP datagram (IP header and data) (16 bits) Minimum length = 576, Maximum length = 65,535 Identification Unique identifying number for this datagram (16 bits) Flags (3 bits) Options that indicate if fragmentation s permitted and/or used Bit 0: Reserved, allways set to 0 Bit 1: 0 = May fragment, 1 = Don't fragment Bit 2: 0 = Last fragment, 1 = More fragments Fragment Offset Indicates where in the entire datagram, this particular fragment (13 bits) belongs. Measured in 64 bit units from the beginning of the initial datagram. Time to live (TTL) Measured in hop counts or seconds. Every transverse through (8 bits) a router or gateway will decrement the hop count. When the TTL equals 0, the datagram is discarded. This stops datagrams from circulating the network forever. Starts at TTL = 255. Protocol Identifies the next protocol that follows the IP header. (8 bits) The full listing of protocols is in Appendix G: IP Header Protocols. Examples are: Decimal Protocol Description 0 Reserved 1 ICMP Internet Control Message 2 IGMP Internet Group Management 6 TCP Transmission Control 37 DDP Datagram Delivery Protocol 46 RSVP Reservation Protocol 93 AX.25 AX.25 Frames Header Checksum A 32 bit Cyclic Redundacy Check that may be checked at each (32 bit) gateway. Source Address An IP address indicating the sender. Ex. 142.110.237.1 (32 bit) Destination Address An IP address indicating the receiptient. Ex. 142.110.237.2 (32 bit) Options Options from the sender such as route specifications. (variable) Padding Ensures that the IP header ends on a 32 bit boundary. - 55. Address Resolution Protocol Address Resolution Protocol (ARP) resides in the bottom half of the Network layer. It can be considered a mechanism for mapping addresses between the Network logical addresses and MAC (Media Access Control) layer physical addresses. For example: the Network layer protocol IP is not aware of 48 bit MAC addresses such as Ethernet. Likewise the MAC layer protocol such as Ethernet is not aware of 32 bit IP addresses. ARP provides the mechanism to map MAC addresses to IP addresses in a temporary memory space called theARP cache. The ARP cache is a dynamic cache and the information is stored only for 120 seconds. After which it is discarded. In this manner, the ARP cache remains small. The ARP cache can be viewed by using the "ARP -a" command at a command prompt. This should display the current ARP cache. If nothing is displayed, then most likely your computer hasn't communicated on the network for the past 120 seconds. Ping another device on the network and see if the ARP cache has changed. The basic operation of ARP is as follows. When the IP layer wants to communicate with another device on the network, it checks the ARP cache to see if there is a match with an Ethernet address. If there is no matching entry in the ARP cache, an ARP broadcast datagram is sent out that basically says "Does anybody know whose Ethernet address belongs to this IP address?". The receiving station that has the IP address, responds with an ARP datagram that says "This is my IP address and here is my Ethernet address". The ARP cache is updated and the original IP layer information is then passed on to the MAC layer for processing. ARP and RARP Protocol Data Unit Hardware Type Physical layer hardware which the request is being made. The full listing of hardware types is given in the table titled: IP Hardware Types Examples of hardware types are: 1 = Ethernet (10 Mb) 3 = Amateur Radio AX.25 4 = Token Ring 6 = IEEE 802 networks 11 = Localtalk Protocol Type Protocol code same as Ethernet frame Type field values. HA Length Hardware address length in octets Field lengths assume HA = 6 octets PA Length Protocol address length in octets Field lengths assume PA length = 4 octets Operation Operation code for this message 1 = ARP request 2 = ARP reply 3 = RARP request 4 = RARP reply Sender HA Sender hardware address Token Ring and Ethernet MAC addresses are 6 octets (48 bits) Sender PA Sender protocol address IP address is 4 octets (32 bits) Target HA Target hardware address Token Ring and Ethernet MAC addresses are 6 octets (48 bits) Target PA Target protocol address IP address is 4 octets (32 bits) - 56. Reverse Address Resolution Protocol Reverse Address Resolution Protocol (RARP) is the reverse of ARP. It is a mechanism to map MAC addresses to IP addresses. It is used mainly by diskless workstations upon boot-up to find out their IP addresses from a BOOTP server. The BOOTP server contains all of the boot-up configuration files that the workstation needs to boot-up. On NICs (network interface cards) there is an empty DIP socket that is used for holding a Boot PROM. The Boot PROM holds a special software program that tells the workstation that upon powering up, to go and find a BOOTP server. One of the first tasks of the workstation is to find out its IP address. The MAC layer address is burnt into the NIC and is already known. A RARP broadcast datagram is sent out that asks "Does any BOOTP server know what my IP address is?". The BOOTP server will reply with "Here's the IP address that belongs to your MAC address". Once the IP address is known, then the rest of the configuration files can be downloaded and the diskless workstation booted up. Note: RARP uses the same PDU header as ARP. See ARP PDU. - 57. ICMP - Internet Control Message Protocol The Internet Control Message Protocol's (ICMP) job is to report errors that may have occurred in processing IP datagrams. ICMP is an integral part of IP and and its messages are encapsulated within an IP datagram. Note: there are 6 messaging formats used by ICMP which will be discussed later. The ICMP messages are indicated by the Type Field and are: - 0 - Echo Reply - 3 - Destination unreachable - 4 - Source Quench - 5 - Redirect - 8 - Echo - 11 - Time exceeded - 12 - Parameter problem - 13 - Timestamp - 14 - Timestamp reply - 15 - Information request (obsolete) - 16 - Information reply (obsolete) - 17 - Address mask request - 18 - Address mask reply The most well-known uses of ICMP are the ping and traceroute (tracert in Window) commands. The ping command sends out a special ICMP echo request message to a destination. If the destination is alive, it will respond with the ICMP echo replay message. This is a fundamental tool used in network troubleshooting as described inSection 67. Handy Unix Network Troubleshooting Commands. Traceroute uses the Timestamp services of ICMP to perform its task of tracing a route to a destination. The Timestamp message and Timestamp Reply measure the roundtrip time that is taken to go from the source to the destination. Traceroute lists the path and the roundtrip time to each router taken from the source to the destination. An example of traceroute is described in Section 67.. - 57a. Echo and Echo Reply Message The Echo and Echo Reply test the communication path from a sender to a destination. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Identifier (16 bits); Sequence number (16 bits). DATA. Type Field The Type Field indicates the type of ICMP message. An Echo request message will have the number 8 in the Type field and an Echo Reply message will have the number 0. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Echo, the code field is always 0. Checksum The Checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Identifier and Sequence Number The Identifier and Sequence Number may be used by the echo sender to aid in matching the replies with the echo requests. For example, the identifier might be used like a port in TCP or UDP to identify a session, and the sequence number might be incremented on each echo request sent. The echoer returns these same values in the echo reply. Data The data received in the echo message must be returned in the echo reply message. - 57b. Destination Unreachable Message The Destination Unreachable message is sent by a router (gateway) or a host when the destination network or service is not available. A destination host may reply with a Destination Unreachable message when the service (port or protocol) is not currently active. A router may reply with a Destination Unreachable message when its entry for the destination network is unreachable (set to infinity) or if the IP datagram Don't Fragment flag is set on and the router must fragment it in order to forward it. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Unused. Internet Header + 64 bits of Original Data Datagram. Type Field The Type Field indicates the type of ICMP message. A Destination Unreachable message will have the number 3 in the Type field. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Destination Unreachable, the code field is: - 0 = net unreachable - 1 = host unreachable - 2 = protocol unreachable - 3 = port unreachable ; - 4 = fragmentation needed and DF set - 5 = source route failed Codes 0, 1, 4, and 5 may be sent from a router. Codes 2 and 3 may be sent from a host Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Unused The 32 bits are not used and ignored. Internet Header + 64 bits of Data Datagram The IP header plus the first 64 bits of the original datagram's data. This data is used by the host to match the message to the appropriate process. If a higher level protocol uses port numbers, they are assumed to be in the first 64 data bits of the original datagram's data. - 57c. Source Quench Message If a router does not have enough buffer space and becomes congested or is approaching its capacity, it may send a Source Quench message to the datagram source host. The source quench message is a request to the host to cut back the rate at which it is sending traffic to the internet destination. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Unused. Internet Header + 64 bits of Original Data Datagram. Type Field The Type Field indicates the type of ICMP message. An Source Quench message will have the number 4 in the Type field . Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Source Quench, the code field is always 0. Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Unused The 32 bits are not used and ignored. Internet Header + 64 bits of Data Datagram The IP header plus the first 64 bits of the original datagram's data. This data is used by the host to match the message to the appropriate process. If a higher level protocol uses port numbers, they are assumed to be in the first 64 data bits of the original datagram's data. - 57d. Time Exceeded Message If a router find the Time To Live (TTL) field has expired (set to zero), the router will discard the IP datagram. It may send a Time Exceeded message to the source Host. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Unused. Internet Header + 64 bits of Original Data Datagram. Type Field The Type Field indicates the type of ICMP message. A Time Exceeded message will have the number 11 in the Type field. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Echo, the code field is: - 0 = time to live exceeded in transi t - 1 = fragment reassembly time exceeded Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Unused The 32 bits are not used and ignored. Internet Header + 64 bits of Data Datagram The IP header plus the first 64 bits of the original datagram's data. This data is used by the host to match the message to the appropriate process. If a higher level protocol uses port numbers, they are assumed to be in the first 64 data bits of the original datagram's data. - 57e. Parameter Problem Message If a router or host finds a problem with an IP header, it must discard the datagram. The source host may be notified by being sent a Parameter Problem message. The Parameter Problem message identifies the octet of the original datagram's header where the error was detected. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Pointer (8 bits); Unused. Internet Header + 64 bits of Original Data Datagram. Type Field The Type Field indicates the type of ICMP message. A Parameter Problem message will have the number 12 in the Type field. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Parameter Problem message, the code field is 0 when the Pointer field indicates the error. Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Pointer The pointer identifies the octet of the original datagram's header where the error was detected (it may be in the middle of an option). Unused The 24 bits are not used and ignored. Internet Header + 64 bits of Data Datagram The IP header plus the first 64 bits of the original datagram's data. This data is used by the host to match the message to the appropriate process. If a higher level protocol uses port numbers, they are assumed to be in the first 64 data bits of the original datagram's data. - 57f. Redirect Message If a router (Router A) receives a datagram to forward from a host on the same network. The router (Router A) checks its routing table and finds out that the shortest path is via another router (Router B) on the same network. Router A sends a Redirect message to the host with the IP address of Router B. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Gateway Internet Address. Internet Header + 64 bits of Original Data Datagram. Type Field The Type Field indicates the type of ICMP message. A Redirect message will have the number 5 in the Type field. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For the Redirect message, the code field is: - 0 = Redirect datagrams for the Network - 1 = Redirect datagrams for the Host - 2 = Redirect datagrams for the Type of Service and Network - 3 = Redirect datagrams for the Type of Service and Host Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Gateway Internet Address This field is used to indicate the router with the shortest path to the destination network Internet Header + 64 bits of Data Datagram The IP header plus the first 64 bits of the original datagram's data. This data is used by the host to match the message to the appropriate process. If a higher level protocol uses port numbers, they are assumed to be in the first 64 data bits of the original datagram's data. - 57g. Timestamp Messages The Timestamp messages are best known as part of the traceroute (tracert in Windows) programs. The purpose is to measure the round trip time it takes to go from a source to a destination host. The Timestamp message is a request for measuring roundtrip time. The Timestamp Reply is the reply to a Timestamp message. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Identifier (16 bits); Sequence number (16 bits). Originate Timestamp. Receive Timestamp. Transmit Timestamp. Type Field The Type Field indicates the type of ICMP message. A Timestamp message will have the number 13 in the Type field and a Timestamp Reply message will have the number 14. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For the Timestamp message, the code field is 0. Checksum The checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Identifier and Sequence Number The Identifier and Sequence Number may be used by the echo sender to aid in matching the replies with the echo requests. For example, the identifier might be used like a port in TCP or UDP to identify a session, and the sequence number might be incremented on each echo request sent. The echoer returns these same values in the echo reply. Originate Timestamp The Originate Timestamp is the time in units of 32 milliseconds since midnight Universal Time (UT) when the datagram was transmitted from the source. Receive Timestamp The Receive Timestamp is the time in units of 32 milliseconds since midnight Universal Time (UT) when the datagram was received at the destination. Transmit Timestamp The Transmit Timestamp is the time in units of 32 milliseconds since midnight Universal Time (UT) when the Timestamp Reply datagram was transmitted from the destination. If the time is not available in miliseconds or cannot be provided with respect to midnight UT then any time can be inserted in a timestamp provided the high order bit of the timestamp is also set to indicate this non-standard value. - 57h. Address Mask Messages The Address Mask Request is used by a host to determine what its address mask is on a network. The Address Mask Reply message is the reply from a router or a host to the source host with the correct address mask for the network. 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Type (8 bits); Code (8 bits); Checksum (16 bits). Identifier (16 bits); Sequence number (16 bits). Address Mask. Type Field The Type Field indicates the type of ICMP message. An Address Mask Request message will have the number 17 in the Type field and an Address Mask Reply message will have the number 18. Code Field The Code Field is used by the different message formats to indicate specific error conditions. For Address Mask messages, the code field is always 0. Checksum The Checksum is the 16-bit ones's complement of the one's complement sum of the ICMP message starting with the ICMP Type. Identifier and Sequence Number The Identifier and Sequence Number may be used by the echo sender to aid in matching the replies with the echo requests. For example, the identifier might be used like a port in TCP or UDP to identify a session, and the sequence number might be incremented on each echo request sent. The echoer returns these same values in the echo reply. Address Mask The Address Mask field contains the 32 bit subnet mask for the network (e.g. 255.255.255.0). - 58. Transmission Control Protocol (TCP) The Transmission Control Protocol (TCP) is responsible for reliable end to end delivery of segments of information. Segments is the term that is used to describe the data that is transmitted and received at the Transport level of the OSI model where TCP resides. TCP also redirects the data to the appropriate which port (upper level service) that is required. The reliable end to end delivery of data is accomplished by: - Connection-oriented service Segments are acknowledged to the source when received by the destination. A sliding window is used to enable unacknowledged segments on the "wire" in order to speed up transmission rates - Sequencing of segments Data is broken up into segments that are numbered (sequenced) when transmitted. The destination TCP layer keeps track of the received segments and places them in the proper order (resequences). - Requesting retransmission of lost data If a segment is lost in transmission (missing sequence number). The destination will timeout and request that all segments starting at the lost segment be retransmitted. - Error checking Segments are checked for data integrity when received using a 32 bit CRC check. The redirection of data to the upper level service is accomplished by using Source and Destination Port numbers. Multiple connections to the same service is allowed. For example, you may have many users (clients) connected to a single web server (http is normally port 80). Each client will have a unique Port number assigned (typically above 8000) but the web server will only use Port 80. - 58a. TCP Header 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Source Port (16 bits); Destination Port (16 bits). Sequence Number. Acknowledgement Number. Offset (1st 4 bits); Flags (last 6 bits); Window. Checksum; Urgent Pointer. Options + Padding. Data. Source Port The Source Port is a 16 bit number that Indicates the upper level service that the source is transmitting. For example: - 21 - ftp servers - 23 - telnet servers - 25 - smtp servers (Internet mail) - 80 - http servicers (web) Appendix I is a complete listing of well known ports. TCP allows port numbers to be in the range from 0 to 65,535. Clients will have a unique port number assigned to them by the server. Typically the number will be above 8,000. Destination Port The Destination Port is a 16 bit number that Indicates the upper level service that the source wishes to communicate with at the destination. Sequence Number The Sequence Number is a 32 bit number that indicates the first octet of information in the data field. This is used to number each TCP segment transmitted in order to keep track of segments for sequencing of segments and error checking of lost segments. The source numbers the sequence of transmitted segments. Acknowledgement Number The Acknowledgement Number is a 32 bit number that is used to acknowledge the receipt of segments by the destination. The acknowledgement is the next sequence number expected. If the sender does not receive an acknowledgement for a segment transmitted, the sender will time-out and retransmit. Offset (4 bits) The Offset field consists of the first 4 bits (xxxx0000) of the first byte. The last 4 bits are reserved for future use and are set to 0. The Offset measures the number of 32 bit (4 byte) words in the TCP header to where the Data field starts. This is necessary because the TCP header has a variable length. The minimum length of the TCP header is 20 bytes which gives an Offset value of 5. Flags (last 6 bits) The Flags Field consist of the last 6 bits (00xxxxxx) of the second byte with the first 2 bits reserved for future use and they are set to 0. The Flags field consists of the following flag bits: - URG (Urgent Flag) When set indicates that the Urgent Pointer field is being used. - ACK (Acknowledge Flag) When set indicates that the Acknowledgement Number is being used. - PSH (Push Flag) An upper level protocol requires immediate data delivery and would use the Push (PSH) flag to immediately forward all of the queued data to the destination. - RST (Reset Flag) When set the connection is reset. This is typically used when the source has timed out waiting for an acknowledgement and is requesting retransmission starting at a sequence number. - SYN (Synchronize Flag) When set, it indicates that this segment is the first one in the sequence. The first sequence number assigned is called the Initial Sequence Number (ISN) - FIN (Finish Flag) When set, it indicates that this is the last data from the sender. Windows (16 bits) This contains the number of unacknowledged segments that are allowed on the network at any one time. This is negogiated by the Source and Destination TCP layers. Checksum The Checksum field is 16 bits long and calculates a checksum based on the complete TCP Header and what is called the TCP Pseudo header. The TCP Pseudo header consists of the Source IP Address, Destination IP Address, Zero, IP Protocol field and TCP Length. The IP Protocol field value is 6 for TCP Urgent Pointer This field communicates the current value of the urgent pointer as a positive offset from the sequence number in this segment. The urgent pointer points to the sequence number of the octet following the urgent data. This field is only be interpreted in segments with the URG control bit set. Options Options may occupy space at the end of the TCP header and are a multiple of 8 bits in length. The allowed options are: - Kind 0 - End of option list. - Kind 1 - No Operation. - Kind 2 - Length 4 Maximum Segment Size. This is used to indicate the maximum segment size allowed. Padding The TCP header padding is used to ensure that the TCP header ends and data begins on a 32 bit boundary. The padding is composed of zeros. Data The data field contains the IP header and data. - 59. User Datagram Protocol (UDP) The User Datagram Protocol (UDP) is a connectionless host to host service that operates at the Transport layer of the OSI model. UDP relies on the upper layer protocol for error correction and reliable service. The protocol is transaction oriented, and delivery and duplicate protection are not guaranteed. The major uses of this protocol are DNS and TFTP. UDP has a small header and for all intensive purposes adds Port addressing to the IP header. The IP header routes datagrams to the correct host on the network and UDP routes the datagram to the correct application. - 59a. UDP Header 0 1 2 3 4 5 6 7; 8 9 10 11 12 13 14 15; 16 17 18 19 20 21 22 23; 24 25 26 27 28 29 30 31. Source Port (16 bits); Destination Port (16 bits). Length (16 bits); Checksum (16 bits). Data. Source Port The Source Port is a 16 bit number that Indicates the upper level service that the source is transmitting. Appendix I is a complete listing of well known ports. UDP allows port numbers to be in the range from 0 to 65,535. The Source Port is optional and if not used, a field of 0s is inserted. Clients will have a unique port number assigned to them by the server. Typically the number will be above 8,000. Destination Port The Destination Port is a 16 bit number that Indicates the upper level service that the source wishes to communicate with at the destination. Length The Length field is 16 bits long and indicates the length of the UDP datagram and has a maximum value of 65, 535 bytes and a minimum value of 8 bytes. Checksum The Checksum field is 16 bits long and calculates a checksum based on the UDP header, Data field and what is called the UDP Pseudo header. The UDP Pseudo header consists of the Source IP Address, Destination IP Address, Zero, IP Protocol field and UDP Length. The IP Protocol field value is 17 for UDP. Data The data field contains the IP header and data. The Data field may be padded with zero octets at the end (if necessary) to make a multiple of two octets. - 60. SNMP SNMP stands for Simple Network Management Protocol. It is not actually a protocol but a client server application that runs on the UDP (User Datagram Protocol) service of the TCP/IP protocol suite. It was developed to be an efficient means of sending network management information over UDP using Ports 161(SNMP) and 162 (SNMPTRAP). SNMP consists of three parts: Messages, Agents and Managers. - SNMP Messages (such as Get and GetResponse) communicate the management information. - SNMP Managers asks the questions (polls) and manages the Agents approximately every 15 minutes to see if anything has changed. - SNMP Agents are resources to be managed such as hosts, servers, routers, hubs... There are only 5 PDUs (Protocol Data Units) associated with SNMP: - GetRequest allows the SNMP Manager to access info stored in the Agent. - GetNextRequest allows the SNMP Manager to obtain multiple values. - GetResponse is a response from the SNMP Agent to GetRequest, GetNextRequest and SetRequest. - SetRequest is used by the SNMP Manager when configuring Agents - Trap reports that an event has occurred by the SNMP Agent. (Error or status has changed) This is where the simple comes from - only 5 commands! Efficiency suffers because UDP restricts the amount of information that will fit into a single UDP packet. For example, at least 1 GetNextRequest is required for each row in a routing table. There may be hundreds of rows in 1 routing table - hundreds of packets generated. UDP is a connectionless protocol. Agents have no method of confirming that the events that they have reported using Trap have been received. This is considered to be a "safety" issue. Security is also a problem. The only mechanism for determining if an authorized network manager is requesting management action is a clear text community name in the packet header. There is no method of providing privacy of management information. Due to these security faults, most SNMP implementation is used for monitoring and no "strong" management functions are defined. - 60a. SNMPv2 to the Rescue SNMPv2 combined the RMON (Remote Network Monitoring) MIB definition (see following section) and Secure SNMP. Secure SNMP provides strong authentication and privacy mechanisms suitable for network management in a generally open environment. These two independent evolutions of SNMP were brought together in a new version of SNMP called SNMPv2. In addition, SNMPv2 was extended to address the concerns over protocol efficiency and safety (acknowledgements of Trap). Two new protocol functions (PDUs) were added (GetBulk and Inform Request) to allow efficient reading of tables and provide reliable communications. NOTE: SNMPv1 and SNMPv2 do not interoperate. The two versions must be translated to communicate with each other. - 60b. MIB - Management Information Base SNMP also consists of an extensible Management Information Base (MIB) that runs on each agent's workstation. Each MIB contains the configuration data for that device. The MIB structure is defined by the Structure of Management Information (SMI) language. Basically the MIB details what information is stored for any specific type of agent, how it is stored and how storage is to be structured. - 60c. RMON - Remote Network Monitoring RMON is a MIB definition that provides for remote network monitoring and manager to manager communications. This standard defines the information that a device must collect to provide an accurate picture of a network's health. The information that the RMON MIB provides is divided into the following 9 groups: - 1. Segment Statistics: Statistics on the attached segment: packets, octets, collisions... - 2. History: Data collected by the statistics group for future retrieval and analysis - 3. Alarms: Thresholds can be set for certain alarm condition. (Collisions/sec) - 4. Events: Thresholds can be set for certain event conditions to trigger certain activities. - 5. Host: Statistics can be gathered on individual nodes or hosts on the LAN. - 6. Host Top N: Statistics can be gathered on individual nodes over a user-specified time. - 7. Traffic Matrix: Shows the amount of traffic and number of errors between any host pair. - 8. Packet Capture: Allows capturing of entire packets or individual slices indicating protocols used. - 9. Filters: The filter group specifies the criteria that the packet is going to be captured by. SNMP allows you to monitor and control a TCP/IP Network using SNMP Managers. All nodes (Agents) on a network, regardless of whether they are SNMP compliant, should show up on the Network map. The SNMP non-compliant nodes typically show up as being present but not accessible. SNMP Compliant nodes allow information to be displayed that indicates how the node is configured. Machine Type, Ethernet Address, DOS Ver, IP Address, Name, etc... 61. Handy Unix Network Troubleshooting Commands - 1. Handy Unix Network Troubleshooting Commands The following network troubleshooting commands will vary slightly in syntax depending on which operating system is used. Some operating systems will provide more options and some less. Please use the following information as a guide and the syntax presented with "a grain of salt". arp Use this command to see the IP to MAC address translation table if you are having problems connecting to other network hosts. It is a dynamic cache which updates every 120 seconds. ARP stands forAddress Resolution Protocol (ARP). arp - a address resolution protocol host name (IP address) at (ethernet address) ping The ping command is the most versatile network troubleshooting command in Unix. Use it to verify that your TCP/IP network services are operating correctly. The ping command allows you to determine that the: - TCP/IP stack is configured properly - Network interface card is configured properly - Default gateway and subnet mask is configured properly - Domain name services is configured properly. The following is a step by step guide in determining if your network stack is configured properly: - ping 127.0.0.1 Checks that your TCP/IP stack is working properly up to the network interface card (NIC). If this fails, check to see if you have TCP/IP services loaded. - ping "IP address of default gateway" Checks that the network interface card is working on the local subnet by pinging the local side of the default gateway which is a router. If this fails, check that TCP/IP is bound to the NIC. Then check that the NIC's IRQ, and base address are set properlyboth on the card itself and in the operating system's interface configuration. - ping "IP address across the gateway" Checks that the default gateway is correctly identified in the TCP/IP configuration and that the proper subnet mask is configured. The IP address selected must not be on the local subnet. - ping "domain name" Checks that the domain name services (DNS) is correctly configured in the TCP/IP stack. A domain name is a name like www.yahoo.com. If it fails, check that the DNS server's IP address is entered in properly in the TCP/IP configuration.. netstat The network status command netstat displays status information about the network interfaces on the host machine and it can display routing table information. - Local interface status - netstat -ain - a - all interfaces are displayed - i - displays configuration - n - IP addresses (!n - host names) - example output: - Name MTU Net/dest address Ierrs Opkts Collis Queue - le0 1500 (net IP addr) (local IP) . .. ... .... - Routing table information - netstat -r (-nr or rn gives ip addresses) - Routing Table - Destination Gateway Flags Interface - (net or host) (IP address) UHGD (name) - U - up H - host G - gateway D- discovered using ICMP Req ifconfig The ifconfig command is used to display the local interface configuration (winipcfg for Windows) and to modify the configuration. Local interfaces can be Ethernet network cards, modems, etc.. ifconfig (interface name) (down/up/nothing) nothing gives status (interface name): flags - 63 (up, broadcast, notra, Running) inet (ip address) netmask FFFFFF00 broadcast 128.6.7.255 route The route command allows you to add static routes to the routing tabling. route (-n) (add/delete/nothing) (dest IP address/subnet/DEFAULT) (local IP address) (hop) traceroute (tracert in Windows) Traceroute displays the routers that are passed through to reach the destination. traceroute "IP address or domain name" Tracing route to www.apllejcok.com [192.168.1.64] over a maximum of 30 hops: 1 116 ms 134 ms 112 ms ts10.dshark.com [192.168.128.20] 2 124 ms 112 ms 114 ms bl1.poufe.com [192.168.130.1] 3 122 ms 118 ms 117 ms fifo.amalag.com [192.168.64.2] 4 130 ms 156 ms 132 ms dfg.apllejcok.com [192.168.1.1]. - 62. X.25 X.25 is an analog packet switching network. It can be considered Slow Packet Switching. The transfer speeds are typically 56 kbps to 2.08 Mbps. There is a world-wide set of Public X.25 Networks and it is possible for an organization to have its own private X.25 network. X.25 is over 20 years old and an established technology. There are many multi-vendor solutions and dissimilar technologies in an organization are allowed to access the X.25 network. In Canada, the main X.25 network is called Datapac which is a public offering of X.25. You pay either a flat rate or by the packet. X.25 is used to connect LANs together. Due to its slow transfer speed, it is used for: - Host terminal emulations: low data - Client/Server applications such as E-mail: small files, bandwidth - File Server: large amount of data & real-time traffic (doesn't work well) - Databases: usually large databases but queries are small inbound and medium size outbound. X.25 has a high protocol overhead compared to other networks. This reduces the transfer speed and bandwidth utilization - means its not as efficient. Overhead Example: Truck A represents X.25, it has a heavy empty weight of 5 tons (overhead). The bridge (medium) only allows 6 tons of weight, this means that Truck A can only carry 1 Ton of cargo (Data). Truck B is a smaller truck and weighs 3 tons empty, this means that it can carry up to 3 tons of cargo (data) across the bridge (medium). Truck B makes better use of its weight when crossing the bridge - utilizes its bandwidth better (it is more efficient). - 62a. X.25 OSI Layers X.25 consists of 3 layers: - X.25.3 - Network Layer: - PLP (Packet Layer Protocol) or - SNDCF (Subnetwork Dependant Convergence Function) - X.25.2 - Datalink: - LAPB (Link Access Procedure Balanced) - HDLC (High Level Data Link Control) - X.25.1 - Physical with 4 different types: - X.21 - Sync Digital Interface 9.6kbps - unbal, 64Kbps - bal - X.21bis - Leased Line Analog Interface - V.24 - RS232 Leased Lines - V.35 - RS232 Duplex operation over Leased Lines In actual fact, pure X.25 only defines the DTE to DCE connection but when we talk about the X.25 Packet Switching Network we talk about the above 3 layers. X.25.1 is the Physical layer and is uses 4 flavours of medium (similar to the multiple Ethernet flavours: 10BaseT, Thinnet and Thicknet). The X.25 packet is carried on serial data lines. X.25.2 uses HDLC & LAPB for the Data Link layer. LAPB is considered a subset of HDLC. Both are similar to IEEE-802.2 LLC (Logical Link Control) and provides 2 way communications. The B in LAPB stands for balanced communications and is another way of saying Full-Duplex - both sides communicating at the same time. The X.25 packet is carried within the LAPB frame's info field similar to how the LLC packet is carried within the MAC frame's info field. X.25.3 is known as SNDCF or Subnetwork Dependant Convergence Function. X.25 uses IP network addresses and is one of the reasons for the high overhead. X.25 connects to the network using a DCE modem or DSU/CSU (Data Service Unit/Channel Service Unit). X.25 allows 4096 logical channels to be connected on 1 physical connection. The Packet Assembler/Disassembler (PAD) connects the DSU/CSU to the DTEs (user devices) which can be terminals or LANs. The X.3 standard governs the operation of the PAD and the X.28 standard governs the operation of the PAD to terminal connection. The X.29 standard defines the End to End communications from DTE to DTE through the X.25 Network. - 62b. X.25 High overhead X.25 has a high overhead because it provides extensive error checking. Each device in the X.25 network acknowledges every packet sent. This slows down the transfer of information and uses up available bandwidth. When X.25 was first introduced, the quality of the analog phone lines required this extensive error checking but now with digital lines available it is not necessary. There are 2 types of connections used with X.25: - PVC - Permanent Virtual Circuits: These are leased lines and require no call connect/disconnect - VC - Virtual Circuits: These are like dial-up lines (switched circuits). They require a call connect/disconnect procedure and end to end communication through the network. VC (virtual circuits) have handshaking very similar to how modems connect as covered earlier. X.25 is often used because an X.25 network is considered 1 Hop. Normal Routing (Non X.25) Between San Francisco to Boston Dashed Route takes 5 hops (Salt Lake City, Denver, Chicago, Detroit, Boston) Dotted Route takes 8 hops (Los Angeles, El Paso, Houston, New Orleans, Alanta, Washington, New York, Boston) With X.25, the complete Network across the United States would look like only 1 Hop. X.25 Packet Networks takes care of the routing path. To connect any point in our example of the USA together would be only 1 hop.

- 62c. X.25 Packet Formats

There are three X.25 packet formats:

- Call Request - Call connection/disconnection

- Control Packet - Data control

- Data Packet - Information transfer

Call Request Frame

The Call Request Frame is used to initiate and setup the call from one X.25 service to another. After the call is established, the Call Request frame is not used.

Logical Channels

There are 4096 Logical Channels available on a single physical connection to an X.25 network. The Logical Channels are divided into Groups and Channels. There can be 16 groups (4 bits) of 256 channels (8 bits). 16 x 256 = 4096. The Logical Channel Numbers (LCN) are used to identify the connections to the Network.

Type Field:

For Call Request the Type Field is always equal to 0000 1011. Note the last bit is called the Control bit and is set to 1.

Length of Calling/Called Address:

The Length of Calling/Called Address allows different sizes of addresses for other protocols. The standard protocol is IP with an address length of 32 bits.

Calling/Called Address:

Only used during Call Connect until the Virtual Channel is established then the LCN (Logical Channel Numbers) are used to identify the connections.

Facilities Length

Indicates the length of the Facilities Field

Facilities Field

Indicates the types of facilities available. Facilities depend on the Network provider and can include information such as Charges or Call Forwarding.

Once a Call is established, The Calling/Called Addresses and Facilities Fields are not required and the Control Packet Format and Data Packet are used.

Control Frame

The Control Frame is used to control the communications during the call.

X.25 Type field table for all 3 packet types

Data Frame

The Data Frame is used to transfer data between destination and source.

Q

Optional - Q is used to distinguish between data and control information.

D

D controls the type of Acknowledgment:

- 0 - Network Control: DTE to DCE

- 1 - End to End Acknowledgement: DTE to DTE

Modulo

It is used by the sliding window. The modulo allows multiple unacknowledged packets on the network for faster response. The modulo can be Modulo 8 or Modulo 128 (127 packets out on the network)

Piggyback

The Piggyback field is used for Acknowledgement with the Modulus (sliding window)

Sequence

The Sequence field holds a unique packet number which identifies the packet.

More

The More field indicates that more data is coming.

- 63. Frame Relay

Packet Switching was developed as a method of multiplexing packets across the phone systems.

The term Bandwidth on Demand arises when using X.25 and Frame Relay. Both use packet switching which is a fancy way of saying that the packets are multiplexed on a common line. LAN traffic tends to be very bursty - periods of inactivity then large fast bursts of data. This is a very inefficient use of bandwidth if a large proportion of the time the line is idle. By using statistical multiplexing, bandwidth is used only when required hence the term Bandwidth on Demand.

Frame Relay is very similar to X.25. In fact Frame Relay was developed to improve on X.25's shortcomings: (slow speed and large overhead). Frame Relay was improved over X.25 by the following:

- Decreased Protocol Overhead

- Digital Switching

- Increased bandwidth to 2 Mbps+

- Voice over data

- 63a. Decreased Protocol Overhead

Frame Relay works on the Data Link and Physical layers:

It uses the LAPD (Link Access Protocol D) for framing the packets. Higher layer protocols such as TCP/IP are encapsulated in the LAPD packet.

Frame Relay Frame

F 	- Flag

HDR	- Header

DLCI	- DL connection

C/R	- Command Response

EA	- Extended Address

CF	- Congestion Forward Notification

CB	- Congestion Backward Notification

DE	- Discard Eligibility bit

CRC 	- Cyclic Redundancy Check

Frame Relay drops error checking at each node and relies on the upper layers such as TCP/IP to perform error checking. If a CRC (bit level error checking) error is found the packet is dropped. It is up to the Network layer to retransmit the packet. The assumption is that the Public Network is reliable enough to allow this.

This means that only the end devices are responsible for recovery from transmission errors. Routers A, B & C do not have to worry about error recovery. It is the responsibility of the end devices: Host A & B to take care of error recovery.

Digital Switching

Frame Relay is made to interface with T1 (1.544 Mbps) and E1 (2.048 Mbps) digital switching lines. This results in a higher transfer speed and a much more reliable connection. This reliability allows Frame Relay to drop some of the error checking from the Data Link and Physical layers and rely on the Network Layer.

Increased Bandwidth

Frame Relay can presently operate up to 2.048 Mbps (E1) but there has been successful testing to 155 Mbps. Due to the low overhead, a Frame Relay network will be more efficient at sending data than an X.25 network. This means that more data will be transferred for the same amount of bytes.

- 63b. LAPD - Link Access Protocol D channel

LAPD (Link Access Protocol D channel) deals with Logical connections: Logical Channel Numbers. LAPD takes care of the Virtual Connections (dial-ups) and Permanent Virtual Connections (leased lines). Again, you can have 1 physical connection to the Frame Relay network and many virtual connections (logical). The logical channel numbers are referred to as DLCI (Data Link Connection Identifiers) in Frame Relay. Looking at the Frame Relay frame, we can see that there are only 10 bits assigned to the DLCI. This means that there can be a maximum of 2^10 = 1024 logical channels per physical connection.

- 63c. LAN to Frame Relay Connection

- PC Block Diagram

- PC Quick ID Guide

- Expansion Bus Identification

8 Bit ISA Bus - 8 MHz Expansion Card + Motherboard Connector

16 Bit ISA Bus - 8 MHz Expansion Card + Motherboard Connector

32 Bit EISA Bus - 8 MHz Expansion Card + Motherboard Connector
(has two rows of connectors)

VESA Local Bus 32 Bit 33 MHz Expansion Card + Motherboard Connector

PCI Bus 32 Bit 66 MHz Expansion Card + Motherboard Connector
(has opposite offset from all other cards in relation to card slot openning)

- CPU Identification

On the CPU will be a part number such as 80386SX-20

80386 indicates a 386 processor
SX indicates subcategory of the processor
20 indicates the processors speed in MHz

- CPU Packages

40 Pin DIP Dual Inline Package 8088 and 8086

SMD Surface Mount Device 80286, 80386SX

PLCC Plastic Leaded Chip Carrier 80286, 80386SX

PGA Pin Grid Array 80386DX 80486 Pentiums

- Keyboards

83 Key XT Keyboard
(No space between numeric keypad and QWERTY keys, no LEDs)

84 Key AT Keyboard
(Space between numeric keypad and QWERTY keys)

101 Key AT Enhanced Keyboard
(12 Function keys moved to top of keyboard in a row, Separate arrow keys and Delete/insert keys..)

Windows 95 Keyboard

- Expansion Card Connectors

DB9 Female (Socket) Video MDA, CGA or EGA

DB15 (2 rows) Female (Socket) Game Port/Joystick

DB15 (3 rows) Female (Socket) Video VGA or SVGA

DB15 Female (Socket) With locking mechanism Ethernet AUI port 10Base5

DB25 Female (Sockets) Parallel/Printer Port

DB37 Female (sockets) External Floppy Drive

DB9 Male (Pins) Serial Port COM1 or Mouse

DB25 Male (Pins) Serial Port COM2

- Printer Cable

PC end DB25 Male (Pins)

Printer end Centronics connector

- Ethernet Card Connectors

10Base2 BNC Bayonet Navy Connector RG-58 Coax cable

10BaseT RJ45 CAT 5 Twisted Pair cable

10Base5 DB15 Female (Socket) With locking mechanism Ethernet AUI port RG-8 Coax

- Video Monitors

MDA - Monochrome Display Adapter (TTL), DB9 Male Connector or RCA Jack 9", Green or Yellow phosphour

CGA - Color Graphics Adapter, DB9 Male Connector, Monitor should say "Color" or "RGB"

EGA - Enhanced Graphics Adapter, DB9 Male Connector, Monitor should say "EGA"

VGA or SVGA - Video Graphics Array or Super Video Graphics Array, DB15 Male Connector

- Hard-drives Interfaces

ST506 or ESDI - Enhanced Small Device Interface Hard-drives, 34 Pin Control and 20 Pin Data Card Edge Connector

IDE Integrated Drive Electronics and EIDE Enhanced IDE Hard-drives, 40 Pin DIP connector

SCSI Hard-drive - Small Computer System Interface, 50 Pin DIP connector or Hi density connector

SCSI - Small Computer System Interface, External 50 pin Connector

- Ribbon Cable Connectors

I/O Card, 10 pin DIP, Ribbon Cable Connector, COM1 or COM2

I/O Card, 16 pin DIP, Ribbon Cable Connector, Game/Joystick Port

I/O Card, 20 pin DIP, Ribbon Cable Connector, ST506 or ESDI, Data Connector

I/O Card 34 pin DIP Ribbon Cable Connector Floppy Drive or ST506/ESDI data connector

I/O Card, 40 pin DIP, Ribbon Cable Connector, IDE or EIDE Connector

I/O Card, 50 pin DIP, Ribbon Cable Connector, SCSI bus

Pin 1 of DIP connector is indicated by:

a number 1 on the silkscreen,
a square or oval pad 
and/or an angle in the silkscreen outline

Pin 1 on a ribbon cable is indicated by a red line marked on one edge of the ribbon cable.

- Memory Devices

DRAM DIP memory Dual Inline Package

Cache RAM, SRAM, Static RAM

BIOS Chip, 28 pin DIP

9 bit DRAM (8 data +1 parity bit), SIPP Memory, Single Inline Pin Package

9 bit DRAM (8 data +1 parity bit), 30 pin SIMM Memory, Single Inline Memory Module, 9th chip is parity bit

8 bit DRAM (8 data bits), 30 pin SIMM Memory, Single Inline Memory Module, 9th chip (parity bit) is missing

32 bit DRAM, 72 pin SIMM Memory, Single Inline Memory Module

- Floppy Drive Cable

Controller 34 pin connector, drive B: then a twist in cable and drive A:
Floppies can use edge connector or 34 pin dual row connector

- Ethernet Type Field

Ethernet          Exp. Ethernet    Description

decimal  Hex      decimal  octal

000   0000-05DC   -       -    IEEE802.3 Length Field]

1536   0600       1536   3000   XEROX NS IDP

0660                     DLOG

0661                     DLOG

2048   0800        513   1001   Internet IP (IPv4)

2049   0801        -      -     X.75 Internet

2050   0802        -      -     NBS Internet

2051   0803        -      -     ECMA Internet

2052   0804        -      -     Chaosnet

2053   0805        -      -     X.25 Level 3

2054   0806        -      -     ARP

2055   0807        -      -     XNS Compatability

2076   081C        -      -     Symbolics Private]

2184   0888-088A   -      -     Xyplex

2304   0900        -      -     Ungermann-Bass net debugr

2560   0A00        -      -     Xerox IEEE802.3 PUP

2561   0A01        -      -     PUP Addr Trans

2989   0BAD        -      -     Banyan Systems

4096   1000        -      -     Berkeley Trailer nego

4097   1001-100F   -      -     Berkeley Trailer encap/IP

5632   1600        -      -     Valid Systems

16962   4242        -      -     PCS Basic Block Protocol

21000   5208        -      -     BBN Simnet

24576   6000        -      -     DEC Unassigned (Exp.)

24577   6001        -      -     DEC MOP Dump/Load

24578   6002        -      -     DEC MOP Remote Console

24579   6003        -      -     DEC DECNET Phase IV Route

24580   6004        -      -     DEC LAT

24581   6005        -      -     DEC Diagnostic Protocol

24582   6006        -      -     DEC Customer Protocol

24583   6007        -      -     DEC LAVC, SCA

24584   6008-6009   -      -     DEC Unassigned

24586   6010-6014   -      -     3Com Corporation

28672   7000        -      -     Ungermann-Bass download

28674   7002        -      -     Ungermann-Bass dia/loop

28704   7020-7029   -      -     LRT

28720   7030        -      -     Proteon

28724   7034        -      -     Cabletron

32771   8003        -      -     Cronus VLN

32772   8004        -      -     Cronus Direct

32773   8005        -      -     HP Probe

32774   8006        -      -     Nestar

32776   8008        -      -     AT&T

32784   8010        -      -     Excelan

32787   8013        -      -     SGI diagnostics

32788   8014        -      -     SGI network games

32789   8015        -      -     SGI reserved

32790   8016        -      -     SGI bounce server

32793   8019        -      -     Apollo Computers

32815   802E        -      -     Tymshare

32816   802F        -      -     Tigan, Inc.

32821   8035        -      -     Reverse ARP

32822   8036        -      -     Aeonic Systems

32824   8038        -      -     DEC LANBridge

32825   8039-803C   -      -     DEC Unassigned

32829   803D        -      -     DEC Ethernet Encryption

32830   803E        -      -     DEC Unassigned

32831   803F        -      -     DEC LAN Traffic Monitor

32832   8040-8042   -      -     DEC Unassigned

32836   8044        -      -     Planning Research Corp.

32838   8046        -      -     AT&T

32839   8047        -      -     AT&T

32841   8049        -      -     ExperData

32859   805B        -      -     Stanford V Kernel exp.

32860   805C        -      -     Stanford V Kernel prod.

32861   805D        -      -     Evans & Sutherland

32864   8060        -      -     Little Machines

32866   8062        -      -     Counterpoint Computers

32869   8065        -      -     Univ. of Mass. @ Amherst

32870   8066        -      -     Univ. of Mass. @ Amherst

32871   8067        -      -     Veeco Integrated Auto.

32872   8068        -      -     General Dynamics

32873   8069        -      -     AT&T

32874   806A        -      -     Autophon

32876   806C        -      -     ComDesign

32877   806D        -      -     Computgraphic Corp.

32878   806E-8077   -      -     Landmark Graphics Corp.

32890   807A        -      -     Matra

32891   807B        -      -     Dansk Data Elektronik

32892   807C        -      -     Merit Internodal

32893   807D-807F   -      -     Vitalink Communications

32896   8080        -      -     Vitalink TransLAN III

32897   8081-8083   -      -     Counterpoint Computers

32923   809B        -      -     Appletalk

32924   809C-809E   -      -     Datability

32927   809F        -      -     Spider Systems Ltd.

32931   80A3        -      -     Nixdorf Computers

32932   80A4-80B3   -      -     Siemens Gammasonics Inc.

32960   80C0-80C3   -      -     DCA Data Exchange Cluster

80C4                     Banyan Systems

80C5                     Banyan Systems

32966   80C6        -      -     Pacer Software

32967   80C7        -      -     Applitek Corporation

32968   80C8-80CC   -      -     Intergraph Corporation

32973   80CD-80CE   -      -     Harris Corporation

32975   80CF-80D2   -      -     Taylor Instrument

32979   80D3-80D4   -      -     Rosemount Corporation

32981   80D5        -      -     IBM SNA Service on Ether

32989   80DD        -      -     Varian Associates

32990   80DE-80DF   -      -     Integrated Solutions TRFS

32992   80E0-80E3   -      -     Allen-Bradley

32996   80E4-80F0   -      -     Datability

33010   80F2        -      -     Retix

33011   80F3        -      -     AppleTalk AARP (Kinetics)

33012   80F4-80F5   -      -     Kinetics

33015   80F7        -      -     Apollo Computer

33023   80FF-8103   -      -     Wellfleet Communications

33031   8107-8109   -      -     Symbolics Private

33072   8130        -      -     Hayes Microcomputers

33073   8131        -      -     VG Laboratory Systems

8132-8136                Bridge Communications

33079   8137-8138   -      -     Novell, Inc.

33081   8139-813D   -      -     KTI

8148                     Logicraft

8149                     Network Computing Devices

814A                     Alpha Micro

814C        -      -     SNMP

814D                     BIIN

814E                     BIIN

814F                     Technically Elite Concept

8150                     Rational Corp

8151-8153                Qualcomm

815C-815E                Computer Protocol Pty Ltd

8164-8166                Charles River Data System

817D-818C                Protocol Engines

818D                     Motorola Computer

819A-81A3                Qualcomm

81A4                     ARAI Bunkichi

81A5-81AE                RAD Network Devices

81B7-81B9                Xyplex

81CC-81D5                Apricot Computers

81D6-81DD                Artisoft

81E6-81EF                Polygon

81F0-81F2                Comsat Labs

81F3-81F5                SAIC

81F6-81F8                VG Analytical

8203-8205                Quantum Software

8221-8222                Ascom Banking Systems

823E-8240                Advanced Encryption System

827F-8282                Athena Programming

8263-826A                Charles River Data System

829A-829B                Inst Ind Info Tech

829C-82AB                Taurus Controls

82AC-8693                Walker Richer & Quinn

8694-869D                Idea Courier

869E-86A1                Computer Network Tech

86A3-86AC                Gateway Communications

86DB                     SECTRA

86DE                     Delta Controls

34543   86DF        -      -     ATOMIC

86E0-86EF                Landis & Gyr Powers

8700-8710                Motorola

8A96-8A97                Invisible Software

36864   9000        -      -     Loopback

36865   9001        -      -     3Com(Bridge) XNS Sys Mgmt

36866   9002        -      -     3Com(Bridge) TCP-IP Sys

36867   9003        -      -     3Com(Bridge) loop detect

65280   FF00        -      -     BBN VITAL-LanBridge cache

FF00-FF0F                ISC Bunker Ramo

- Ethernet Address Assignments

Ethernet addresses are often written in several different forms and displayed as 6 hexadecimal numbers:

080023AF54F7	08-00-23-AF-54-F7	08 00 23 AF 54 F7	08.00.23.AF.54.F7

This document contains:

- Vendor Addresses

- Ethernet Multicast Addresses

- Broadcast Address

For current up-to-date links contact : map-ne.com/Ethernet/

Vendor Addresses

The first 3 hexadecimal digits are assigned by the IEEE to vendors. This is a partial list of vendor Ethernet assigned MAC addresses.

00000C	Cisco

00000E	Fujitsu

00000F	NeXT

000010	Sytek

00001D	Cabletron

000020	DIAB (Data Intdustrier AB)

000022	Visual Technology

00002A	TRW

000032	GPT Limited (reassigned from GEC Computers Ltd)

00005A	S & Koch

00005E	IANA

000065	Network General

00006B	MIPS

000077	MIPS

00007A	Ardent

000089	Cayman Systems Gatorbox

000093	Proteon

00009F	Ameristar Technology

0000A2	Wellfleet

0000A3	Network Application Technology

0000A6	Network General (internal assignment, not for products)

0000A7	NCD X-terminals

0000A9	Network Systems

0000AA	Xerox	 Xerox machines

0000B3	CIMLinc

0000B7	Dove Fastnet

0000BC	Allen-Bradley

0000C0	Western Digital

0000C5	Farallon phone net card

0000C6	HP Intelligent Networks Operation (formerly Eon Systems)

0000C8	Altos

0000C9	Emulex Terminal Servers

0000D7	Dartmouth College (NED Router)

0000D8	3Com? Novell? PS/2

0000DD	Gould

0000DE	Unigraph

0000E2	Acer Counterpoint

0000EF	Alantec

0000FD	High Level Hardvare (Orion, UK)

000102	BBN internal usage (not registered)

0020AF	3COM ???

001700	Kabel

008064	Wyse Technology / Link Technologies

00802B	IMAC ???

00802D	Xylogics, Inc.	Annex terminal servers

00808C	Frontier Software Development

0080C2	IEEE 802.1 Committee

0080D3	Shiva

00AA00	Intel

00DD00	Ungermann-Bass

00DD01	Ungermann-Bass

020701	Racal InterLan

020406	BBN internal usage (not registered)

026086	Satelcom MegaPac (UK)

02608C	3Com	IBM PC; Imagen; Valid; Cisco

02CF1F	CMC	 Masscomp; Silicon Graphics; Prime EXL

080002	3Com (Formerly Bridge)

080003	ACC (Advanced Computer Communications)

080005	Symbolics LISP machines

080008	BBN

080009	Hewlett-Packard

08000A	Nestar Systems

08000B	Unisys

080011	Tektronix, Inc.

080014	Excelan  BBN Butterfly, Masscomp, Silicon Graphics

080017	NSC

08001A	Data General

08001B	Data General

08001E	Apollo

080020	Sun machines

080022	NBI

080025	CDC

080026	Norsk Data (Nord)

080027	PCS Computer Systems GmbH

080028	TI Explorer

08002B	DEC

08002E	Metaphor

08002F	Prime Computer	Prime 50-Series LHC300

080036	Intergraph CAE stations

080037	Fujitsu-Xerox

080038	Bull

080039	Spider Systems

080041	DCA Digital Comm. Assoc.

080045	???? (maybe Xylogics, but they claim not to know this number)

080046	Sony

080047	Sequent

080049	Univation

08004C	Encore

08004E	BICC

080056	Stanford University

080058	???	DECsystem-20

08005A	IBM

080067	Comdesign

080068	Ridge

080069	Silicon Graphics

08006E	Concurrent Masscomp

080075	DDE (Danish Data Elektronik A/S)

08007C	Vitalink TransLAN III

080080	XIOS

080086	Imagen/QMS

080087	Xyplex	 terminal servers

080089	Kinetics AppleTalk-Ethernet interface

08008B	Pyramid

08008D	XyVision machines

080090	Retix Inc Bridges

484453	HDS ???

800010	AT&T

AA0000 - 6	DEC obsolete

Ethernet Multicast Addresses

An Ethernet multicast address consists of the multicast bit, the 23-bit vendor component, and the 24-bit group identifier assigned by the vendor. For example, DEC is assigned the vendor component 08-00-2B, so multicast addresses assigned by DEC have the first 24-bits 09-00-2B (since the multicast bit is the low-order bit of the first byte, which is "the first bit on the wire").

Multicast Addresses:

MAC Address 		Type Field	Description

01-00-5E-00-00-00-      0800   	 	Internet Multicast        [RFC1112]

01-00-5E-7F-FF-FF

01-00-5E-80-00-00-      ????    	Internet reserved by IANA

01-00-5E-FF-FF-FF

01-80-C2-00-00-00       -802-   	Spanning tree (for bridges)

09-00-02-04-00-01?      8080?   	Vitalink printer

09-00-02-04-00-02?      8080?		Vitalink management

09-00-09-00-00-01       8005    	HP Probe

09-00-09-00-00-01       -802-   	HP Probe

09-00-09-00-00-04       8005?   	HP DTC

09-00-1E-00-00-00       8019?   	Apollo DOMAIN

09-00-2B-00-00-00       6009?   	DEC MUMPS?

09-00-2B-00-00-01       8039?   	DEC DSM/DTP?

09-00-2B-00-00-02       803B?   	DEC VAXELN?

09-00-2B-00-00-03       8038    	DEC Lanbridge Traffic Monitor (LTM)

09-00-2B-00-00-04       ????    	DEC MAP End System Hello

09-00-2B-00-00-05       ????    	DEC MAP Intermediate System Hello

09-00-2B-00-00-06       803D?		DEC CSMA/CD Encryption?

09-00-2B-00-00-07       8040?		DEC NetBios Emulator?

09-00-2B-00-00-0F       6004    	DEC Local Area Transport (LAT)

09-00-2B-00-00-1x       ????    	DEC Experimental

09-00-2B-01-00-01       8038    	DEC LanBridge Hello packets

09-00-2B-02-00-00       ????    	DEC DNA Lev. 2 Routing Layer routers?

09-00-2B-02-01-00       803C?   	DEC DNA Naming Service Advertisement?

09-00-2B-02-01-01       803C?   	DEC DNA Naming Service Solicitation?

09-00-2B-02-01-02       803E?   	DEC DNA Time Service?

09-00-2B-03-xx-xx       ????    	DEC default filtering by bridges?

09-00-2B-04-00-00       8041?   	DEC Local Area Sys. Transport (LAST)?

09-00-2B-23-00-00       803A?   	DEC Argonaut Console?

09-00-4E-00-00-02?      8137?   	Novell IPX

09-00-56-00-00-00-      ????    	Stanford reserved

09-00-56-FE-FF-FF

09-00-56-FF-00-00-      805C    	Stanford V Kernel, version 6.0

09-00-56-FF-FF-FF

09-00-77-00-00-01       ????    	Retix spanning tree bridges

09-00-7C-02-00-05       8080?   	Vitalink diagnostics

09-00-7C-05-00-01       8080?   	Vitalink gateway?

0D-1E-15-BA-DD-06       ????    	HP

AB-00-00-01-00-00       6001    	DEC Maintenance Operation Protocol

AB-00-00-02-00-00       6002    	DEC Maintenance Operation Protocol

AB-00-00-03-00-00       6003    	DECNET Phase IV end node Hello

AB-00-00-04-00-00       6003    	DECNET Phase IV Router Hello packets

AB-00-00-05-00-00       ????    	Reserved DEC through

AB-00-03-FF-FF-FF

AB-00-03-00-00-00       6004    	DEC Local Area Transport (LAT) - old

AB-00-04-00-xx-xx       ????    	Reserved DEC customer private use

AB-00-04-01-xx-yy       6007    	DEC Local Area VAX Cluster groups

CF-00-00-00-00-00       9000    	Ethernet Configuration Test protocol (Loopback)

Broadcast Address:

MAC Address 		Type Field	Description

FF-FF-FF-FF-FF-FF       0600    	XNS packets, Hello or gateway search?

FF-FF-FF-FF-FF-FF       0800    	IP (e.g. RWHOD via UDP) as needed

FF-FF-FF-FF-FF-FF       0804    	CHAOS

FF-FF-FF-FF-FF-FF       0806    	ARP (for IP and CHAOS) as needed

FF-FF-FF-FF-FF-FF       0BAD    	Banyan

FF-FF-FF-FF-FF-FF       1600    	VALID packets, Hello or gateway

FF-FF-FF-FF-FF-FF       8035    	Reverse ARP

FF-FF-FF-FF-FF-FF       807C    	Merit Internodal (INP)

FF-FF-FF-FF-FF-FF       809B    	EtherTalk

- Internet Protocol Address Space

The listed address blocks are for Class A IP addresses.

Address Block	Registry - Purpose			Date

000		IANA - Reserved				Sep 81

001		IANA - Reserved				Sep 81

002		IANA - Reserved				Sep 81

003		General Electric Company		May 94

004		Bolt Beranek and Newman Inc.		Dec 92

005		IANA - Reserved				Jul 95

006		Army Information Systems Center		Feb 94

007		IANA - Reserved				Apr 95

008		Bolt Beranek and Newman Inc.		Dec 92

009		IBM					Aug 92

010		IANA - Private Use			Jun 95

011		DoD Intel Information Systems		May 93

012		AT&T Bell Laboratories			Jun 95

013		Xerox Corporation			Sep 91

014		IANA - Public Data Network		Jun 91

015		Hewlett-Packard Company			Jul 94

016		Digital Equipment Corporation		Nov 94

017		Apple Computer Inc.			Jul 92

018		MIT					Jan 94

019		Ford Motor Company			May 95

020		Computer Sciences Corporation		Oct 94

021		DDN-RVN					Jul 91

022		Defense Information Systems Agency	May 93

023		IANA - Reserved				Jul 95

024		IANA - Cable Block			Jul 95

025		Royal Signals and Radar Establishment	Jan 95

026		Defense Information Systems Agency	May 95

027		IANA - Reserved				Apr 95

028		DSI-North				Jul 92

029		Defense Information Systems Agency	Jul 91

030		Defense Information Systems Agency	Jul 91

031		IANA - Reserved				Apr 99

032		Norsk Informasjonsteknologi		Jun 94

033		DLA Systems Automation Center		Jan 91

034		Halliburton Company			Mar 93

035		MERIT Computer Network			Apr 94

036		Stanford University			Apr 93

037		IANA - Reserved				Apr 95

038		Performance Systems International	Sep 94

039		IANA - Reserved				Apr 95

040		Eli Lily and Company			Jun 94

041		IANA - Reserved				May 95

042		IANA - Reserved				Jul 95

043		Japan Inet				Jan 91

044		Amateur Radio Digital Communications	Jul 92

045		Interop Show Network			Jan 95

046		Bolt Beranek and Newman Inc.		Dec 92

047		Bell-Northern Research			Jan 91

048		Prudential Securities Inc.		May 95

049		Joint Technical Command			May 94 Returned to IANA Mar 98

050		Joint Technical Command			May 94 Returned to IANA Mar 98

051		Deparment of Social Security of UK	Aug 94

052		E.I. duPont de Nemours and Co., Inc.	Dec 91

053		Cap Debis CCS				Oct 93

054		Merck and Co., Inc.			Mar 92

055		Boeing Computer Services		Apr 95

056		U.S. Postal Service			Jun 94

057		SITA					May 95

058		IANA - Reserved				Sep 81

059		IANA - Reserved				Sep 81

060		IANA - Reserved				Sep 81

061		APNIC - Pacific Rim			Apr 97

062		RIPE NCC - Europe			Apr 97

063		ARIN					Apr 97

064		ARIN					Jul 99

065-095		IANA - Reserved				Sep 81

096-126		IANA - Reserved				Sep 81

127		IANA - Reserved				Sep 81

128-191		Various Registries 			May 93

192		Various Registries - MultiRegional	May 93

193		RIPE NCC - Europe			May 93

194		RIPE NCC - Europe			May 93

195		RIPE NCC - Europe			May 93

196		Various Registries			May 93

197		IANA - Reserved				May 93

198		Various Registries 			May 93

199		ARIN - North America 			May 93

200		ARIN - Central and South America	May 93

201		Reserved - Central and South America	May 93

202		APNIC - Pacific Rim			May 93

203		APNIC - Pacific Rim			May 93

204		ARIN - North America 			Mar 94

205		ARIN - North America 			Mar 94

206		ARIN - North America 			Apr 95

207		ARIN - North America 			Nov 95

208		ARIN - North America 			Apr 96

209		ARIN - North America			Jun 96

210		APNIC - Pacific Rim			Jun 96

211		APNIC - Pacific Rim			Jun 96

212          	RIPE NCC - Europe			Oct 97

213  	     	RIPE NCC - Europe                       Mar 99

214		US-DOD					Mar 98

215		US-DOD 					Mar 98

216     	ARIN - North America			Apr 98

217     	IANA - Reserved				Sep 81

218-223       	IANA - Reserved				Sep 81

224-239		IANA - Multicast			Sep 81

240-255		IANA - Reserved				Sep 81

- Internet Multicast Addresses

IP address		Description

224.0.0.0		Base Address (Reserved)

224.0.0.1		All Systems on this Subnet

224.0.0.2		All Routers on this Subnet

224.0.0.3		Unassigned

224.0.0.4		DVMRP Routers

224.0.0.5		OSPFIGP All Routers

224.0.0.6		OSPFIGP Designated Routers

224.0.0.7		ST Routers

224.0.0.8		ST Hosts

224.0.0.9		RIP2 Routers

224.0.0.10 		IGRP Routers

224.0.0.11 		Mobile-Agents

224.0.0.12 		DHCP Server / Relay Agent

224.0.0.13 		All PIM Routers

224.0.0.14 		RSVP-ENCAPSULATION

224.0.0.15 		all-cbt-routers

224.0.0.16 		designated-sbm

224.0.0.17 		all-sbms

224.0.0.18 		VRRP

224.0.0.19-255 		Unassigned

224.0.1.0		VMTP Managers Group

224.0.1.1		NTP - Network Time Protocol

224.0.1.2		SGI-Dogfight

224.0.1.3		Rwhod

224.0.1.4		VNP

224.0.1.5		Artificial Horizons - Aviator

224.0.1.6		NSS - Name Service Server

224.0.1.7		AUDIONEWS -  Multicast

224.0.1.8		SUN NIS+ Information Service

224.0.1.9		MTP Multicast Transport Protocol

224.0.1.10 		IETF-1-LOW-AUDIO

224.0.1.11 		IETF-1-AUDIO

224.0.1.12 		IETF-1-VIDEO

224.0.1.13 		IETF-2-LOW-AUDIO

224.0.1.14 		IETF-2-AUDIO

224.0.1.15 		IETF-2-VIDEO

224.0.1.16 		MUSIC-SERVICE

224.0.1.17 		SEANET-TELEMETRY

224.0.1.18 		SEANET-IMAGE

224.0.1.19 		MLOADD

224.0.1.20 		any private experiment

224.0.1.21 		DVMRP on MOSPF

224.0.1.22 		SVRLOC

224.0.1.23 		XINGTV

224.0.1.24 		microsoft-ds

224.0.1.25 		nbc-pro

224.0.1.26 		nbc-pfn

224.0.1.27 		lmsc-calren-1

224.0.1.28 		lmsc-calren-2

224.0.1.29 		lmsc-calren-3

224.0.1.30 		lmsc-calren-4

224.0.1.31 		ampr-info

224.0.1.32 		mtrace

224.0.1.33 		RSVP-encap-1

224.0.1.34 		RSVP-encap-2

224.0.1.35 		SVRLOC-DA

224.0.1.36 		rln-server

224.0.1.37 		proshare-mc

224.0.1.38 		dantz

224.0.1.39 		cisco-rp-announce

224.0.1.40 		cisco-rp-discovery

224.0.1.41 		gatekeeper

224.0.1.42 		iberiagames

224.0.1.43 		nwn-discovery

224.0.1.44 		nwn-adaptor

224.0.1.45 		isma-1

224.0.1.46 		isma-2

224.0.1.47 		telerate

224.0.1.48 		ciena

224.0.1.49 		dcap-servers

224.0.1.50 		dcap-clients

224.0.1.51 		mcntp-directory

224.0.1.52 		mbone-vcr-directory

224.0.1.53 		heartbeat

224.0.1.54 		sun-mc-grp

224.0.1.55 		extended-sys

224.0.1.56 		pdrncs

224.0.1.57 		tns-adv-multi

224.0.1.58 		vcals-dmu

224.0.1.59 		zuba

224.0.1.60 		hp-device-disc

224.0.1.61 		tms-production

224.0.1.62 		sunscalar

224.0.1.63 		mmtp-poll

224.0.1.64 		compaq-peer

224.0.1.65 		iapp

224.0.1.66 		multihasc-com

224.0.1.67 		serv-discovery

224.0.1.68 		mdhcpdisover

224.0.1.69 		MMP-bundle-discovery1

224.0.1.70 		MMP-bundle-discovery2

224.0.1.71 		XYPOINT DGPS Data Feed

224.0.1.72 		GilatSkySurfer

224.0.1.73 		SharesLive

224.0.1.74 		NorthernData

224.0.1.75 		SIP

224.0.1.76 		IAPP

224.0.1.77 		AGENTVIEW

224.0.1.78 		Tibco Multicast1

224.0.1.79 		Tibco Multicast2

224.0.1.80 		MSP

224.0.1.81 		OTT (One-way Trip Time)

224.0.1.82 		TRACKTICKER

224.0.1.83 		dtn-mc

224.0.1.84 		jini-announcement

224.0.1.85 		jini-request

224.0.1.86 		sde-discovery

224.0.1.87 		DirecPC-SI

224.0.1.88 		B1RMonitor

224.0.1.89 		3Com-AMP3 dRMON

224.0.1.90 		imFtmSvc

224.0.1.91 		NQDS4

224.0.1.92 		NQDS5

224.0.1.93 		NQDS6

224.0.1.94 		NLVL12

224.0.1.95 		NTDS1

224.0.1.96 		NTDS2

224.0.1.97 		NODSA

224.0.1.98 		NODSB

224.0.1.99 		NODSC

224.0.1.100 		NODSD

224.0.1.101 		NQDS4R

224.0.1.102 		NQDS5R

224.0.1.103 		NQDS6R

224.0.1.104 		NLVL12R

224.0.1.105 		NTDS1R

224.0.1.106 		NTDS2R

224.0.1.107 		NODSAR

224.0.1.108 		NODSBR

224.0.1.109 		NODSCR

224.0.1.110 		NODSDR

224.0.1.111 		MRM

224.0.1.112 		TVE-FILE

224.0.1.113 		TVE-ANNOUNCE

224.0.1.114 		Mac Srv Loc

224.0.1.115 		Simple Multicast

224.0.1.116 		SpectraLinkGW

224.0.1.117 		dieboldmcast

224.0.1.118 		Tivoli Systems

224.0.1.119 		pq-lic-mcast

224.0.1.120 		HYPERFEED

224.0.1.121 		Pipesplatform

224.0.1.122 		LiebDevMgmg-DM

224.0.1.123 		TRIBALVOICE

224.0.1.124 		UDLR-DTCP

224.0.1.125 		PolyCom Relay1

224.0.1.126 		Infront Multi1

224.0.1.127 		XRX DEVICE DISC

224.0.1.128 		CNN

224.0.1.129 		PTP-primary

224.0.1.130 		PTP-alternate1

224.0.1.131 		PTP-alternate2

224.0.1.132 		PTP-alternate3

224.0.1.133 		ProCast

224.0.1.134 		3Com Discp

224.0.1.135 		CS-Multicasting

224.0.1.136 		TS-MC-1

224.0.1.137 		Make Source

224.0.1.138 		Teleborsa

224.0.1.139-255		Unassigned

224.0.2.1		"rwho" Group (BSD)

224.0.2.2		SUN RPC

224.0.2.064-095 	SIAC MDD Service

224.0.2.096-27 		CoolCast

224.0.2.128-191 	WOZ-Garage

224.0.2.192-255 	SIAC MDD Market Service

224.0.3.000-255 	RFE Generic Service

224.0.4.000-255 	RFE Individual Conferences

224.0.5.000-127 	CDPD Groups

224.0.5.128-191 	SIAC Market Service

224.0.5.192-255 	Unassigned

224.0.6.000-127 	Cornell ISIS Project

224.0.6.128-255 	Unassigned

224.0.7.000-255 	Where-Are-You

224.0.8.000-255 	INTV

224.0.9.000-255 	Invisible Worlds

224.0.10.000-255 	DLSw Groups

224.0.11.000-255	NCC.NEA Audio

224.0.12.000-063 	Microsoft and MSNBC

224.0.13.000-255 	UUNET PIPEX Net News

224.0.14.000-255 	NLANR

224.0.15.000-255 	Hewlett Packard

224.0.16.000-255 	XingNet

224.0.17.000-031 	Merc & Commodity Exchange

224.0.17.032-063 	NDQMD1

224.0.17.064-127 	ODN-DTV

224.0.18.000-255 	Dow Jones

224.0.19.000-063 	Walt Disney Company

224.0.19.064-095 	Cal Multicast

224.0.19.096-127 	SIAC Market Service

224.0.19.128-191 	IIG Multicast

224.0.19.192-207 	Metropol

224.0.19.208-239 	Xenoscience, Inc.

224.0.20.000-063 	MS-IP/TV

224.0.20.064-127 	Reliable Network Solutions

224.0.20.128-143 	TRACKTICKER Group

224.0.21.000-127 	Talarian MCAST

224.0.22.000-255 	WORLD MCAST

224.0.252.000-255 	Domain Scoped Group

224.0.253.000-255 	Report Group

224.0.254.000-255 	Query Group

224.0.255.000-255 	Border Routers

224.1.0.0-255		ST Multicast Groups

224.2.0.0-53		Multimedia Conference Calls

224.2.127.254		SAPv1 Announcements

224.2.127.255		SAPv0 Announce (deprecated)

224.2.128.0-255		SAP Dyn Assign

224.252.0.0-255.255 	DIS transient groups

225.0.0.0-255.255.255	MALLOC

232.0.0.0-255.255.255	VMTP trans groups

233.0.0.0-255.255.255	Static Allocations

239.0.0.0-255.255.255 	Admin Scoped

239.0.0.0-063.255.255 	Reserved

239.64.0.0-127.255.255	Reserved

239.128.0.0-191.255.255	Reserved

239.192.0.0-255.255 	Org-Local Scope

239.252.0.0-255.255 	Site-Local Scope

239.253.0.0-255.255 	Site-Local Scope

239.254.0.0-255.255 	Site-Local Scope

239.255.0.0-255.255 	Site-Local Scope

- IP Header Protocols

Decimal	Protocol	Description

0			Reserved

1	ICMP		Internet Control Message

2	IGMP		Internet Group Management

3	GGP		Gateway-to-Gateway

4	IP		IP in IP (encapsulation)

5	ST		Stream

6	TCP		Transmission Control

7	UCL		UCL

8	EGP		Exterior Gateway Protocol

9	IGP		any private interior gateway

10	BBN-RCC-MON 	BBN RCC Monitoring

11	NVP-II		Network Voice Protocol

12	PUP		PUP

13	ARGUS		ARGUS

14	EMCON		EMCON

15	XNET		Cross Net Debugger

16	CHAOS		Chaos

17	UDP		User Datagram

18	MUX		Multiplexing

19	DCN-MEAS	DCN Measurement Subsystems

20	HMP		Host Monitoring

21	PRM		Packet Radio Measurement

22	XNS-IDP		XEROX NS IDP

23	TRUNK-1		Trunk-1

24	TRUNK-2		Trunk-2

25	LEAF-1		Leaf-1

26	LEAF-2		Leaf-2

27	RDP		Reliable Data Protocol

28	IRTP		Internet Reliable Transaction

29	ISO-TP4		ISO Transport Protocol Class 4

30	NETBLT		Bulk Data Transfer Protocol

31	MFE-NSP		MFE Network Services Protocol

32	MERIT-INP	MERIT Internodal Protocol

33	SEP		Sequential Exchange Protocol

34	3PC		Third Party Connect Protocol

35	IDPR		Inter-Domain Policy Routing Protocol

36	XTP		XTP

37	DDP		Datagram Delivery Protocol

38	IDPR-CMTP	IDPR Control Message Transport Proto

39	TP++		TP++ Transport Protocol

40	IL		IL Transport Protocol

41	SIP		Simple Internet Protocol

42	SDRP		Source Demand Routing Protocol

43	SIP-SR		SIP Source Route

44	SIP-FRAG	SIP Fragment

45	IDRP		Inter-Domain Routing Protocol

46	RSVP		Reservation Protocol

47	GRE		General Routing Encapsulation

48	MHRP		Mobile Host Routing Protocol

49	BNA		BNA

50	SIPP-ESP	SIPP Encap Security Payload

51	SIPP-AH		SIPP Authentication Header

52	I-NLSP		Integrated Net Layer Security

53	SWIPE		IP with Encryption

54	NHRP		NBMA Next Hop Resolution Protocol

55-60			Unassigned

61			any host internal protocol

62	CFTP		CFTP

63			any local network

64	SAT-EXPAK	SATNET and Backroom EXPAK

65	KRYPTOLAN	Kryptolan

66	RVD		MIT Remote Virtual Disk Protocol

67	IPPC		Internet Pluribus Packet Core

68			any distributed file system

69	SAT-MON		SATNET Monitoring

70	VISA		VISA Protocol

71	IPCV		Internet Packet Core Utility

72	CPNX		Computer Protocol Network Executive

73	CPHB 		Computer Protocol Heart Beat

74	WSN		Wang Span Network

75	PVP		Packet Video Protocol

76	BR-SAT-MON	Backroom SATNET Monitoring

77	SUN-ND		SUN ND PROTOCOL-Temporary

78	WB-MON		WIDEBAND Monitoring

79	WB-EXPAK	WIDEBAND EXPAK

80	ISO-IP		ISO Internet Protocol

81	VMTP		VMTP

82	SECURE-VMTP	SECURE-VMTP

83	VINES		VINES

84	TTP		TTP

85	NSFNET-IGP	NSFNET-IGP

86	DGP		Dissimilar Gateway Protocol

87	TCF		TCF

88	IGRP		IGRP

89	OSPFIGP		OSPFIGP

90	Sprite-RPC	Sprite RPC Protocol

91	LARP		Locus Address Resolution Protocol

92	MTP		Multicast Transport Protocol

93	AX.25		AX.25 Frames

94	IPIP		IP-within-IP Encapsulation Protocol

95	MICP		Mobile Internetworking Control Pro.

96	SCC-SP		Semaphore Communications Sec. Pro.

97	ETHERIP		Ethernet-within-IP Encapsulation

98	ENCAP		Encapsulation Header

99			any private encryption scheme

100	GMTP		GMTP

101-254			Unassigned

255			Reserved

- IP Hardware Types

1	Ethernet (10Mb)

2	Experimental Ethernet (3Mb)

3 	Amateur Radio AX.25

4 	Proteon ProNET Token Ring

5 	Chaos

6 	IEEE 802 Networks

7 	ARCNET

8 	Hyperchannel

9 	Lanstar

10 	Autonet Short Address

11 	LocalTalk

12 	LocalNet (IBM PCNet or SYTEK LocalNET)

13 	Ultra link

14 	SMDS

15 	Frame Relay

16 	Asynchronous Transmission Mode (ATM)

17 	HDLC

18 	Fibre Channel

19 	Asynchronous Transmission Mode (ATM)

20 	Serial Line

21 	Asynchronous Transmission Mode (ATM)

- TCP/IP Well Known Ports

Keyword 		Decimal 	Description

0/tcp		Reserved

0/udp		Reserved

tcpmux			1/tcp		TCP Port Service Multiplexer

tcpmux			1/udp		TCP Port Service Multiplexer

compressnet		2/tcp		Management Utility

compressnet		2/udp		Management Utility

compressnet		3/tcp		Compression Process

compressnet		3/udp		Compression Process

#			4/tcp		Unassigned

#			4/udp		Unassigned

rje			5/tcp		Remote Job Entry

rje			5/udp		Remote Job Entry

#			6/tcp   	Unassigned

#	 		6/udp	 	Unassigned

echo			7/tcp    	Echo

echo			7/udp    	Echo

#       		8/tcp    	Unassigned

#       		8/udp    	Unassigned

discard           	9/tcp    	Discard

discard           	9/udp    	Discard

#                	10/tcp    	Unassigned

#               	10/udp    	Unassigned

systat           	11/tcp    	Active Users

systat           	11/udp    	Active Users

#                	12/tcp    	Unassigned

#               	12/udp    	Unassigned

daytime          	13/tcp    	Daytime

daytime          	13/udp    	Daytime

#                	14/tcp    	Unassigned

#                	14/udp    	Unassigned

#                	15/tcp    	Unassigned [was netstat]

#                	15/udp   	Unassigned

#                	16/tcp    	Unassigned

#                	16/udp   	Unassigned

qotd             	17/tcp    	Quote of the Day

qotd             	17/udp    	Quote of the Day

msp              	18/tcp    	Message Send Protocol

msp              	18/udp    	Message Send Protocol

chargen          	19/tcp    	Character Generator

chargen          	19/udp    	Character Generator

ftp-data         	20/tcp    	File Transfer [Default Data]

ftp-data         	20/udp    	File Transfer [Default Data]

ftp              	21/tcp    	File Transfer [Control]

ftp              	21/udp    	File Transfer [Control]

#                	22/tcp    	Unassigned

#               	22/udp    	Unassigned

telnet           	23/tcp    	Telnet

telnet           	23/udp    	Telnet

24/tcp    	any private mail system

24/udp    	any private mail system

smtp             	25/tcp    	Simple Mail Transfer

smtp             	25/udp    	Simple Mail Transfer

#                	26/tcp    	Unassigned

#               	26/udp    	Unassigned

nsw-fe           	27/tcp    	NSW User System FE

nsw-fe           	27/udp    	NSW User System FE

#                	28/tcp    	Unassigned

#                	28/udp    	Unassigned

msg-icp          	29/tcp    	MSG ICP

msg-icp          	29/udp    	MSG ICP

#                	30/tcp    	Unassigned

#               	30/udp    	Unassigned

msg-auth         	31/tcp    	MSG Authentication

msg-auth         	31/udp    	MSG Authentication

#                	32/tcp    	Unassigned

#                	32/udp    	Unassigned

dsp              	33/tcp    	Display Support Protocol

dsp              	33/udp    	Display Support Protocol

#                	34/tcp    	Unassigned

#                	34/udp    	Unassigned

35/tcp    	any private printer server

35/udp    	any private printer server

#                	36/tcp    	Unassigned

#                	36/udp    	Unassigned

time             	37/tcp    	Time

time             	37/udp    	Time

rap              	38/tcp    	Route Access Protocol

rap             	38/udp    	Route Access Protocol

rlp              	39/tcp    	Resource Location Protocol

rlp              	39/udp    	Resource Location Protocol

#               	40/tcp    	Unassigned

#                	40/udp    	Unassigned

graphics         	41/tcp    	Graphics

graphics         	41/udp    	Graphics

nameserver       	42/tcp    	Host Name Server

nameserver      	42/udp   	Host Name Server

nicname          	43/tcp    	Who Is

nicname          	43/udp    	Who Is

mpm-flags        	44/tcp    	MPM FLAGS Protocol

mpm-flags        	44/udp    	MPM FLAGS Protocol

mpm              	45/tcp    	Msg  Process Module [recv]

mpm              	45/udp   	Msg Process Module [recv]

mpm-snd          	46/tcp    	MPM [default send]

mpm-snd          	46/udp    	MPM [default send]

ni-ftp           	47/tcp    	NI FTP

ni-ftp           	47/udp    	NI FTP

auditd           	48/tcp    	Digital Audit Daemon

auditd           	48/udp    	Digital Audit Daemon

login            	49/tcp    	Login Host Protocol

login            	49/udp    	Login Host Protocol

re-mail-ck       	50/tcp    	Rem Mail Checking Protocol

re-mail-ck       	50/udp    	Rem Mail Checking Protocol

la-maint         	51/tcp    	IMP Logical Addr Maint

la-maint         	51/udp    	IMP Logical Addr Maint

xns-time         	52/tcp    	XNS Time Protocol

xns-time         	52/udp    	XNS Time Protocol

domain           	53/tcp    	Domain Name Server

domain           	53/udp    	Domain Name Server

xns-ch           	54/tcp    	XNS Clearinghouse

xns-ch           	54/udp    	XNS Clearinghouse

isi-gl           	55/tcp    	ISI Graphics Language

isi-gl           	55/udp    	ISI Graphics Language

xns-auth         	56/tcp    	XNS Authentication

xns-auth         	56/udp    	XNS Authentication

57/tcp    	any private terminal access

57/udp    	any private terminal access

xns-mail         	58/tcp    	XNS Mail

xns-mail         	58/udp    	XNS Mail

59/tcp    	any private file service

59/udp    	any private file service 			60/tcp    	Unassigned

60/udp    	Unassigned

ni-mail         	61/tcp    	NI MAIL

ni-mail          	61/udp    	NI MAIL

acas             	62/tcp    	ACA Services

acas             	62/udp    	ACA Services

#                	63/tcp    	Unassigned

#                	63/udp    	Unassigned

covia			64/tcp    	Comm Integrator (CI)

covia			64/udp    	Comm Integrator (CI)

tacacs-ds        	65/tcp    	TACACS-Database Service

tacacs-ds        	65/udp    	TACACS-Database Service

sql*net          	66/tcp    	Oracle SQL*NET

sql*net          	66/udp    	Oracle SQL*NET

bootps           	67/tcp    	Bootstrap Protocol Server

bootps           	67/udp    	Bootstrap Protocol Server

bootpc           	68/tcp    	Bootstrap Protocol Client

bootpc           	68/udp    	Bootstrap Protocol Client

tftp             	69/tcp    	Trivial File Transfer

tftp             	69/udp    	Trivial File Transfer

gopher           	70/tcp    	Gopher

gopher           	70/udp    	Gopher

netrjs-1         	71/tcp    	Remote Job Service

netrjs-1         	71/udp    	Remote Job Service

netrjs-2         	72/tcp    	Remote Job Service

netrjs-2         	72/udp    	Remote Job Service

netrjs-3         	73/tcp    	Remote Job Service

netrjs-3         	73/udp    	Remote Job Service

netrjs-4         	74/tcp    	Remote Job Service

netrjs-4         	74/udp    	Remote Job Service

75/tcp    	any private dial out service

75/udp    	any private dial out service

deos             	76/tcp    	Distributed Ext Object Store

deos             	76/udp    	Distributed Ext Object Store

77/tcp    	any private RJE service

77/udp    	any private RJE service

vettcp           	78/tcp    	vettcp

vettcp           	78/udp    	vettcp

finger           	79/tcp    	Finger

finger           	79/udp    	Finger

www-http         	80/tcp    	World Wide Web HTTP

www-http         	80/udp    	World Wide Web HTTP

hosts2-ns        	81/tcp    	HOSTS2 Name Server

hosts2-ns        	81/udp    	HOSTS2 Name Server

xfer             	82/tcp    	XFER Utility

xfer             	82/udp    	XFER Utility

mit-ml-dev       	83/tcp    	MIT ML Device

mit-ml-dev       	83/udp    	MIT ML Device

ctf              	84/tcp    	Common Trace Facility

ctf              	84/udp    	Common Trace Facility

mit-ml-dev       	85/tcp    	MIT ML Device

mit-ml-dev       	85/udp    	MIT ML Device

mfcobol          	86/tcp    	Micro Focus Cobol

mfcobol          	86/udp    	Micro Focus Cobol

87/tcp    	any private terminal link

87/udp    	any private terminal link

kerberos         	88/tcp    	Kerberos

kerberos         	88/udp    	Kerberos

su-mit-tg        	89/tcp    	SU/MIT Telnet Gateway

su-mit-tg        	89/udp    	SU/MIT Telnet Gateway

dnsix            	90/tcp    	DNSIX Securit Attrib Tok Map

dnsix            	90/udp    	DNSIX Securit Attrib Tok Map

mit-dov          	91/tcp    	MIT Dover Spooler

mit-dov          	91/udp    	MIT Dover Spooler

npp              	92/tcp    	Network Printing Protocol

npp              	92/udp    	Network Printing Protocol

dcp              	93/tcp    	Device Control Protocol

dcp              	93/udp    	Device Control Protocol

objcall          	94/tcp    	Tivoli Object Dispatcher

objcall          	94/udp    	Tivoli Object Dispatcher

supdup           	95/tcp    	SUPDUP

supdup           	95/udp    	SUPDUP

dixie            	96/tcp    	DIXIE Protocol Specification

dixie            	96/udp    	DIXIE Protocol Specification

swift-rvf        	97/tcp    	Swift Rem Vitural File Protocol

swift-rvf        	97/udp   	Swift Rem Vitural File Protocol

tacnews          	98/tcp    	TAC News

tacnews          	98/udp    	TAC News

metagram         	99/tcp    	Metagram Relay

metagram         	99/udp    	Metagram Relay

newacct         	100/tcp    	[unauthorized use]

hostname        	101/tcp    	NIC Host Name Server

hostname        	101/udp    	NIC Host Name Server

iso-tsap        	102/tcp    	ISO-TSAP

iso-tsap        	102/udp    	ISO-TSAP

gppitnp         	103/tcp    	Genesis Pt-to-Pt Trans Net

gppitnp         	103/udp    	Genesis Pt-to-Pt Trans Net

acr-nema        	104/tcp    	ACR-NEMA

acr-nema        	104/udp    	ACR-NEMA

csnet-ns        	105/tcp    	Mailbox Name Nameserver

csnet-ns        	105/udp    	Mailbox Name Nameserver

3com-tsmux     		106/tcp    	3COM-TSMUX

3com-tsmux     		106/udp    	3COM-TSMUX

rtelnet         	107/tcp    	Remote Telnet Service

rtelnet         	107/udp    	Remote Telnet Service

snagas          	108/tcp    	SNA Gateway Access Server

snagas          	108/udp    	SNA Gateway Access Server

pop2            	109/tcp    	Post Office Protocol - Ver 2

pop2            	109/udp    	Post Office Protocol - Ver 2

pop3            	110/tcp    	Post Office Protocol - Ver 3

pop3            	110/udp    	Post Office Protocol - Ver 3

sunrpc          	111/tcp    	SUN Rem Procedure Call

sunrpc          	111/udp    	SUN Rem Procedure Call

mcidas          	112/tcp    	McIDAS Data Trans Protocol

mcidas          	112/udp    	McIDAS Data Trans Protocol

auth            	113/tcp    	Authentication Service

auth            	113/udp    	Authentication Service

audionews       	114/tcp    	Audio News Multicast

audionews       	114/udp    	Audio News Multicast

sftp            	115/tcp    	Simple File Transfer Protocol

sftp            	115/udp    	Simple File Transfer Protocol

ansanotify      	116/tcp    	ANSA REX Notify

ansanotify      	116/udp    	ANSA REX Notify

uucp-path       	117/tcp    	UUCP Path Service

uucp-path       	117/udp    	UUCP Path Service

sqlserv         	118/tcp    	SQL Services

sqlserv         	118/udp    	SQL Services

nntp            	119/tcp    	Network News Transfer

nntp            	119/udp    	Network News Transfer

cfdptkt         	120/tcp    	CFDPTKT

cfdptkt         	120/udp    	CFDPTKT

erpc            	121/tcp    	Encore Expedited RPC

erpc            	121/udp    	Encore Expedited RPC

smakynet        	122/tcp    	SMAKYNET

smakynet        	122/udp    	SMAKYNET

ntp             	123/tcp    	Network Time Protocol

ntp             	123/udp    	Network Time Protocol

ansatrader      	124/tcp    	ANSA REX Trader

ansatrader      	124/udp    	ANSA REX Trader

locus-map       	125/tcp    	Locus PC-Inter Net Map Ser

locus-map       	125/udp    	Locus PC-Inter Net Map Ser

unitary         	126/tcp    	Unisys Unitary Login

unitary         	126/udp    	Unisys Unitary Login

locus-con       	127/tcp    	Locus PC-Inter Conn Server

locus-con       	127/udp    	Locus PC-Inter Conn Server

gss-xlicen      	128/tcp    	GSS X License Verification

gss-xlicen      	128/udp    	GSS X License Verification

pwdgen          	129/tcp    	Password Generator Protocol

pwdgen          	129/udp    	Password Generator Protocol

cisco-fna       	130/tcp    	cisco FNATIVE

cisco-fna       	130/udp    	cisco FNATIVE

cisco-tna       	131/tcp    	cisco TNATIVE

cisco-tna       	131/udp    	cisco TNATIVE

cisco-sys       	132/tcp    	cisco SYSMAINT

cisco-sys       	132/udp    	cisco SYSMAINT

statsrv         	133/tcp    	Statistics Service

statsrv         	133/udp    	Statistics Service

ingres-net      	134/tcp    	INGRES-NET Service

ingres-net      	134/udp    	INGRES-NET Service

loc-srv         	135/tcp    	Location Service

loc-srv         	135/udp    	Location Service

profile         	136/tcp    	ROFILE Naming System

profile         	136/udp    	PROFILE Naming System

netbios-ns      	137/tcp    	NETBIOS Name Service

netbios-ns      	137/udp    	NETBIOS Name Service

netbios-dgm     	138/tcp    	NETBIOS Datagram Service

netbios-dgm     	138/udp    	NETBIOS Datagram Service

netbios-ssn     	139/tcp    	NETBIOS Session Service

netbios-ssn     	139/udp    	NETBIOS Session Service

emfis-data      	140/tcp    	EMFIS Data Service

emfis-data      	140/udp    	EMFIS Data Service

emfis-cntl      	141/tcp    	EMFIS Control Service

emfis-cntl      	141/udp    	EMFIS Control Service

bl-idm          	142/tcp    	Britton-Lee IDM

bl-idm          	142/udp    	Britton-Lee IDM

imap2           	143/tcp   	Interim Mail Access  v2

imap2           	143/udp    	Interim Mail Access  v2

news            	144/tcp    	NewS

news            	144/udp    	NewS

uaac            	145/tcp    	UAAC Protocol

uaac            	145/udp    	UAAC Protocol

iso-tp0         	146/tcp    	ISO-IP0

iso-tp0         	146/udp    	ISO-IP0

iso-ip          	147/tcp    	ISO-IP

iso-ip          	147/udp    	ISO-IP

cronus          	148/tcp    	CRONUS-SUPPORT

cronus          	148/udp    	CRONUS-SUPPORT

aed-512         	149/tcp    	AED 512 Emulation Service

aed-512         	149/udp    	AED 512 Emulation Service

sql-net         	150/tcp    	SQL-NET

sql-net         	150/udp    	SQL-NET

hems            	151/tcp    	HEMS

hems            	151/udp    	HEMS

bftp            	152/tcp    	Background ftp

bftp            	152/udp    	Background ftp

sgmp            	153/tcp   	SGMP

sgmp            	153/udp    	SGMP

netsc-prod      	154/tcp    	NETSC

netsc-prod      	154/udp    	NETSC

netsc-dev       	155/tcp    	NETSC

netsc-dev       	155/udp    	NETSC

sqlsrv          	156/tcp    	SQL Service

sqlsrv          	156/udp    	SQL Service

knet-cmp        	157/tcp    	KNET/VM Com/Msg Protocol

knet-cmp        	157/udp    	KNET/VM Com/Msg Protocol

pcmail-srv      	158/tcp    	PCMail Server

pcmail-srv      	158/udp    	PCMail Server

nss-routing     	159/tcp    	NSS-Routing

nss-routing     	159/udp    	NSS-Routing

sgmp-traps      	160/tcp    	SGMP-TRAPS

sgmp-traps      	160/udp    	SGMP-TRAPS

snmp            	161/tcp    	SNMP

snmp            	161/udp    	SNMP

snmptrap        	162/tcp    	SNMPTRAP

snmptrap        	162/udp    	SNMPTRAP

cmip-man        	163/tcp    	CMIP/TCP Manager

cmip-man        	163/udp    	CMIP/TCP Manager

cmip-agent      	164/tcp    	CMIP/TCP Agent

smip-agent      	164/udp    	CMIP/TCP Agent

xns-courier     	165/tcp    	Xerox

xns-courier     	165/udp    	Xerox

s-net           	166/tcp    	Sirius Systems

s-net           	166/udp    	Sirius Systems

namp            	167/tcp    	NAMP

namp            	167/udp    	NAMP

rsvd            	168/tcp    	RSVD

rsvd            	168/udp    	RSVD

send            	169/tcp    	SEND

send            	169/udp    	SEND

print-srv       	170/tcp    	Network PostScript

print-srv       	170/udp    	Network PostScript

multiplex       	171/tcp    	Network Innovations Multiplex

multiplex       	171/udp    	Network Innovations Multiplex

cl/1            	172/tcp    	Network Innovations CL/1

cl/1            	172/udp    	Network Innovations CL/1

xyplex-mux      	173/tcp    	Xyplex

xyplex-mux      	173/udp    	Xyplex

mailq           	174/tcp    	MAILQ

mailq           	174/udp    	MAILQ

vmnet           	175/tcp    	VMNET

vmnet           	175/udp    	VMNET

genrad-mux 		176/tcp    	GENRAD-MUX

genrad-mux		176/udp    	GENRAD-MUX

xdmcp           	177/tcp    	X Display Manager Control

xdmcp           	177/udp    	X Display Manager Control

nextstep        	178/tcp    	NextStep Window Server

NextStep        	178/udp    	NextStep Window Server

bgp             	179/tcp    	Border Gateway Protocol

bgp             	179/udp    	Border Gateway Protocol

ris             	180/tcp    	Intergraph

ris             	180/udp    	Intergraph

unify           	181/tcp    	Unify

unify           	181/udp    	Unify

audit           	182/tcp    	Unisys Audit SITP

audit           	182/udp    	Unisys Audit SITP

ocbinder        	183/tcp    	OCBinder

ocbinder        	183/udp    	OCBinder

ocserver        	184/tcp    	OCServer

ocserver        	184/udp    	OCServer

remote-kis      	185/tcp    	Remote-KIS

remote-kis      	185/udp    	Remote-KIS

kis             	186/tcp    	KIS Protocol

kis             	186/udp    	KIS Protocol

aci             	187/tcp    	Application Comm Interface

aci             	187/udp    	Application Com Interface

mumps           	188/tcp    	Plus Five's MUMPS

mumps           	188/udp    	Plus Five's MUMPS

qft             	189/tcp    	Queued File Transport

qft             	189/udp    	Queued File Transport

gacp            	190/tcp    	Gateway Access Ctrl Protocol

cacp            	190/udp   	Gateway Access Ctrl Protocol

prospero        	191/tcp    	Prospero Directory Service

prospero        	191/udp    	Prospero Directory Service

osu-nms         	192/tcp    	OSU Network Mon System

osu-nms         	192/udp    	OSU Network Mon System

srmp            	193/tcp    	Spider Rem Mon Protocol

srmp            	193/udp    	Spider Rem Mon Protocol

irc             	194/tcp    	Internet Relay Chat Protocol

irc             	194/udp    	Internet Relay Chat Protocol

dn6-nlm-aud 		195/tcp    	DNSIX Net Level Mod Audit

dn6-nlm-aud 		195/udp    	DNSIX Net Level Mod Audit

dn6-smm-red 		196/tcp    	DNSIX Sess Mgt Mod Aud Red

dn6-smm-red 		196/udp    	DNSIX Sess Mgt Mod Aud Red

dls             	197/tcp    	Directory Location Service

dls             	197/udp    	Directory Location Service

dls-mon         	198/tcp    	Directory Location Service Mon

dls-mon         	198/udp    	Directory Location Service Mon

smux            	199/tcp    	SMUX

smux            	199/udp    	SMUX

src             	200/tcp    	IBM Sys Resource Controller

src             	200/udp    	IBM Sys Resource Controller

at-rtmp         	201/tcp    	AppleTalk Routing Maint

at-rtmp         	201/udp    	AppleTalk Routing Maint

at-nbp          	202/tcp    	AppleTalk Name Binding

at-nbp          	202/udp    	AppleTalk Name Binding

at-3            	203/tcp    	AppleTalk Unused

at-3            	203/udp    	AppleTalk Unused

at-echo         	204/tcp    	AppleTalk Echo

at-echo         	204/udp    	AppleTalk Echo

at-5            	205/tcp    	AppleTalk Unused

at-5            	205/udp    	AppleTalk Unused

at-zis          	206/tcp    	AppleTalk Zone Information

at-zis          	206/udp    	AppleTalk Zone Information

at-7            	207/tcp    	AppleTalk Unused

at-7            	207/udp    	AppleTalk Unused

at-8            	208/tcp    	AppleTalk Unused

at-8            	208/udp    	AppleTalk Unused

tam             	209/tcp    	Trivial Auth Mail Protocol

tam             	209/udp   	Trivial Auth Mail Protocol

z39.50          	210/tcp    	ANSI Z39.50

z39.50          	210/udp    	ANSI Z39.50

914c/g          	211/tcp    	TI 914C/G Terminal

914c/g         	 	211/udp    	TI 914C/G Terminal

anet            	212/tcp    	ATEXSSTR

anet            	212/udp    	ATEXSSTR

ipx             	213/tcp    	IPX

ipx             	213/udp    	IPX

vmpwscs         	214/tcp    	VM PWSCS

vmpwscs         	214/udp    	VM PWSCS

softpc          	215/tcp    	Insignia Solutions

softpc          	215/udp    	Insignia Solutions

atls            	216/tcp    	Access Tech License Server

atls            	216/udp    	Access Tech License Server

dbase           	217/tcp    	dBASE Unix

dbase           	217/udp    	dBASE Unix

mpp             	218/tcp    	Netix Message Posting Protocol

mpp             	218/udp    	Netix Message Posting Protocol

uarps           	219/tcp    	Unisys ARPs

uarps           	219/udp    	Unisys ARPs

imap3           	220/tcp    	IMAP3

imap3           	220/udp    	IMAP3

fln-spx         	221/tcp    	Berkeley rlogind with SPX auth

fln-spx        		221/udp    	Berkeley rlogind with SPX auth

rsh-spx        		222/tcp    	Berkeley rshd with SPX auth

rsh-spx         	222/udp    	Berkeley rshd with SPX auth

cdc             	223/tcp    	Certificate Distribution Center

cdc             	223/udp    	Certificate Distribution Center

#               	224-241    	Reserved

#              		242/tcp    	Unassigned

#              		242/udp    	Unassigned

sur-meas        	243/tcp    	Survey Measurement

sur-meas        	243/udp    	Survey Measurement

#               	244/tcp    	Unassigned

#               	244/udp    	Unassigned

link            	245/tcp    	LINK

link            	245/udp    	LINK

dsp3270         	246/tcp    	Display Systems Protocol

dsp3270         	246/udp    	Display Systems Protocol

#               	247-255    	Reserved

#               	256-343    	Unassigned

pdap            	344/tcp    	Prospero Data Access Protocol

pdap            	344/udp    	Prospero Data Access Protocol

pawserv         	345/tcp    	Perf Analysis Workbench

pawserv        		345/udp    	Perf Analysis Workbench

zserv           	346/tcp    	Zebra server

zserv           	346/udp    	Zebra server

fatserv         	347/tcp    	Fatmen Server

fatserv         	347/udp    	Fatmen Server

csi-sgwp        	348/tcp    	Cabletron Mgnt Protocol

csi-sgwp        	348/udp    	Cabletron Mgnt Protocol

#               	349-370    	Unassigned

clearcase       	371/tcp    	Clearcase

clearcase       	371/udp    	Clearcase

ulistserv       	372/tcp    	Unix Listserv

ulistserv       	372/udp    	Unix Listserv

legent-1        	373/tcp    	Legent Corporation

legent-1        	373/udp    	Legent Corporation

legent-2        	374/tcp    	Legent Corporation

legent-2        	374/udp    	Legent Corporation

hassle          	375/tcp    	Hassle

hassle          	375/udp    	Hassle

nip             	376/tcp    	Amiga Envoy Net Inquiry Proto

nip             	376/udp    	Amiga Envoy Net Inquiry Proto

tnETOS          	377/tcp    	NEC Corporation

tnETOS          	377/udp    	NEC Corporation

dsETOS          	378/tcp    	NEC Corporation

dsETOS          	378/udp    	NEC Corporation

is99c           	379/tcp    	TIA/EIA/IS-99 modem client

is99c           	379/udp    	TIA/EIA/IS-99 modem client

is99s           	380/tcp    	TIA/EIA/IS-99 modem server

is99s           	380/udp    	TIA/EIA/IS-99 modem server

hp-collector    	381/tcp    	hp perf data collector

hp-collector    	381/udp    	hp perf data collector

hp-mngd-node 		382/tcp    	hp perf data managed node

hp-mngd-node 		382/udp    	hp perf data managed node

hp-alarm-mgr		383/tcp    	hp perf data alarm manager

hp-alarm-mgr		383/udp    	hp perf data alarm manager

arns            	384/tcp    	A Rem Network Server System

arns            	384/udp    	A Rem Network Server System

ibm-app         	385/tcp   	IBM Application

ibm-app         	385/udp    	IBM Application

asa             	386/tcp    	ASA Message Rout Object Def.

asa             	386/udp    	ASA Message Rout Object Def.

aurp            	387/tcp    	Apple Update-Based Rout Pro.

aurp            	387/udp    	Apple Update-Based Rout Pro.

unidata-ldm     	388/tcp    	Unidata LDM Version 4

unidata-ldm     	388/udp    	Unidata LDM Version 4

ldap            	389/tcp   	Lightweight Dir Access Prot

ldap            	389/udp    	Lightweight Diry Access Prot

uis             	390/tcp    	UIS

uis             	390/udp    	UIS

synotics-relay  	391/tcp    	SynOptics SNMP Relay Port

synotics-relay  	391/udp    	SynOptics SNMP Relay Port

synotics-broker 	392/tcp    	SynOptics Port Broker Port

synotics-broker 	392/udp    	SynOptics Port Broker Port

dis             	393/tcp    	Data Interpretation System

dis             	393/udp    	Data Interpretation System

embl-ndt        	394/tcp    	EMBL Nucleic Data Transfer

embl-ndt        	394/udp    	EMBL Nucleic Data Transfer

netcp           	395/tcp    	NETscout Control Protocol

netcp           	395/udp    	NETscout Control Protocol

netware-ip      	396/tcp    	Novell Netware over IP

netware-ip      	396/udp    	Novell Netware over IP

mptn            	397/tcp    	Multi Protocol Trans. Net.

mptn            	397/udp    	Multi Protocol Trans. Net.

kryptolan       	398/tcp    	Kryptolan

kryptolan       	398/udp    	Kryptolan

#               	399/tcp    	Unassigned

#               	399/udp    	Unassigned

work-sol       	 	400/tcp    	Workstation Solutions

work-sol        	400/udp    	Workstation Solutions

ups             	401/tcp    	Uninterruptible Power Supply

ups             	401/udp    	Uninterruptible Power Supply

genie           	402/tcp    	Genie Protocol

genie           	402/udp    	Genie Protocol

decap           	403/tcp    	decap

decap           	403/udp    	decap

nced            	404/tcp    	nced

nced            	404/udp    	nced

ncld            	405/tcp    	ncld

ncld            	405/udp    	ncld

imsp            	406/tcp    	Interactive Mail Support Prot

imsp            	406/udp    	Interactive Mail Support Prot

timbuktu        	407/tcp    	Timbuktu

timbuktu        	407/udp    	Timbuktu

prm-sm          	408/tcp    	Prospero Res Mgr Sys. Man.

prm-sm          	408/udp    	Prospero Res Mgr Sys. Man.

prm-nm          	409/tcp    	Prospero Res Mgr Node Man.

prm-nm          	409/udp    	Prospero Res Mgr Node Man.

decladebug		410/tcp    	DECLadebug Rem Debug Prot

decladebug		410/udp    	DECLadebug Rem Debug Prot

rmt             	411/tcp    	Remote MT Protocol

rmt             	411/udp    	Remote MT Protocol

synoptics-trap  	412/tcp    	Trap Convention Port

synoptics-trap  	412/udp    	Trap Convention Port

smsp            	413/tcp    	SMSP

smsp            	413/udp    	SMSP

infoseek        	414/tcp    	InfoSeek

infoseek        	414/udp    	InfoSeek

bnet            	415/tcp    	BNet

bnet            	415/udp    	BNet

silverplatter   	416/tcp    	Silverplatter

silverplatter   	416/udp    	Silverplatter

onmux           	417/tcp    	Onmux

onmux           	417/udp    	Onmux

hyper-g         	418/tcp    	Hyper-G

hyper-g         	418/udp    	Hyper-G

ariel1          	419/tcp    	Ariel

ariel1          	419/udp    	Ariel

smpte           	420/tcp    	SMPTE

smpte           	420/udp    	SMPTE

ariel2          	421/tcp    	Ariel

ariel2          	421/udp    	Ariel

ariel3          	422/tcp    	Ariel

ariel3          	422/udp    	Ariel

opc-job-start   	423/tcp  	IBM Op Plan & Control Start

opc-job-start   	423/udp    	IBM Op Plan & Control Start

opc-job-track   	424/tcp    	IBM Op Plan & Control Track

opc-job-track   	424/udp   	IBM Op Plan & Control Track

icad-el         	425/tcp    	ICAD

icad-el         	425/udp    	ICAD

smartsdp        	426/tcp    	smartsdp

smartsdp        	426/udp    	smartsdp

svrloc          	427/tcp    	Server Location

svrloc          	427/udp    	Server Location

ocs_cmu         	428/tcp    	OCS_CMU

ocs_cmu         	428/udp    	OCS_CMU

ocs_amu         	429/tcp    	OCS_AMU

ocs_amu         	429/udp    	OCS_AMU

utmpsd          	430/tcp    	UTMPSD

utmpsd          	430/udp    	UTMPSD

utmpcd          	431/tcp    	UTMPCD

utmpcd          	431/udp    	UTMPCD

iasd            	432/tcp    	IASD

iasd            	432/udp    	IASD

nnsp            	433/tcp    	NNSP

nnsp            	433/udp    	NNSP

mobileip-agent		434/tcp    	MobileIP-Agent

mobileip-agent		434/udp    	MobileIP-Agent

mobilip-mn 		435/tcp    	MobilIP-MN

mobilip-mn 		435/udp    	MobilIP-MN

dna-cml         	436/tcp    	DNA-CML

dna-cml         	436/udp    	DNA-CML

comscm          	437/tcp    	comscm

comscm          	437/udp    	comscm

dsfgw           	438/tcp    	dsfgw

dsfgw           	438/udp    	dsfgw

dasp            	439/tcp    	dasp

dasp            	439/udp    	dasp

sgcp            	440/tcp    	sgcp

sgcp            	440/udp    	sgcp

vms-sysmgt   		441/tcp    	decvms-sysmgt

vms-sysmgt   		441/udp    	decvms-sysmgt

cvc_hostd       	442/tcp    	cvc_hostd

cvc_hostd       	442/udp    	cvc_hostd

https           	443/tcp    	https  MCom

https           	443/udp    	https  MCom

snpp            	444/tcp    	Simple Net Paging Protocol

snpp            	444/udp    	Simple Net Paging Protocol

microsoft-ds    	445/tcp    	Microsoft-DS

microsoft-ds    	445/udp    	Microsoft-DS

ddm-rdb         	446/tcp    	DDM-RDB

ddm-rdb         	446/udp    	DDM-RDB

ddm-dfm         	447/tcp    	DDM-RFM

ddm-dfm         	447/udp    	DDM-RFM

ddm-byte        	448/tcp    	DDM-BYTE

ddm-byte        	448/udp    	DDM-BYTE

as-servermap		449/tcp    	AS Server Mapper

as-servermap		449/udp    	AS Server Mapper

tserver         	450/tcp    	TServer

tserver         	450/udp    	TServer

#               	451-511    	Unassigned

exec            	512/tcp    	remote process execution;

biff            	512/udp    	mail system notification

login           	513/tcp    	remote login a la telnet;

who             	513/udp    	shows who's logged in

cmd             	514/tcp    	like exec, but auto auth

syslog          	514/udp

printer         	515/tcp    	spooler

printer         	515/udp    	spooler

#               	516/tcp    	Unassigned

#               	516/udp    	Unassigned

talk            	517/tcp    	talk

talk            	517/udp    	talk

ntalk           	518/tcp

ntalk           	518/udp

utime           	519/tcp    	unixtime

utime           	519/udp    	unixtime

efs             	520/tcp    	extended file name server

router          	520/udp    	local routing process (on site);

#               	521-524    	Unassigned

timed           	525/tcp    	timeserver

timed           	525/udp    	timeserver

tempo           	526/tcp    	newdate

tempo           	526/udp    	newdate

#               	527-529    	Unassigned

courier         	530/tcp    	rpc

courier         	530/udp    	rpc

conference      	531/tcp    	chat

conference      	531/udp    	chat

netnews         	532/tcp    	readnews

netnews         	532/udp    	readnews

netwall         	533/tcp    	for emergency broadcasts

netwall         	533/udp    	for emergency broadcasts

#               	534-538    	Unassigned

apertus-ldp     	539/tcp    	Apertus Tech Load Determ

apertus-ldp     	539/udp    	Apertus Tech Load Determ

uucp            	540/tcp    	uucpd

uucp            	540/udp    	uucpd

uucp-rlogin     	541/tcp    	uucp-rlogin

uucp-rlogin     	541/udp    	uucp-rlogin

#               	542/tcp    	Unassigned

#               	542/udp    	Unassigned

klogin          	543/tcp

klogin          	543/udp

kshell          	544/tcp    	krcmd

kshell          	544/udp    	krcmd

#               	545-549    	Unassigned

new-rwho		550/tcp    	new-who

new-rwho 		550/udp    	new-who

#               	551-555    	Unassigned

dsf             	555/tcp

dsf             	555/udp

remotefs        	556/tcp    	rfs server

remotefs        	556/udp    	rfs server

#               	557-559    	Unassigned

rmonitor        	560/tcp    	rmonitord

rmonitor        	560/udp    	rmonitord

monitor         	561/tcp

monitor         	561/udp

chshell         	562/tcp    	chcmd

chshell         	562/udp    	chcmd

#               	563/tcp    	Unassigned

#               	563/udp    	Unassigned

9pfs            	564/tcp    	plan 9 file service

9pfs            	564/udp    	plan 9 file service

whoami          	565/tcp    	whoami

whoami          	565/udp    	whoami

#               	566-569    	Unassigned

meter           	570/tcp    	demon

meter           	570/udp    	demon

meter           	571/tcp    	udemon

meter           	571/udp    	udemon

#               	572-599    	Unassigned

ipcserver       	600/tcp    	Sun IPC server

ipcserver       	600/udp    	Sun IPC server

nqs             	607/tcp    	nqs

nqs             	607/udp    	nqs

urm             	606/tcp    	Cray Unified Resource Manager

urm             	606/udp    	Cray Unified Resource Manager

sift-uft        	608/tcp    	Sender-Init/Unsol File Trans

sift-uft        	608/udp    	Sender-Init/Unsol File Trans

npmp-trap       	609/tcp    	npmp-trap

npmp-trap       	609/udp    	npmp-trap

npmp-local      	610/tcp    	npmp-local

npmp-local      	610/udp    	npmp-local

npmp-gui        	611/tcp    	npmp-gui

npmp-gui        	611/udp    	npmp-gui

ginad           	634/tcp    	ginad

ginad           	634/udp    	ginad

mdqs            	666/tcp

mdqs           		666/udp

doom            	666/tcp    	doom Id Software

doom           		666/udp    	doom Id Software

elcsd           	704/tcp    	errlog copy/server daemon

elcsd           	704/udp    	errlog copy/server daemon

entrustmgr 		709/tcp    	EntrustManager

entrustmgr  		709/udp    	EntrustManager

netviewdm1		729/tcp    	IBM NV DM/6000 Srvr/Client

netviewdm1		729/udp    	IBM NV DM/6000 Srvr/Client

netviewdm2		730/tcp    	IBM NV DM/6000 send/tcp

netviewdm2		730/udp    	IBM NV DM/6000 send/tcp

netviewdm3		731/tcp    	IBM NV DM/6000 receive/tcp

netviewdm3		731/udp    	IBM NV DM/6000 receive/tcp

netgw           	741/tcp    	netGW

netgw           	741/udp    	netGW

netrcs          	742/tcp    	Network based Rev. Cont. Sys.

netrcs          	742/udp    	Network based Rev. Cont. Sys.

flexlm         		744/tcp    	Flexible License Manager

flexlm          	744/udp   	Flexible License Manager

fujitsu-dev     	747/tcp    	Fujitsu Device Control

fujitsu-dev     	747/udp    	Fujitsu Device Control

ris-cm          	748/tcp    	Russell Info Sci Calendar Mgr

ris-cm          	748/udp    	Russell Info Sci Calendar Mgr

kerberos-adm		749/tcp    	kerberos administration

kerberos-adm		749/udp    	kerberos administration

rfile           	750/tcp

loadav          	750/udp

pump            	751/tcp

pump            	751/udp

qrh             	752/tcp

qrh             	752/udp

rrh             	753/tcp

rrh             	753/udp

tell            	754/tcp     	send

tell            	754/udp     	send

nlogin          	758/tcp

nlogin          	758/udp

con             	759/tcp

con             	759/udp

ns              	760/tcp

ns              	760/udp

rxe            		761/tcp

rxe             	761/udp

quotad          	762/tcp

quotad         		762/udp

cycleserv       	763/tcp

cycleserv       	763/udp

omserv          	764/tcp

omserv          	764/udp

webster         	765/tcp

webster         	765/udp

phonebook 		767/tcp    	phone

phonebook		767/udp    	phone

vid             	769/tcp

vid             	769/udp

cadlock         	770/tcp

cadlock         	770/udp

rtip 			771/tcp

rtip            	771/udp

cycleserv2      	772/tcp

cycleserv2      	772/udp

submit          	773/tcp

notify          	773/udp

rpasswd         	774/tcp

acmaint_dbd		774/udp

entomb          	775/tcp

acmaint_transd		775/udp

wpages          	776/tcp

wpages          	776/udp

wpgs            	780/tcp

wpgs            	780/udp

concert         	786/tcp       	Concert

concert         	786/udp       	Concert

mdbs_daemon		800/tcp

mdbs_daemon		800/udp

device          	801/tcp

device 			801/udp

xtreelic        	996/tcp        	Central Point Software

xtreelic        	996/udp        	Central Point Software

maitrd          	997/tcp

maitrd          	997/udp

busboy          	998/tcp

puparp          	998/udp

garcon          	999/tcp

applix          	999/udp        	Applix ac

puprouter       	999/tcp

puprouter       	999/udp

cadlock         	1000/tcp

ock             	1000/udp

1023/tcp	Reserved

1024/udp	Reserved

- AT Command Set (Partial listing)

All commands are preceded by AT. Commands can be grouped after one AT string.

| Command |  | Function |
| --- | --- | --- |
| A |  | Answer incoming call |
| B0 |  | Use V.22 1200 baud connection |
| B1 |  | Use Bell 212A 1200 baud connection |
| Dxxx-xxxx |  | Dials a phone number xxx-xxxx |
|  |  | D can be followed by: |
|  |  | ! Goes on hook for time given in S29 |
|  |  | , Pauses during dial (pause time is in S8) |
|  |  | S=#{#=0-3} Dials number stored in &Z# |
|  |  | ; (comes after dial string) Return to |
|  |  | command mode after dialing |
|  |  | @ Waits for 5 sec of silence before dialing |
| L |  | Redials last number dialed |
| P |  | Pulse dial |
| R |  | Accept command, but don't act on it [...] |
| W |  | Wait for dial tone |
| ^ |  | Turn on call tone |
| E0 |  | Echo off - no command characters |
| E1 |  | Echo command characters |
| H0 |  | Hang up |
| H1 |  | Off hook (same as hang up) |
| I0 |  | Info - returns product code |
| I1 |  | Info - Returns ROM checksum |
| I2 |  | Info - Checks ROM checksum and returns OK or ERROR |
| I3 |  | Info - Returns firmware revision code |
| I4 |  | Info - Returns modem ID string |
| I5 |  | Info - Returns country code |
| I6 |  | Info - Returns data pump info |
| L0 |  | Sets speaker volume to lowest |
| L1 |  | Sets speaker volume to low |
| L2 |  | Sets speaker volume to medium |
| L3 |  | Sets speaker volume to highest |
| M0 |  | Speaker off |
| M1 |  | Speaker is on until a carrier detect |
| M2 |  | Speaker is always on |
| M3 |  | Speaker is on during answering only |
| N0 |  | Disables automatic modulation negotation |
| O0 |  | Puts modem in data mode |
| O1 |  | Takes modem out of data mode |
| P |  | Pulse dialing |
| Q0 |  | Sets DTR line |
| Q1 |  | Clears DTR line |
| S# |  | Make S-register # default register |
| S#=x |  | Set S-register # to x |
| S#? |  | Returns value of register # |
| T |  | Tone dialing |
| V0 |  | Numeric responses |
| V1 |  | Verbose responses |
| W0 |  | Returns computer-modem speed |
| W1 |  | Returns tranfer rate and error correction protocol |
| W2 |  | Returns modem-modem speed |
| X0 |  | Verbose error codes |
| X1 |  | X0 responses and connect speed |
| X2 |  | X1 responses and NO DIALTONE |
| X3 |  | X1 responses and BUSY |
| X4 |  | All responses |
| Y0 |  | Disable disconnection on pause |
| Y1 |  | Enable disconnection on pause |
| Z0 |  | Sets modem to profile 0 (set in &W0) |
| Z1 |  | Sets modem to profile 1 (set in &W1) |

The following AT commands are not part of the original specification and are called the Extended AT command set. Each command is preceded by a "&" symbol.

| &C0 |  | Sets DCD on |
| --- | --- | --- |
| &C1 |  | Sets DCD to follow the actual carrier |
| &D0 |  | DTR active |
| &D1 |  | DTR drop as an escape sequence |
| &D2 |  | DTR drop as a hang up command |
| &D3 |  | DTR drop as a reset; loads profile from &Y |
| &F |  | Loads the factory profile (may be many) |
| &G0 |  | Disables gaurd tone |
| &G1 |  | Disables gaurd tone |
| &G2 |  | Enables 1800 Hz guard tone |
| &K0 |  | No flow control |
| &K3 |  | Hardware flow control |
| &K4 |  | Software (XON/XOFF) flow control |
| &M0 |  | No error correction, no speed control |
| &Q0 |  | Does same thing as &M0 |
| &Q5 |  | Modem negotiates error correction |
| &Q6 |  | &M0 with speed buffering |
| &S0 |  | Data Set Ready active |
| &S1 |  | DSR active after answer detected until carrier is lost |
| &T0-8 |  | {Engineering tests} |
| &V |  | Returns profiles [&Y] and stored numbers [&Z] |
| &W0 |  | Save current config as profile 0 |
| &W1 |  | Save current config as profile 1 |
| &Y0 |  | Modem uses profile 0 on startup |
| &Y1 |  | Modem uses profile 1 on startup |
| &Z#=x |  | Set stored number (0-3) to # |

- ISO 3166 Country Codes

ad 	Andorra        	fi	Finland		lb	Lebanon

ae 	United Arab	fj	Fiji		lc	Saint Lucia	sc 	Seychelles

Emirates	fk 	Falkland	li	Liechtenstein	sd 	Sudan

af 	Afghanistan	    	Islands		lk	Sri Lanka      	se 	Sweden

ag 	Antigua and		(Malvinas)	lr	Liberia        	sg 	Singapore

Barbuda        	fm	Micronesia,    	ls	Lesotho        	sh 	St. Helena

ai 	Anguilla          	Federated      	lt	Lithuania      	si 	Slovenia

al 	Albania           	States of      	lu	Luxembourg     	sj 	Svalbard and

am	Armenia        	fo 	Faroe Islands  	lv	Latvia            	Jan Mayen

an 	Netherlands    	fr 	France         	ly	Libyan Arab      	Islands

Antilles       	fx 	Metropolitan      	 Jamahiriya     sk 	Slovakia

ao	Angola   		France         	ma 	Morocco			(Slovak

aq	Antarctica     	ga	Gabon          	mc 	Monaco     		Republic)

ar 	Argentina      	gb 	United Kingdom 	md 	Moldova,       	sl 	Sierra Leone

as 	American Samoa 	gd 	Grenada           	Republic of    	sm 	San Marino

at 	Austria        	ge 	Georgia        	mg 	Madagascar     	sn 	Senegal

au 	Australia      	gf 	French Guiana  	mh 	Marshall       	so 	Somalia

aw 	Aruba          	gh 	Ghana            	Islands        	sr 	Suriname

az 	Azerbaijan     	gi 	Gibraltar      	mk 	Macedonia, The 	st 	Sao Tome and

ba 	Bosnia and     	gl 	Greenland         	Former            	Principe

Herzegowina    	gm 	Gambia            	Yugoslav       	sv 	El Salvador

bb 	Barbados       	gn 	Guinea            	Republic of    	sy 	Syrian Arab

bd 	Bangladesh     	gp 	Guadeloupe     	ml 	Mali              	Republic

be 	Belgium        	gq 	Equatorial     	mm	Myanmar        	sz 	Swaziland

bf 	Burkina Faso      	Guinea         	mn 	Mongolia       	tc 	Turks and

bg 	Bulgaria       	gr 	Greece         	mo 	Macau             	Caicos Islands

bh 	Bahrain        	gs 	South Georgia  	mp 	Northern       	td 	Chad

bi 	Burundi           	and The South     	Mariana        	tf 	French

bj 	Benin             	Sandwich          	Islands           	Southern

bm 	Bermuda           	Islands        	mq	Martinique        	Territories

bn 	Brunei         	gt 	Guatemala      	mr	Mauritania     	tg 	Togo

Darussalam     	gu 	Guam           	ms 	Montserrat     	th	Thailand

bo 	Bolivia        	gw 	Guinea-Bissau  	mt 	Malta          	tj	Tajikistan

br 	Brazil         	gy 	Guyana         	mu 	Mauritius      	tk 	Tokelau

bs 	Bahamas        	hk 	Hong Kong      	mv 	Maldives       	tm 	Turkmenistan

bt 	Bhutan         	hm 	Heard and Mc   	mw 	Malawi         	tn 	Tunisia

bv 	Bouvet Island     	Donald Islands 	mx 	Mexico         	to 	Tonga

bw 	Botswana       	hn 	Honduras       	my 	Malaysia       	tp 	East Timor

by 	Belarus        	hr 	Croatia        	mz 	Mozambique     	tr 	Turkey

bz 	Belize            	(Hrvatska)     	na 	Namibia        	tt 	Trinidad and

ca 	Canada         	ht 	Haiti          	nc 	New Caledonia     	Tobago

cc 	Cocos          	hu 	Hungary        	ne 	Niger          	tv 	Tuvalu

(Keeling)      	id 	Indonesia      	nf 	Norfolk Island 	tw 	Taiwan,

Islands        	ie 	Ireland        	ng 	Nigeria           	Province of

cf 	Central		il 	Israel         	ni 	Nicaragua         	China

African		in 	India          	nl 	Netherlands    	tz 	Tanzania,

Republic	io 	British Indian 	no 	Norway            	United

cg 	Congo             	Ocean          	np 	Nepal             	Republic of

ch 	Switzerland       	Territory      	nr 	Nauru          	ua 	Ukraine

ci 	Cote D'ivoire  	iq 	Iraq           	nu 	Niue           	ug 	Uganda

ck 	Cook Islands   	ir 	Iran (Islamic  	nz 	New Zealand    	um 	United States

cl 	Chile             	Republic of)   	om 	Oman              	Minor Outlying

cm 	Cameroon       	is 	Iceland        	pa 	Panama            	Islands

cn 	China          	it 	Italy          	pe 	Peru           	us 	United States

co 	Colombia       	jm 	Jamaica        	pf 	French         	uy 	Uruguay

cr 	Costa Rica     	jo 	Jordan            	Polynesia      	uz 	Uzbekistan

cu 	Cuba           	jp 	Japan          	pg 	Papua New      	va 	Vatican City

cv 	Cape Verde     	ke 	Kenya             	Guinea            	State (Holy

cx 	Christmas      	kg 	Kyrgyzstan     	ph 	Philippines       	See)

Island         	kh 	Cambodia       	pk 	Pakistan       	vc 	Saint Vincent

cy 	Cyprus         	ki 	Kiribati       	pl 	Poland            	and The

cz 	Czech Republic 	km 	Comoros        	pm 	St. Pierre and    	Grenadines

de 	Germany        	kn 	Saint Kitts       	Miquelon       	ve 	Venezuela

dj 	Djibouti          	and Nevis      	pn 	Pitcairn       	vg 	Virgin Islands

dk 	Denmark        	kp 	Korea,         	pr 	Puerto Rico       	(British)

dm 	Dominica          	Democratic     	pt 	Portugal       	vi 	Virgin Islands (U.S)

do 	Dominican         	People's       	pw 	Palau		vn 	Viet Nam

Republic          	Republic of    	py 	Paraguay       	vu 	Vanuatu

dz 	Algeria        	kr 	Korea,         	qa 	Qatar          	wf 	Wallis and

ec 	Ecuador           	Republic of    	re 	Reunion        		Futuna Islands

ee 	Estonia        	kw	Kuwait         	ro 	Romania		ws 	Samoa

eg 	Egypt          	ky 	Cayman Islands 	ru 	Russian        	ye 	Yemen

eh 	Western Sahara 	kz 	Kazakhstan        	Federation     	yt 	Mayotte

er 	Eritrea        	la 	Lao People's   	rw 	Rwanda         	yu 	Yugoslavia

es 	Spain             	Democratic     	sa 	Saudi Arabia   	za 	South Africa

et 	Ethiopia          	Republic       	sb 	Solomon        	zm	Zambia

- Token Ring - Major Vector IDs

| Frame Control ZZZZ bits | MVID | MAC Frame | Source | Destination | Subvectors |
| --- | --- | --- | --- | --- | --- |
| 00 | 00 | Response | Ring Station | Ring Station | 09 [O] Correlator 20 [R] Response Code |
| 00 | 08 | Lobe Test | Ring Station | Null Address | 26 [R] Wrap Data |
| 00 | 09 | Transmit Forward | CRS | Ring Station | 27 [R] Frame Forward |
| 00 | 0C | Change Parameters | CRS | Ring Station | 03 [O] Local Ring Number 04 [O] Assign Physical Location 05 [O] Soft Error Report Timer Value 06 [O] Enabled Function Classes 07 [O] Allowed Access Priority 09 [O] Correlator |
| 00 | 0D | Initialize Ring Station | RPS | Ring Station | 03 [O] Local Ring Number 04 [O] Assign Physical Location 05 [O] Soft Error Report Timer Value 09 [O] Correlator |
| 00 | 0E | Request Ring Station Address | CRS | Ring Station | 09 [O] Correlator |
| 00 | 0F | Request Ring Station State | CRS | Ring Station | 09 [O] Correlator |
| 00 | 10 | Request Ring Station Attachments | CRS | Ring Station | 09 [O] Correlator |
| 00 | 20 | Request Initialization | Ring Station | RPS | 02 [R] NAUN 22 [R] Product Instance ID 23 [R] Ring Station Microcode Level |
| 00 | 22 | Report Ring Station Address | Ring Station | CRS | 02 [R] NAUN 09 [R] Correlator 0B [R] Physical Address 2B [R] Group Address 2C [R] Functional Address |
| 00 | 23 | Report Ring Station State | Ring Station | CRS | 09 [R] Correlator 23 [R] Ring Station Microcode Level 28 [R] Station ID 29 [R] Ring Station Status Vector |
| 00 | 24 | Report Ring Station Attachments | Ring Station | CRS | 06 [R] Enabled Function Classes 07 [R] Allowed Access Priority 09 [R] Correlator 22 [R] Product Instance ID 2C [R] Functional Address |
| 00 | 25 | Report New Active Monitor | Ring Station | CRS | 02 [R] NAUN 0B [R] Physical Location 22 [R] Product Instance ID |
| 00 | 26 | Report NAUN Change | Ring Station | CRS | 02 [R] NAUN 0B [R] Physical Location |
| 00 | 27 | Report Neighbour Notification Complete | Ring Station | REM | 0A [R] Address of last Neighbour Notification |
| 00 | 28 | Report Active Monitor Error | Ring Station | REM | 02 [R] NAUN 0B [R] Physical Location 30 [R] Error Code |
| 00 | 29 | Report Soft Error | Ring Station | REM | 02 [R] NAUN 0B [R] Physical Location 2D [R] Isolating Error Counts 2E [R] Non-isolating Error Counts |
| 00 | 2A | Report Transmit Forward | Ring Station | CRS | 2A [R] Transmit Status Code |
| 01 | 07 | Duplicate Address Test | Ring Station | Own Address | None |
| 01 | 0B | Remove Ring Station | CRS | Ring Station | None |
| 02 | 02 | Beacon | Ring Station | All Stations | 01 [R] Beacon Type 02 [R] NAUN 0B [R] Phyical Location |
| 03 | 03 | Claim Token | Ring Station | All Stations | 02 [R] NAUN 0B [R] Physical Location |
| 04 | 04 | Ring Purge | Ring Station | All Stations | 02 [R] NAUN 0B [R] Physical Location |
| 05 | 05 | Active Monitor Present | Ring Station | All Stations | 02 [R] NAUN 0B [R] Physical Location |
| 06 | 06 | Standby Monitor Present | Ring Station | All Stations | 02 [R] NAUN 0B [R] Physical Location |

- The GNU General Public License

Copyright ©1989, 1991 Free Software Foundation, Inc. 675 Mass Ave, Cambridge, MA 02139, USA Everyone is permitted to copy and distribute verbatim copies of this license document, but changing it is not allowed.

The licenses for most software are designed to take away your freedom to share and change it. By contrast, the GNU General Public License is intended to guarantee your freedom to share and change free software-to make sure the software is free for all its users. This General Public License applies to most of the Free Software Foundation's software and to any other program whose authors commit to using it. (Some other Free Software Foundation software is covered by the GNU Library General Public License instead.) You can apply it to your programs, too.

When we speak of free software, we are referring to freedom, not price. Our General Public Licenses are designed to make sure that you have the freedom to distribute copies of free software (and charge for this service if you wish), that you receive source code or can get it if you want it, that you can change the software or use pieces of it in new free programs; and that you know you can do these things.

To protect your rights, we need to make restrictions that forbid anyone to deny you these rights or to ask you to surrender the rights. These restrictions translate to certain responsibilities for you if you distribute copies of the software, or if you modify it.

For example, if you distribute copies of such a program, whether gratis or for a fee, you must give the recipients all the rights that you have. You must make sure that they, too, receive or can get the source code. And you must show them these terms so they know their rights.

We protect your rights with two steps: (1) copyright the software, and (2) offer you this license which gives you legal permission to copy, distribute and/or modify the software.

Also, for each author's protection and ours, we want to make certain that everyone understands that there is no warranty for this free software. If the software is modified by someone else and passed on, we want its recipients to know that what they have is not the original, so that any problems introduced by others will not reflect on the original authors' reputations.

Finally, any free program is threatened constantly by software patents. We wish to avoid the danger that redistributors of a free program will individually obtain patent licenses, in effect making the program proprietary. To prevent this, we have made it clear that any patent must be licensed for everyone's free use or not licensed at all.

The precise terms and conditions for copying, distribution and modification follow.

0.

This License applies to any program or other work which contains a notice placed by the copyright holder saying it may be distributed under the terms of this General Public License. The ``Program'', below, refers to any such program or work, and a ``work based on the Program'' means either the Program or any derivative work under copyright law: that is to say, a work containing the Program or a portion of it, either verbatim or with modifications and/or translated into another language. (Hereinafter, translation is included without limitation in the term ``modification''.) Each licensee is addressed as ``you''.

Activities other than copying, distribution and modification are not covered by this License; they are outside its scope. The act of running the Program is not restricted, and the output from the Program is covered only if its contents constitute a work based on the Program (independent of having been made by running the Program). Whether that is true depends on what the Program does.

1.

You may copy and distribute verbatim copies of the Program's source code as you receive it, in any medium, provided that you conspicuously and appropriately publish on each copy an appropriate copyright notice and disclaimer of warranty; keep intact all the notices that refer to this License and to the absence of any warranty; and give any other recipients of the Program a copy of this License along with the Program.

You may charge a fee for the physical act of transferring a copy, and you may at your option offer warranty protection in exchange for a fee.

2.

You may modify your copy or copies of the Program or any portion of it, thus forming a work based on the Program, and copy and distribute such modifications or work under the terms of Section 1 above, provided that you also meet all of these conditions:

a.

You must cause the modified files to carry prominent notices stating that you changed the files and the date of any change.

b.

You must cause any work that you distribute or publish, that in whole or in part contains or is derived from the Program or any part thereof, to be licensed as a whole at no charge to all third parties under the terms of this License.

c.

If the modified program normally reads commands interactively when run, you must cause it, when started running for such interactive use in the most ordinary way, to print or display an announcement including an appropriate copyright notice and a notice that there is no warranty (or else, saying that you provide a warranty) and that users may redistribute the program under these conditions, and telling the user how to view a copy of this License. (Exception: if the Program itself is interactive but does not normally print such an announcement, your work based on the Program is not required to print an announcement.)

These requirements apply to the modified work as a whole. If identifiable sections of that work are not derived from the Program, and can be reasonably considered independent and separate works in themselves, then this License, and its terms, do not apply to those sections when you distribute them as separate works. But when you distribute the same sections as part of a whole which is a work based on the Program, the distribution of the whole must be on the terms of this License, whose permissions for other licensees extend to the entire whole, and thus to each and every part regardless of who wrote it.

Thus, it is not the intent of this section to claim rights or contest your rights to work written entirely by you; rather, the intent is to exercise the right to control the distribution of derivative or collective works based on the Program.

In addition, mere aggregation of another work not based on the Program with the Program (or with a work based on the Program) on a volume of a storage or distribution medium does not bring the other work under the scope of this License.

3.

You may copy and distribute the Program (or a work based on it, under Section 2) in object code or executable form under the terms of Sections 1 and 2 above provided that you also do one of the following:

a.

Accompany it with the complete corresponding machine-readable source code, which must be distributed under the terms of Sections 1 and 2 above on a medium customarily used for software interchange; or,

b.

Accompany it with a written offer, valid for at least three years, to give any third party, for a charge no more than your cost of physically performing source distribution, a complete machine-readable copy of the corresponding source code, to be distributed under the terms of Sections 1 and 2 above on a medium customarily used for software interchange; or,

c.

Accompany it with the information you received as to the offer to distribute corresponding source code. (This alternative is allowed only for noncommercial distribution and only if you received the program in object code or executable form with such an offer, in accord with Subsection b above.)

The source code for a work means the preferred form of the work for making modifications to it. For an executable work, complete source code means all the source code for all modules it contains, plus any associated interface definition files, plus the scripts used to control compilation and installation of the executable. However, as a special exception, the source code distributed need not include anything that is normally distributed (in either source or binary form) with the major components (compiler, kernel, and so on) of the operating system on which the executable runs, unless that component itself accompanies the executable.

If distribution of executable or object code is made by offering access to copy from a designated place, then offering equivalent access to copy the source code from the same place counts as distribution of the source code, even though third parties are not compelled to copy the source along with the object code.

4.

You may not copy, modify, sublicense, or distribute the Program except as expressly provided under this License. Any attempt otherwise to copy, modify, sublicense or distribute the Program is void, and will automatically terminate your rights under this License. However, parties who have received copies, or rights, from you under this License will not have their licenses terminated so long as such parties remain in full compliance.

5.

You are not required to accept this License, since you have not signed it. However, nothing else grants you permission to modify or distribute the Program or its derivative works. These actions are prohibited by law if you do not accept this License. Therefore, by modifying or distributing the Program (or any work based on the Program), you indicate your acceptance of this License to do so, and all its terms and conditions for copying, distributing or modifying the Program or works based on it.

6.

Each time you redistribute the Program (or any work based on the Program), the recipient automatically receives a license from the original licensor to copy, distribute or modify the Program subject to these terms and conditions. You may not impose any further restrictions on the recipients' exercise of the rights granted herein. You are not responsible for enforcing compliance by third parties to this License.

7.

If, as a consequence of a court judgment or allegation of patent infringement or for any other reason (not limited to patent issues), conditions are imposed on you (whether by court order, agreement or otherwise) that contradict the conditions of this License, they do not excuse you from the conditions of this License. If you cannot distribute so as to satisfy simultaneously your obligations under this License and any other pertinent obligations, then as a consequence you may not distribute the Program at all. For example, if a patent license would not permit royalty-free redistribution of the Program by all those who receive copies directly or indirectly through you, then the only way you could satisfy both it and this License would be to refrain entirely from distribution of the Program.

If any portion of this section is held invalid or unenforceable under any particular circumstance, the balance of the section is intended to apply and the section as a whole is intended to apply in other circumstances.

It is not the purpose of this section to induce you to infringe any patents or other property right claims or to contest validity of any such claims; this section has the sole purpose of protecting the integrity of the free software distribution system, which is implemented by public license practices. Many people have made generous contributions to the wide range of software distributed through that system in reliance on consistent application of that system; it is up to the author/donor to decide if he or she is willing to distribute software through any other system and a licensee cannot impose that choice.

This section is intended to make thoroughly clear what is believed to be a consequence of the rest of this License.

8.

If the distribution and/or use of the Program is restricted in certain countries either by patents or by copyrighted interfaces, the original copyright holder who places the Program under this License may add an explicit geographical distribution limitation excluding those countries, so that distribution is permitted only in or among countries not thus excluded. In such case, this License incorporates the limitation as if written in the body of this License.

9.

The Free Software Foundation may publish revised and/or new versions of the General Public License from time to time. Such new versions will be similar in spirit to the present version, but may differ in detail to address new problems or concerns.

Each version is given a distinguishing version number. If the Program specifies a version number of this License which applies to it and ``any later version'', you have the option of following the terms and conditions either of that version or of any later version published by the Free Software Foundation. If the Program does not specify a version number of this License, you may choose any version ever published by the Free Software Foundation.

10.

If you wish to incorporate parts of the Program into other free programs whose distribution conditions are different, write to the author to ask for permission. For software which is copyrighted by the Free Software Foundation, write to the Free Software Foundation; we sometimes make exceptions for this. Our decision will be guided by the two goals of preserving the free status of all derivatives of our free software and of promoting the sharing and reuse of software generally.

11.

BECAUSE THE PROGRAM IS LICENSED FREE OF CHARGE, THERE IS NO WARRANTY FOR THE PROGRAM, TO THE EXTENT PERMITTED BY APPLICABLE LAW. EXCEPT WHEN OTHERWISE STATED IN WRITING THE COPYRIGHT HOLDERS AND/OR OTHER PARTIES PROVIDE THE PROGRAM ``AS IS'' WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH YOU. SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

12.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW OR AGREED TO IN WRITING WILL ANY COPYRIGHT HOLDER, OR ANY OTHER PARTY WHO MAY MODIFY AND/OR REDISTRIBUTE THE PROGRAM AS PERMITTED ABOVE, BE LIABLE TO YOU FOR DAMAGES, INCLUDING ANY GENERAL, SPECIAL, INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE PROGRAM (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR DATA BEING RENDERED INACCURATE OR LOSSES SUSTAINED BY YOU OR THIRD PARTIES OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER PROGRAMS), EVEN IF SUCH HOLDER OR OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

If you develop a new program, and you want it to be of the greatest possible use to the public, the best way to achieve this is to make it free software which everyone can redistribute and change under these terms.

To do so, attach the following notices to the program. It is safest to attach them to the start of each source file to most effectively convey the exclusion of warranty; and each file should have at least the ``copyright'' line and a pointer to where the full notice is found.

one line to give the program's name and a brief idea of what it does. Copyright ©19yy name of author

This program is free software; you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation; either version 2 of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

You should have received a copy of the GNU General Public License along with this program; if not, write to the Free Software Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

Also add information on how to contact you by electronic and paper mail.

If the program is interactive, make it output a short notice like this when it starts in an interactive mode:

The hypothetical commands `show w' and `show c' should show the appropriate parts of the General Public License. Of course, the commands you use may be called something other than `show w' and `show c'; they could even be mouse-clicks or menu items-whatever suits your program.

You should also get your employer (if you work as a programmer) or your school, if any, to sign a ``copyright disclaimer'' for the program, if necessary. Here is a sample; alter the names:

Yoyodyne, Inc., hereby disclaims all copyright interest in the program `Gnomovision' (which makes passes at compilers) written by James Hacker.

signature of Ty Coon, 1 April 1989 
Ty Coon, President of Vice

This General Public License does not permit incorporating your program into proprietary programs. If your program is a subroutine library, you may consider it more useful to permit linking proprietary applications with the library. If this is what you want to do, use the GNU Library General Public License instead of this License.

- Eugene Blanchard

- Name: Eugene Blanchard

- School: Southern Alberta Institute of Technology

- Location: Calgary, AB

- Department: Information Technology